3GPP TSG-RAN WG2 Meeting #127	R2-2406507

Maastricht, Netherlands, Aug 18th – 23rd, 2024

Agenda item:		7.11.2

Source:				Samsung

Title:					Corrections for Multicast Reception

Document for:		Discussion & Decision

# 1 Introduction

This paper discusses certain corrections for multicast reception in RRC_INACTIVE.

# 2 Discussion

### 2.1 Error data handling

In R2#126, we agreed to handle an exception case for MBS multicast wherein a MAC subPDU belonging to a suspended multicast MRB can be received by the UE receiving multicast in RRC_INACTIVE and it needs to be discarded.

[Excerpt from R2-2405582] For a UE configured with G-RNTI 1, if multicast session 1 is configured to be received in RRC_INACTIVE and multicast session 2 is NOT configured to be received in RRC_INACTIVE, it will suspends multicast MRB 2 in RRC_INACTIVE. Obviously, when the UE is in RRC_INACTIVE, it may receive a MAC PDU for MAC entity's G-RNTI 1 containing an LCID 2 associated with the suspended multicast MRB 2. If UE accepts the subPDU contains LCID 2 associated with the suspended multicast MRB 2, this subPDU will result in error.

Spec Excerpts:

### 5.3.3 Disassembly and demultiplexing The MAC entity shall disassemble and demultiplex a MAC PDU as defined in clauses 6.1.2 and 6.1.5a. When a MAC entity receives a MAC PDU for MAC entity's G-RNTI or G-CS-RNTI, or by the configured downlink assignment for MBS multicast containing an LCID or eLCID which is not configured, the MAC entity shall at least: 1> discard the received subPDU.

## 5.13 Handling of unknown, unforeseen and erroneous protocol data When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI or by the configured downlink assignment, containing a Reserved LCID or eLCID value, or an LCID or eLCID value the MAC Entity does not support, the MAC entity shall at least: 1> discard the received subPDU and any remaining subPDUs in the MAC PDU. When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI, CS-RNTI or G-RNTI , or by the configured downlink assignment, containing an LCID or eLCID value which is not configured, or an LCID or eLCID value associated with a suspended RB as specified in TS 38.331 [5], the MAC entity shall at least: 1> discard the received subPDU.

However, the incorporated spec change has two problems as depicted in the shown spec excerpts:

- The blue highlighted part is relevant but it should be limited to G-RNTI for "MBS multicast in RRC_INACTIVE".

- The green highligted part is redundant/duplicated as it is already addressed in clause 5.3.3. It pertains to a valid case of demultiplexing.

Two text proposals are provided. TP 1A is a simple clarification and addresses a) only. Whereas, TP 1B addresses both a) and b), but it brings more spec change.

Proposal 1: RAN2 to discuss the issues for error data handling and decide to adopt either TP 1A or TP 1B.

- Error data handling should be limited to G-RNTI for "MBS multicast in RRC_INACTIVE" (TP 1A, TP 1B)

- Avoid redundancy between demultiplexing and error data handling procedures (TP 1B)

### 2.2 Nomenclature issues

2.2.1 Non-ServingCellMII

#### 5.3.5.3 Reception of an RRCReconfiguration by the UE The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA, CPC, or subsequent CPAC), or upon execution of an LTM cell switch: < Omitted Text > 2> if reconfigurationWithSync was included in masterCellGroup and the target cell provides SIB21 or provides SIB1 including non-ServingCellMII: 3> if the UE initiated transmission of an MBSInterestIndication message during the last 1 second preceding reception of this RRCReconfiguration message; or 3> if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, and the UE has initiated transmission of an MBSInterestIndication message after having received this RRCReconfiguration message: 4> initiate transmission of an MBSInterestIndication message in accordance with clause 5.9.4;

The term non-ServingCellMII used in clause 5.3.5.3 is incorrect and should be replaced by “nonServingCellMII”.

Proposal 2: Replace the term non-ServingCellMII used in clause 5.3.5.3 by “nonServingCellMII”. Adopt TP2.

2.2.2 MCCH vs Multicast MCCH

– PDSCH-ConfigBroadcast The IE PDSCH-ConfigBroadcast is used to configure parameters for acquiring the PDSCH for MCCH and MTCH.

PDSCH-ConfigBroadcast is also used to configure MBS multicast in RRC_INACTIVE. Note that MTCH is a generic term and applicable for both broadcast and multicast. However, the term MCCH is no longer generic to cover multicast MCCH. Therefore, we need to add "multicast MCCH" specifically.

Proposal 3: Add in the definition of PDSCH-ConfigBroadcast an additional term “multicast MCCH”. Adopt TP3.

# 3 Conclusion

Request RAN2 to discuss and agree to the following proposals:

Proposal 1: RAN2 to discuss the issues for error data handling and decide to adopt either TP 1A or TP 1B.

- Error data handling should be limited to G-RNTI for "MBS multicast in RRC_INACTIVE" (TP 1A, TP 1B)

- Avoid redundancy between demultiplexing and error data handling procedures (TP 1B)

Proposal 2: Replace the term non-ServingCellMII used in clause 5.3.5.3 by “nonServingCellMII”. Adopt TP2.

Proposal 3: Add in the definition of PDSCH-ConfigBroadcast an additional term “multicast MCCH”. Adopt TP3.

# 4 Text Proposals

TP 1A

## 5.13 Handling of unknown, unforeseen and erroneous protocol data When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI or by the configured downlink assignment, containing a Reserved LCID or eLCID value, or an LCID or eLCID value the MAC Entity does not support, the MAC entity shall at least: 1> discard the received subPDU and any remaining subPDUs in the MAC PDU. When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI, CS-RNTI or G-RNTI (for MBS Multicast in RRC_INACTIVE), or by the configured downlink assignment, containing an LCID or eLCID value which is not configured, or an LCID or eLCID value associated with a suspended RB as specified in TS 38.331 [5], the MAC entity shall at least: 1> discard the received subPDU.

TP 1B

## 5.13 Handling of unknown, unforeseen and erroneous protocol data When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI or by the configured downlink assignment, containing a Reserved LCID or eLCID value, or an LCID or eLCID value the MAC Entity does not support, the MAC entity shall at least: 1> discard the received subPDU and any remaining subPDUs in the MAC PDU. When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing an LCID or eLCID value which is not configured, or an LCID or eLCID value associated with a suspended RB or a MAC PDU for the MAC entity’s G-RNTI (for MBS Multicast in RRC_INACTIVE) containing an LCID or eLCID value associated with a suspended MRB as specified in TS 38.331 [5], the MAC entity shall at least: 1> discard the received subPDU.

TP 2

#### 5.3.5.3 Reception of an RRCReconfiguration by the UE The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA, CPC, or subsequent CPAC), or upon execution of an LTM cell switch: < Omitted Text > 2> if reconfigurationWithSync was included in masterCellGroup and the target cell provides SIB21 or provides SIB1 including nonServingCellMII: 3> if the UE initiated transmission of an MBSInterestIndication message during the last 1 second preceding reception of this RRCReconfiguration message; or 3> if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, and the UE has initiated transmission of an MBSInterestIndication message after having received this RRCReconfiguration message: 4> initiate transmission of an MBSInterestIndication message in accordance with clause 5.9.4;

TP 3

– PDSCH-ConfigBroadcast The IE PDSCH-ConfigBroadcast is used to configure parameters for acquiring the PDSCH for MCCH, multicast MCCH and MTCH.