3GPP TSG-RAN WG2 Meeting #127	R2-2406511

Maastricht, Netherlands, August 19-23, 2024

Source: 			Qualcomm Incorporated

Title: 	Discussion on cell barring and barring exemption

Document for: 	Decision

Agenda Item: 	7.24.2.2

# Introduction

Through multiple releases, cell barring mechanisms have been specified for different features. While many of them share similar behaviours, the way the corresponding behaviours are specified is not very consistent in the current specifications.

In this document, we review how the cell barring behaviour is specified for different features, and propose a specification clean-up.

# Discussion

  - Barring exemption

In RAN2#126 meeting, the barring exemption mechanism was introduced for RedCap 1Rx/2Rx, eRedCap 1Rx/2Rx and 2Rx XR UEs. In the email discussion thread on [AT126][002][XR] “Emergency call support for Redcap, eRedcap and 2Rx XR”, the following clarification was made.

- FFS check in RAN2-127 if further clarifications are required to ensure that once the cell is considered acceptable according to new conditions, the UE doesn’t follow the procedures in 38.304 associated with cell barred case.

In our view, the CRs to 38.304 [1][2] introduced the following issues.

Issue 1:	A cell can be considered as an acceptable cell and a barred cell at the same time.

acceptable cell: An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls and receive ETWS and CMAS notifications). Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS and CMAS notification in an NR network: - The cell is not barred, see clause 5.3.1 for details and exceptions; - The cell selection criteria are fulfilled, see clause 5.2.3.2.

Issue 2:	Cell reservations are ignored after barring exemption.

Due to the placement of the new text, the UE can consider the cell as not barred even if the cell is under “cell reservation”.

Issue 3:	After the barring exemption, the UE still is required to select another cell.

### 5.3.1 Cell status and cell reservations […] When cell status "barred" is indicated or to be treated as if the cell status is "barred", - The UE is not permitted to select/reselect this cell, not even for emergency calls except for the below cases: - When cellBarredRedCap1Rx is set to "barred" in SIB1, a RedCap UE that supports only 1Rx branch can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to "barred" and in SIB1, barringExemptEmergencyCall is present and, if the RedCap UE supports only half duplex FDD operation, halfDuplexRedCapAllowed is set to "true"; or - When cellBarredRedCap2Rx is set to "barred" in SIB1, a RedCap UE that supports 2Rx branches can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to "barred" and in SIB1, barringExemptEmergencyCall is present and, if the RedCap UE supports only half duplex FDD operation, halfDuplexRedCapAllowed is set to "true"; or - When cellBarred-eRedCap1Rx is set to "barred" in SIB1, an eRedCap UE that supports only 1Rx branch can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to "barred" and in SIB1, barringExemptEmergencyCall is present and, if the eRedCap UE supports only half duplex FDD operation, halfDuplexRedCapAllowed is set to "true"; or - When cellBarred-eRedCap2Rx is set to "barred" in SIB1, an eRedCap UE that supports 2Rx branches can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to "barred" and in SIB1, barringExemptEmergencyCall is present and, if the eRedCap UE supports only half duplex FDD operation, halfDuplexRedCapAllowed is set to "true"; or - When cellBarred2RxXR is present in SIB1, a 2Rx XR UE can consider the cell as acceptable cell, only if the cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to "barred" and in SIB1, barringExemptEmergencyCall is present; - The UE shall select another cell according to the following rule: […]

We propose the following corrections in 38.304.

Proposal 1:	Clarify that an acceptable cell is not a barred cell without any exception.

Proposal 2:	Clarify that after the barring exemption, the UE is still required to check the cell reservations.

Proposal 3:	Clarify that after the barring exemption, the UE is no longer required to select another cell on the same frequency or other frequency.

  - Ignoring cellBarred in MIB

The field cellBarred in MIB is ignored by NTN UE, ATG UE, IAB-MT and NCR-MT. The behaviour is specified differently as shown in the table below.

| UE, MT type | 38.331 | 38.304 |
| --- | --- | --- |
| NTN UE | Section 5.2.2.4.1 Field description of MIB | Not specified |
| ATG UE |  |  |
| IAB-MT | Field description of MIB | NOTE in section 5.3.1 |
| NCR-MT |  |  |

  - Determination of cell barring

This section summarizes how the determination of cell barring status is defined in the current specifications. It should be noted that the specifications define how the UE determines that the cell is considered as “barred”, which only implicitly define when the cell is considered as “not barred”.

It can be seen that the descriptions in 38.304 are either duplication of 38.331 or incomplete.

NTN UE

| cellBarredNTN (SIB1) | cellBarredFixedVSAT / cellBarredMobileVSAT (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- | --- |
| Not present | Don’t care | Barred | Section 5.2.2.4.2 | Section 5.3.1 |
| Set to “barred” | Don’t care |  |  | Not specified |
| Set to “not barred” | Not present |  |  |  |
|  | Set to “barred” |  |  |  |
|  | Set to “not barred” | Not barred | Not specified | Not specified |

Note that cellBarred in MIB is ignored by the UE of this type.

ATG UE

| cellBarredATG (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- |
| Not present | Barred | Section 5.2.2.4.2 | Section 5.3.1 |
| Set to “barred” |  |  | Not specified |
| Set to “not barred” | Not barred | Not specified | Not specified |

Note that cellBarred in MIB is ignored by the UE of this type.

(e)RedCap UE

| cellBarred (MIB) | intraFreqReselectionRedCap (SIB1) | cellBarred(e)RedCap1Rx / cellBarred(e)RedCap2Rx (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- | --- | --- |
| Barred | Don’t care | Don’t care | Barred | Section 5.2.2.4.1 Section 5.2.2.4.2 | Not specified |
| Not barred | Not present | Don’t care |  |  |  |
|  | Present | Set to “barred” |  |  |  |
|  |  | Not present | Not barred | Not specified |  |
|  |  | Set to “not barred” |  |  |  |

(e)RedCap half duplex UE

| cellBarred (MIB) | intraFreqReselectionRedCap (SIB1) | halfDuplexRedCapAllowed (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- | --- | --- |
| Barred | Don’t care | Don’t care | Barred | Section 5.2.2.4.1 Section 5.2.2.4.2 | Not specified |
| Not barred | Not present | Don’t care |  |  |  |
|  | Present | Set to “barred” |  |  |  |
|  |  | Not present |  |  | Section 5.3.1 |
|  |  | Set to “not barred” | Not barred | Not specified | Not specified |

2RX XR UE

| cellBarred (MIB) | cellBarred2RxXR (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- | --- |
| Barred | Don’t care | Barred | Section 5.2.2.4.1 Section 5.2.2.4.2 | Not specified |
| Not barred | Set to “barred” |  |  | Section 5.3.1 |
|  | Not present | Not barred | Not specified | Not specified |

Not that the field cellBarred2RxXR has the single value “barred”.

NES UE

| cellBarred (MIB) | cellBarredNES (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- | --- |
| Barred | Not present | Barred | Section 5.2.2.4.1 Section 5.2.2.4.2 | Section 5.3.1 |
|  | Set to “not barred” | Not barred | Not specified | Not specified |
| Not barred | Don’t care |  |  |  |

Not that the field cellBarredNES has the single value “not barred”.

IAB-MT

| iab-Support / mobileIAB-Support (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- |
| Not present | Barred | Section 5.2.2.4.2 | Not specified |
| Present | Not barred | Not specified |  |

Note that cellBarred in MIB is ignored by the MT of this type.

NCR-MT

| ncr-Support (SIB1) | Cell barred? | 38.331 | 38.304 |
| --- | --- | --- | --- |
| Not present | Barred | Section 5.2.2.4.2 | Not specified |
| Present | Not barred | Not specified |  |

Note that cellBarred in MIB is ignored by the MT of this type.

  - Cell reselection on the same frequency when the cell is barred

When the cell is considered as “barred”, the UE determines if reselection to another cell on the same frequency is allowed or not. The table below summarizes how the UE behaviour is defined.

It can be seen that some descriptions in 38.331 and 38.304 are inconsistent (while the final behaviour is the same). Also the descriptions in 38.331 are either duplication of 38.304 or even incomplete.

| UE type | 38.331 | 38.304 |
| --- | --- | --- |
| NTN | Section 5.2.2.4.2 - Behave according to 38.304. Section 5.2.2.2 - May select another cell on the same frequency if unable to acquire MIB/SIB1. | Section 5.3.1 - Follow intraFreqReselection in MIB if acquired. - May select another cell on the same frequency if unable to acquire MIB/SIB1. |
| ATG | Section 5.2.2.4.2 - Behave according to 38.304. Section 5.2.2.2 - May select another cell on the same frequency if unable to acquire MIB/SIB1. | Section 5.3.1 - Follow intraFreqReselection in MIB if acquired. - May select another cell on the same frequency if unable to acquire MIB/SIB1. |
| (e)RedCap | Section 5.2.2.4.2 - Follow intraFreqReselection(-e)RedCap if present. - Consider as if intraFreqReselection(-e)RedCap = allowed if not present. Section 5.2.2.2 - Consider as if intraFreqReselection(-e)RedCap = allowed if unable to acquire MIB/SIB1. | Section 5.3.1 - Follow intraFreqReselection(-e)RedCap if present. - Consider as if intraFreqReselection(-e)RedCap = allowed if the cell does not support (e)RedCap. - May select another cell on the same frequency if unable to acquire MIB/SIB1. |
| 2Rx XR | Section 5.2.2.4.2 - Follow intraFreqReselection2RxXR if present. - Consider as if intraFreqReselection2RxXR = allowed if not present. Section 5.2.2.2 - Consider as if intraFreqReselection2RxXR = allowed if unable to acquire MIB/SIB1. | Section 5.3.1 - Follow intraFreqReselection2RxXR if present. - May select another cell on the same frequency if unable to acquire MIB/SIB1. |
| NES | Section 5.2.2.4.2 - Behave according to 38.304. Section 5.2.2.2 - May select another cell on the same frequency if unable to acquire MIB/SIB1. | Section 5.3.1 - Follow intraFreqReselection in MIB if acquired. - May select another cell on the same frequency if unable to acquire MIB/SIB1. |
| IAB-MT | Section 5.2.2.4.2 - Not specified. Section 5.2.2.2 - May select another cell on the same frequency if unable to acquire MIB/SIB1. | Section 5.3.1 - NOTE: Ignore intraFreqReselection (i.e. treat intraFreqReselection as if it was set to allowed) - May select another cell on the same frequency if unable to acquire MIB/SIB1. |
| NCR-MT | Section 5.2.2.4.2 - Not specified Section 5.2.2.2 - May select another cell on the same frequency if unable to acquire MIB/SIB1. | Section 5.3.1 - NOTE: Ignore intraFreqReselection (i.e. treat intraFreqReselection as if it was set to allowed) - May select another cell on the same frequency if unable to acquire MIB/SIB1. |

  - Proposed principles for clean-up

As shown in the previous sections, the current specifications, 38.331 and 38.304, are not very consistent in describing the bahaviours related to cell barring. Such inconsistency is prone to misinterpretation and errors in future specification maintenance. It was also found that the description of some bahaviours are duplicated in 38.331 and 38.304, even though consistent.

Proposal 4:	Agree on the following principles with regards to the “work split” between 38.331 and 38.304.

| 38.331 | 38.304 |
| --- | --- |
| - Specify the determination of cell barring status. | - Specify the behaviour associated with cell reservations (which can result in cell barring) - Specify the barring exemption. - Specify whether cell reselection to another cell on the same frequency as barred cell is allowed or not. |

# Proposal

Based on the discussion in this document, we made the following proposals.

On cell barring exemption

Proposal 1:	Clarify that an acceptable cell is not a barred cell without any exception.

Proposal 2:	Clarify that after the barring exemption, the UE is still required to check the cell reservations.

Proposal 3:	Clarify that after the barring exemption, the UE is no longer required to select another cell on the same frequency or other frequency.

On inconsistency between 38.331 and 38.304

Proposal 4:	Agree on the following principles with regards to the “work split” between 38.331 and 38.304.

| 38.331 | 38.304 |
| --- | --- |
| - Specify the determination of cell barring status. | - Specify the behaviour associated with cell reservations (which can result in cell barring) - Specify the barring exemption. - Specify whether cell reselection to another cell on the same frequency as barred cell is allowed or not. |

CRs to 38.331 and 38.304 are provided in [4] and [5].

# Reference

[1]	R2-2405956	Rel-18 CR to 38.304 on Introduction of barring exemption for RedCap UEs for emergency calls [RedCap_EM_Call] (Early implementable)

[2]	R2-2405958	Rel-18 CR to 38.304 on Introduction of barring exemption for (e)RedCap and 2RX XR UEs for emergency calls [EM_Call_Exemption]

[3]	R2-2406512	Rel-18 CR to 38.331 on Clarifications on cell barring [EM_Call_Exemption]	Qualcomm Incorporated

[4]	R2-2406513	Rel-18 CR to 38.304 on Clarifications on cell barring [EM_Call_Exemption]	Qualcomm Incorporated