3GPP TSG-RAN WG2 Meeting #127	R2-2406518

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:		7.15.2

Source:				ASUSTeK

Title:		Discussion on SR prioritization regarding SL transmission

Document for: 	Discussion and Decision

- Introduction

In the current MAC specification, for each triggered SR, the UE determines whether to prioritize a SR over overlapping transmissions, including UL-SCH resource, SL-SCH resources and SL-PRS resources:

As long as at least one SR is pending, the MAC entity shall for each pending SR: 1> if the MAC entity has no valid PUCCH resource configured for the pending SR; and 1> if there is no ongoing RACH-less LTM cell switch; and 1> if rach-LessHO is not configured: 2> initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR. 1> else, for the SR configuration corresponding to the pending SR: 2> when the MAC entity has an SR transmission occasion on the valid PUCCH resource for SR configured; and 2> if sr-ProhibitTimer is not running at the time of the SR transmission occasion; and 2> if the PUCCH resource for the SR transmission occasion does not overlap with a measurement gap: 3> if the PUCCH resource for the SR transmission occasion overlaps with neither a UL-SCH resource whose simultaneous transmission with the SR is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup nor an SL-SCH resource; or 3> if the MAC entity is able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource; or 3> if the MAC entity is configured with lch-basedPrioritization, and the PUCCH resource for the SR transmission occasion does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or with the PUSCH duration of a MSGA payload, and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5 overlaps with any other UL-SCH resource(s), and the physical layer can signal the SR on one valid PUCCH resource for SR, and the priority of the logical channel that triggered SR is higher than the priority of the uplink grant(s) for any UL-SCH resource(s) where the uplink grant was not already de-prioritized and its simultaneous transmission with the SR is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCHgroups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the uplink grant is determined as specified in clause 5.4.1; or 3> if both sl-PrioritizationThres and ul-PrioritizationThres are configured and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5 overlaps with any UL-SCH resource(s) carrying a MAC PDU, and the value of the priority of the triggered SR determined as specified in clause 5.22.1.5 is lower than sl-PrioritizationThres and the value of the highest priority of the logical channel(s) in the MAC PDU is higher than or equal to ul-PrioritizationThres and any MAC CE prioritized as described in clause 5.4.3.1.3 is not included in the MAC PDU and the MAC PDU is not prioritized by upper layer according to TS 23.287 [19]; or 3> if an SL-SCH resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource, and either transmission on the SL-SCH resource is not prioritized as described in clause 5.22.1.3.1a or the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres, if configured; or 3> if an SL-SCH resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource, and the priority of the triggered SR determined as specified in clause 5.22.1.5 is higher than the priority of the MAC PDU determined as specified in clause 5.22.1.3.1a for the SL-SCH resource; or 3> if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and either transmission on the SL-PRS resource is not prioritized as described in clause 5.22.1.3.1a or the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres, if configured; or 3> if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and the priority of the triggered SR determined as specified in clause 5.22.1.5 is higher than the priority of the MAC PDU and SL-PRS, if available, determined as specified in clause 5.22.1.3.1a for the SL-PRS resource: 4> consider the SR transmission as a prioritized SR transmission.

In this contribution, we discuss some issues in several scenarios when determining prioritized SR transmission in the current specification and how to correct the UE behavior.

- Discussion

Missing condition for prioritization between overlapping SR and SL-PRS.

In Rel-18, SL-PRS is introduced. The UE could perform SL-PRS transmission via SL-PRS resources. Currently, the UE determines prioritization between SR and SL-PRS using corresponding priorities when the MAC entity cannot perform the SR and SL-PRS simultaneously:

… 3> if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and either transmission on the SL-PRS resource is not prioritized as described in clause 5.22.1.3.1a or 5.22.1.3.1b or the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres, if configured; or 3> if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and the priority of the triggered SR determined as specified in clause 5.22.1.5 is higher than the priority of the MAC PDU and SL-PRS, if available, determined as specified in clause 5.22.1.3.1a for the SL-PRS resource: 4> consider the SR transmission as a prioritized SR transmission.

However, the case where the MAC entity can perform the SR transmission simultaneously with the SL-PRS is not covered in current condition check, and the UE cannot consider the SR as prioritized SR transmission in such scenario.

In contrast, the scenario where the UE is able to perform SR transmission simultaneously with the transmission of the SL-SCH resource is currently captured:

3> if the PUCCH resource for the SR transmission occasion overlaps with neither a UL-SCH resource whose simultaneous transmission with the SR is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups nor an SL-SCH resource; or 3> if the MAC entity is able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource; or … 4> consider the SR transmission as a prioritized SR transmission. 4> consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s), except for the overlapping uplink grant(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups;

It would be beneficial to update the condition check and complement the part for SL-PRS as well.

Observation 1: In the current specification, the UE does not consider SR transmission to be prioritized when the UE has an overlapping SL-PRS transmission that is able to be performed simultaneously with the SR transmission.

Incorrect SR prioritization when overlapping with both UL-SCH and SL transmissions.

In the current specification, the UE performs each condition check separately on whether the SR transmission can be prioritized over different overlapping transmissions. That is, the UE performs checks on whether the SR can be prioritized over UL-SCH resource, SL-SCH resource, and SL-PRS individually and the conditions are concatenated with “or”. An issue can occur when the SR transmission overlaps with both UL and SL transmission. The UE will wrongly consider the SR to be prioritized over all other transmissions when any one of the condition check is fulfilled even if the other condition checks have failed.

For an example, a UE has a triggered SR of which SR transmission occasion overlaps with a SL-SCH resource and a UL-SCH resource. The uplink grant associated with the UL-SCH resource has a higher priority than the priority of the logical channel that triggered the SR, and the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres. In this scenario, the UL-SCH resource should have been prioritized; however, according to the current condition check structure, the SR will be prioritized and the uplink grant will be de-priortized:

As long as at least one SR is pending, the MAC entity shall for each pending SR: … This condition is not fulfilled 3> if the MAC entity is configured with lch-basedPrioritization, and the PUCCH resource for the SR transmission occasion does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or with the PUSCH duration of a MSGA payload, and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5 overlaps with any other UL-SCH resource(s), and the physical layer can signal the SR on one valid PUCCH resource for SR, and the priority of the logical channel that triggered SR is higher than the priority of the uplink grant(s) for any UL-SCH resource(s) where the uplink grant was not already de-prioritized and its simultaneous transmission with the SR is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCHgroups, and the priority of the uplink grant is determined as specified in clause 5.4.1; or 3> if both sl-PrioritizationThres and ul-PrioritizationThres are configured and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5 overlaps with any UL-SCH resource(s) carrying a MAC PDU, and the value of the priority of the triggered SR determined as specified in clause 5.22.1.5 is lower than sl-PrioritizationThres and the value of the highest priority of the logical channel(s) in the MAC PDU is higher than or equal to ul-PrioritizationThres and any MAC CE prioritized as described in clause 5.4.3.1.3 is not included in the MAC PDU and the MAC PDU is not prioritized by upper layer according to TS 23.287 [19]; or This condition is fulfilled 3> if an SL-SCH resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource, and either transmission on the SL-SCH resource is not prioritized as described in clause 5.22.1.3.1a or the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres, if configured; or … 4> consider the SR transmission as a prioritized SR transmission. This uplink grant is de-prioritized even if the priority is higher 4> consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s), except for the overlapping uplink grant(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups;

Observation 2: In the current specification, the UE will wrongly consider SR transmission to be prioritized when the SR is prioritized over only one of the overlapping SL and UL transmission.

To address these issues, We propose a text proposal in the Annex to 1) add a condition for the UE to prioritize SR over SL-PRS that can be transmit simultaneously, and 2) restructure the condition check for SR prioritization so that the UE can consider prioritization results for all overlapping UL and SL transmissions before deciding whether the SR transmission is to be prioritized or not. The restructuring is to make the UE firstly consider SR prioritization between overlapping Uu transmissions, if any, and if the prioritization against UL is fulfilled, the UE performs prioritization between overlapping SL-SCH and SL-PRS, if any, and derives the final result.

Proposal: RAN2 Adopts the Text proposal in Annex so that the UE can

- consider the SR transmission as prioritized SR transmission when an SL-PRS resource overlaps with the SR transmission and the UE can perform the SR transmission simultaneously with the SL-PRS.

- Determining whether to prioritize the SR transmission by considering all overlapping SL and UL transmissions.

- Conclusion

In this contribution, we have following observations and proposal:

Observation 1: In the current specification, the UE does not consider SR transmission to be prioritized when the UE has an overlapping SL-PRS transmission that is able to be performed simultaneously with the SR transmission.

Observation 2: In the current specification, the UE will wrongly consider SR transmission to be prioritized when the SR is prioritized over only one of the overlapping SL and UL transmission.

Proposal: RAN2 Adopts the Text proposal in Annex so that the UE can

- consider the SR transmission as prioritized SR transmission when an SL-PRS resource overlaps with the SR transmission and the UE can perform the SR transmission simultaneously with the SL-PRS.

- Determining whether to prioritize the SR transmission by considering all overlapping SL and UL transmissions.

- Reference

- Annex: Possible TP for TS 38.321

### 5.4.4	Scheduling Request

…

As long as at least one SR is pending, the MAC entity shall for each pending SR:

1>	if the MAC entity has no valid PUCCH resource configured for the pending SR; and

1>	if there is no ongoing RACH-less LTM cell switch; and

1>	if rach-LessHO is not configured:

2>	initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR.

1>	else, for the SR configuration corresponding to the pending SR:

2>	when the MAC entity has an SR transmission occasion on the valid PUCCH resource for SR configured; and

2>	if sr-ProhibitTimer is not running at the time of the SR transmission occasion; and

2>	if the PUCCH resource for the SR transmission occasion does not overlap with a measurement gap:

3>	if the PUCCH resource for the SR transmission occasion does not overlap with a UL-SCH resource whose simultaneous transmission with the SR is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup; or

3>	if the MAC entity is able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource; or

3>	if the MAC entity is configured with lch-basedPrioritization, and the PUCCH resource for the SR transmission occasion does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or with the PUSCH duration of a MSGA payload, and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5 overlaps with any other UL-SCH resource(s), and the physical layer can signal the SR on one valid PUCCH resource for SR, and the priority of the logical channel that triggered SR is higher than the priority of the uplink grant(s) for any UL-SCH resource(s) where the uplink grant was not already de-prioritized and its simultaneous transmission with the SR is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCHgroups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the uplink grant is determined as specified in clause 5.4.1; or

3>	if both sl-PrioritizationThres and ul-PrioritizationThres are configured and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5 overlaps with any UL-SCH resource(s) carrying a MAC PDU, and the value of the priority of the triggered SR determined as specified in clause 5.22.1.5 is lower than sl-PrioritizationThres and the value of the highest priority of the logical channel(s) in the MAC PDU is higher than or equal to ul-PrioritizationThres and any MAC CE prioritized as described in clause 5.4.3.1.3 is not included in the MAC PDU and the MAC PDU is not prioritized by upper layer according to TS 23.287 [19]:

4>	if the PUCCH resource for the SR transmission occasion overlaps with neither an SL-SCH resource nor a SL-PRS resource; or

4>if an SL-SCH resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR, and	 the MAC entity is able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource; or

4>	if an SL-SCH resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource, and either transmission on the SL-SCH resource is not prioritized as described in clause 5.22.1.3.1a or the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres, if configured; or

4>	if an SL-SCH resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource, and the priority of the triggered SR determined as specified in clause 5.22.1.5 is higher than the priority of the MAC PDU determined as specified in clause 5.22.1.3.1a for the SL-SCH resource; or

4>	if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR, and the MAC entity is able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource; or

4>	if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and either transmission on the SL-PRS resource is not prioritized as described in clause 5.22.1.3.1a or the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres, if configured; or

4>	if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and the priority of the triggered SR determined as specified in clause 5.22.1.5 is higher than the priority of the MAC PDU and SL-PRS, if available, determined as specified in clause 5.22.1.3.1a for the SL-PRS resource:

5>	consider the SR transmission as a prioritized SR transmission.

5>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s), except for the overlapping uplink grant(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup;

5>	if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started:

6>	stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s);

6>	stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

5>	if SR_COUNTER < sr-TransMax:

6>	instruct the physical layer to signal the SR on one valid PUCCH resource for SR;

6>	if LBT failure indication is not received from lower layers:

7>	increment SR_COUNTER by 1;

7>	start the sr-ProhibitTimer.

6>	else if lbt-FailureRecoveryConfig is not configured:

7>	increment SR_COUNTER by 1.

5>	else:

6>	notify RRC to release PUCCH for all Serving Cells;

6>	notify RRC to release SRS for all Serving Cells;

6>	clear any configured downlink assignments and uplink grants;

6>	clear any PUSCH resources for semi-persistent CSI reporting;

6>	if rach-LessHO is not configured and if there is no ongoing RACH-less LTM cell switch:

7>	initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel all pending SRs.

4>	else:

5>	consider the SR transmission as a de-prioritized SR transmission.

…