3GPP TSG-RAN WG2 Meeting #127	R2-2406519

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:		7.20.2

Source:				ASUSTeK

Title:		Discussion on remaining issue for 8Tx in MAC specification

Document for: 	Discussion and Decision

- Introduction

8Tx UL has been introduced by RAN1 in release 18 MIMO. In the last meeting, some corresponding changes for TS 38.321 were discussed. In this contribution, we further discuss the remaining issue of the NOTE from LTE MAC regarding MAC PDU generation.

- Discussion

In NR release 18, 8Tx UL was introduced for MIMO. According to PHY and RRC, the maxRank and maxMIMO-Layers could be configured above 4 (up to 8), which indicates the maximum MIMO layer to be used for PUSCH. Based on the configuration, there could be up to two code words for PUSCH. That is to say, the UE could transmit up to two TBs at a same timing.

In the last meeting, it was agreed to capture “Each HARQ process supports one or two TBs” in MAC for UL HARQ. Moreover, as discussed offline in [201] as following [1], the note for multiplexing and assembly could be postponed:

R2-2405426 Discussion on introducing 8Tx in MAC specification ASUSTeK discussion Rel-18 38.321 NR_MIMO_evo_DL_UL-Core Observation: 8Tx which supports up to two TBs for a PUSCH has been introduced in PHY, RRC and stage 2 specifications but no corresponding changes have been made in MAC specification. Q10: Similar to DL HARQ, for UL HARQ, each HARQ process supports one or two TBs. add one sentence in MAC 5.4.2.1 HARQ Entity AsusTek: 8Tx already in RRC and stage-2, MAC changes are missing LG: agree some changes are needed, TP regarding two UL grant is not needed, we only need “Each HARQ process supports one or two TBs.”, ZTE, OPPO agrees AsusTek: ok for one UL grant, but the note may be needed OPPO , ZTE, LG: the note is assuming 2 UL grants, so not needed either Ericsson, Nokia: the note can be postponed - Keep the change “Each HARQ process supports one or two TBs.”

In LTE, when the UE is configured to transmit up to two TBs at a same timing and receives two UL grants for two TBs, the UE needs to generate both MAC PDUs once one MAC PDU is to be generated. More specifically, according to TS 36.321 as below [2], when the PHY is configured for uplink spatial multiplexing, the MAC can receive two UL grants for two TBs for the same TTI. And if at least one MAC PDU is to be generated for this TTI, the MAC generates MAC PDUs corresponding to all UL grants for this TTI (even if a condition for UL skipping may be fulfilled).

Observation 1: In LTE, the UE generates both MAC PDUs if one MAC PDU is to be generated, when the UE receives two UL grant for the TTI.

5.4.1 UL Grant reception In order to transmit on the UL-SCH the MAC entity must have a valid uplink grant (except for non-adaptive HARQ retransmissions) which it may receive dynamically on the PDCCH or in a Random Access Response or which may be configured semi-persistently or preallocated by RRC or provided by RRC for transmission using PUR (see clause 5.4.7). To perform requested transmissions, the MAC layer receives HARQ information from lower layers. When the physical layer is configured for uplink spatial multiplexing, the MAC layer can receive up to two grants (one per HARQ process) for the same TTI from lower layers. … ### 5.4.3 Multiplexing and assembly #### 5.4.3.1 Logical channel prioritization The Logical Channel Prioritization procedure is applied when a new transmission is performed. … If the MAC PDU includes only the MAC CE for padding BSR or periodic BSR with zero MAC SDUs and there is no aperiodic CSI requested for this TTI, as specified in TS 36.213 [2], the MAC entity shall not generate a MAC PDU for the HARQ entity in the following cases: - in case the MAC entity is configured with skipUplinkTxDynamic and the grant indicated to the HARQ entity was addressed to a C-RNTI; or - in case the MAC entity is configured with skipUplinkTxSPS and the grant indicated to the HARQ entity is a configured uplink grant activated by the MAC entity's Semi-Persistent Scheduling C-RNTI or by the MAC entity's UL Semi-Persistent Scheduling V-RNTI; or - in case the grant indicated to the HARQ entity is a configured uplink grant activated by the MAC entity's AUL C-RNTI; or - in case the grant indicated to the HARQ entity is a preconfigured uplink grant. NOTE 1a: If at least one MAC PDU is to be generated for the HARQ entity for this TTI, the MAC entity generates MAC PDUs corresponding to all UL grants indicated to the HARQ entity for this TTI. …

The intention for adding the NOTE in LTE is that PHY is unable to perform corresponding transmission properly when MAC generates only one TB for UL spatial multiplexing, as a two TB transmission is scheduled by DCI and expected by eNB. When UL spatial multiplexing is configured, MAC can receive two dynamic UL grants at a given TTI and PHY would expect to receive two TBs from MAC, as PHY is unable to perform corresponding transmission if only one TB is generated from MAC. Without NOTE 1a, whether to generate MAC PDU is determined separately for each MAC PDU, so that the HARQ entity would generate only one MAC PDU if there is insufficient available data at the TTI. The Multiplexing and assembly entity would not generate the MAC PDU comprising pure padding when skipUplinkTxDynamic is configured, resulting in only one MAC PDU being generated.

Observation 2: The intention for the NOTE 1a in LTE is that PHY is unable to perform corresponding transmission properly with only one TB when two TBs are expected for UL spatial multiplexing.

As a result, this NOTE is needed irrespective of how many UL grants are received. For 8Tx UL in NR, even if one UL grant is used to schedule two TBs, whether to generate MAC PDU is still determined separately for each MAC PDU so that the same issue in LTE UL spatial multiplexing occurs. That is, in NR, when UE receives one UL grant for two TBs and there is insufficient available data, the Multiplexing and assembly entity would not generate the MAC PDU comprising pure padding when skipUplinkTxDynamic is configured and thus results in only one MAC PDU generated. Therefore, to address the same issue, it is proposed to add a similar note in NR as shown in the TP in Annex. If deemed necessary, an LS could be sent to RAN1 to verify whether it’s feasible in NR for MAC to generate only one TB when the DCI indicates two TBs for 8Tx.

Proposal 1: Add a NOTE in Multiplexing and assembly based on NOTE 1a in TS 36.321 section 5.4.3.1 to ensure generation of both MAC PDUs in 8Tx scenario.

Proposal 2: If Proposal 1 is agreed, RAN2 adopts the TP in Annex.

Proposal 3: If Proposal 1 cannot be agreed, RAN2 to discuss whether to send an LS to RAN1 to verify whether it’s feasible in NR for MAC to generate only one TB when the DCI indicates two TBs for 8Tx.

- Conclusion

In this contribution, we have following observations and proposals:

Observation 1: In LTE, the UE generates both MAC PDUs if one MAC PDU is to be generated, when the UE receives two UL grant for the TTI.

Observation 2: The intention for the NOTE 1a in LTE is that PHY is unable to perform corresponding transmission properly with only one TB when two TBs are expected for UL spatial multiplexing.

Proposal 1: Add a NOTE in Multiplexing and assembly based on NOTE 1a in TS 36.321 section 5.4.3.1 to ensure generation of both MAC PDUs in 8Tx scenario.

Proposal 2: If Proposal 1 is agreed, RAN2 adopts the TP in Annex.

Proposal 3: If Proposal 1 cannot be agreed, RAN2 to discuss whether to send an LS to RAN1 to verify whether it’s feasible in NR for MAC to generate only one TB when the DCI indicates two TBs for 8Tx.

- Reference

- R2-2405733, “Report of [AT126][201][MIMOevo] remaining MAC issues (Samsung).”

- 3GPP TS 36.321 V18.1.0 (2024-03).

- Annex: Possible TP for the NOTE in TS 38.321

### 5.4.3	Multiplexing and assembly

#### 5.4.3.1	Logical Channel Prioritization

##### 5.4.3.1.3	Allocation of resources

…

The MAC entity shall:

1>	if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

1>	else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

NOTE 1a:	If at least one MAC PDU is to be generated for the HARQ process, the MAC entity generates MAC PDUs corresponding to the UL grant indicated to the HARQ process.