3GPP TSG RAN WG2 #127				R2-2406521

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.2.4

Source:		ASUSTeK

Title:				Discussion on Ambient IoT access message

Document for: Discussion and Decision

- Introduction

In RAN2#126 meeting [1], the following agreements on Ambient IoT device random access were made:

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.  “Msg2” usage/presence can be further discussed

And the following agreements on Ambient IoT device paging were made:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In RAN1#117 meeting [2], the following agreements on Ambient IoT channel design were made:

Agreement

Scheduling information of PDRCH transmission is provided by a corresponding PRDCH.

Agreement

For R2D, the only physical channel is PRDCH.

- PRDCH carries any higher-layer payload

- PRDCH carries L1 R2D control information (if defined)

- FFS details of device behaviour(s) for receiving PRDCH

Agreement

For D2R

- PDRCH carries any higher-layer payload

- PDRCH carries L1 D2R control information (if defined)

- Note: PDRCH carries the response agreed at RAN1#116

- Discussion

In RAN2#126 meeting, it was agreed that A-IoT paging message indicates information from which the device can determine resources to be used for response (i.e. A-IoT Msg1). For a “4-step” (or so called 3-step) access procedure, the device would perform the A-IoT Msg1 transmission after receiving the A-IoT paging message. The device determines its resource (e.g. time resource and/or frequency resource) for the A-IoT Msg1 transmission based on the received information form the A-IoT paging message. After performing the A-IoT Msg1 transmission, the device would receive a corresponding A-IoT Msg2. As agreed in the last meeting, the A-IoT Msg2 includes information of whether the contention resolution is successful (e.g. by echoing the ID in A-IoT Msg1). After receiving the A-IoT Msg2, the device performs a A-IoT Msg3 transmission for data transmission if the contention resolution is successful.

For the A-IoT Msg3 transmission, the time and/or frequency resource should also be indicated by the network. For example, the resource for the A-IoT Msg3 could be explicitly included in the A-IoT Msg2. In this case, decoding complexity for the device may increase in the presence of larger message size of A-IoT Msg2. On the other hand, the resource could be implicitly provided to the device. For example, the resource for A-IoT Msg3 transmission could be associated with the resource used for A-IoT Msg1 transmission. The device could apply the same frequency resource as the one used for A-IoT Msg1 transmission, since there may not be collision on the same resource for the device successful in contention resolution. However, it may be less flexible for the network to allocate the resource. Additionally, the indication of A-IoT Msg3 resource could be a hybrid method of the two.

Proposal 1:  The device determines resource to perform the A-IoT Msg3 transmission based on network indication:

Option 1: the resource for A-IoT Msg3 is indicated in A-IoT Msg2;

Option 2: the resource for A-IoT Msg3 is associated with the resource for A-IoT Msg1.

On the other hand, if the device receives A-IoT Msg2 but the contention resolution is not successful, e.g., the ID included in A-IoT Msg2 does not match the ID transmitted by the device in A-IoT Msg1, the device should make another attempt to transmit A-IoT Msg1. Based on RAN1 agreement that scheduling information of PDRCH transmission is provided by a corresponding PRDCH, similar to the case of A-IoT Msg1 transmission triggered by A-IoT paging message, where the device determines the resources for A-IoT Msg1 transmission based on scheduling information in A-IoT paging message, the A-IoT Msg2 could include scheduling information for the another A-IoT Msg1 transmission.

Proposal 2:  If the contention resolution is not successful, the device performs another A-IoT Msg1 transmission with the resource indicated in A-IoT Msg2.

- Conclusion

Proposal 1:  The device determines resource to perform the A-IoT Msg3 transmission based on network indication:

Option 1: the resource for A-IoT Msg3 is indicated in A-IoT Msg2;

Option 2: the resource for A-IoT Msg3 is associated with the resource for A-IoT Msg1.

Proposal 2:  If the contention resolution is not successful, the device performs another A-IoT Msg1 transmission with the resource indicated in A-IoT Msg2.

- Reference

[1] Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

[2] RAN1 Chair’s Notes for 3GPP TSG RAN WG1 #117 (Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024)