3GPP TSG-RAN WG2 Meeting #127	R2-2406522

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.2.5

Source:			ASUSTeK

Title:	Consideration on resource and scenario in topology 2

Document for: 	Discussion and Decision

- Introduction

In TR 38.848 [1], connectivity topologies for Ambient IoT networks and devices are defined. Among the connectivity topologies, topology 2 is specified as below:

### 4.2.1 Connectivity topologies #### 4.2.1.0 Introduction The following connectivity topologies for Ambient IoT networks and devices are defined for the purposes of the study. In all these topologies, the Ambient IoT device may be provided with a carrier wave from other node(s) either inside or outside the topology. The links in each topology may be bidirectional or unidirectional. BS, UE, assisting node, or intermediate node could be multiple BSs or UEs, respectively. The mixture of indoor and outdoor placement of such nodes is regarded as a network implementation choice. Account would need to be taken of potential impact on device or node complexity. In the connectivity topologies, this does not imply the existence of multi-hop assisting or intermediate nodes. #### 4.2.1.2 Topology 2: BS ↔ intermediate node ↔ Ambient IoT device Figure 4.2.1.2-1: Topology 2 In Topology 2, the Ambient IoT device communicates bidirectionally with an intermediate node between the device and basestation. In this topology, the intermediate node can be a relay, IAB node, UE, repeater, etc. which is capable of Ambient IoT. The intermediate node transfers Ambient IoT data and/or signalling between BS and the Ambient IoT device.

In this contribution, we would like to discuss how to allocate resources for A-IoT transmissions between the device and the reader in Topology 2.

- Discussion

In the study of Ambient IoT, it has been agreed to study Topology 2 in addition to Topology 1. As specified in TR 38.848 [1], an intermediate node is introduced to support data or signalling transmission between the base station and the device in Topology 2. The Ambient IoT device communicates with the intermediate node, which transfers the Ambient IoT data/signalling to BS (or gNB). Compared to Topology 1, the intermediate node is involved to play a role of the reader in a D2R transmission or a R2D transmission with the Ambient IoT device.

For R2D transmission, the transmission is originated from the reader, and for D2R transmission, since only DT and DO-DTT transmissions are considered at the moment, a D2R transmission from an Ambient IoT device should also be triggered by the reader.

In Topology 1, when the gNB plays the role of the reader, the gNB could determine to initiate a R2D transmission or trigger a D2R transmission based on the request from CN. It has been agreed in the last meeting that AIoT paging message indicates scheduling information for the Ambient IoT device to determine resource to be used for D2R transmission. Therefore, the transmission resource for both R2D transmission and D2R transmission should be determined by the gNB in Topology 1.

In Topology 2, when the intermediate node plays the role of the reader, the intermediate node may have no means to determine whether to initiate a R2D transmission or to trigger a D2R transmission by itself, and the determination should still come from CN request, as in Topology 1. If this is the case, since the intermediate node would be connected to a gNB, when the gNB receives a request from CN, the gNB should inform the intermediate node to initiate a R2D transmission or to trigger a D2R transmission. Similarly, resource scheduling of the intermediate node should also be controlled by the gNB. Therefore, the resource used for the R2D transmission or the D2R transmission should be determined by the gNB.

Proposal 1:  For topology 2, the resources for transmissions between the device and the reader should be determined by gNB.

If proposal 1 is agreed, the gNB should determine the resource used for R2D transmission or D2R transmission. And the gNB should indicate the scheduling information for the R2D transmission or D2R transmission to the intermediate node. For example, when the gNB informs the intermediate node to initiate a R2D transmission or to trigger a D2R transmission, the gNB could also provide the scheduling information to the intermediate node.

Proposal 2: The gNB provides scheduling information to the intermediate node for the transmissions between the device and the reader.

On the other hand, in RAN1#116bis meeting [2], several scenarios for topology 2 are defined as following:

| Scenario | CW Inside/outside topology | Diagram of the scenario | Description of the scenario | Device 1/2a/2b | CW spectrum | D2R spectrum |
| --- | --- | --- | --- | --- | --- | --- |
| D2T2-A1 | CW inside topology |  | - CW node inside topology 2 - ‘CW’ in CW2D and ‘R2’ in D2R are different - ‘CW’ in CW2D and ‘R1’ in R2D are same - ‘R1’ in R2D and ‘R2’ in D2R are different - BS communicates with R1 and R2 | Device 1, 2a | Case 2-2 (inside topology, UL) | Same as CW |
| D2T2-A2 |  |  | - CW node inside topology 2 - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D - BS communicates with R |  | Same as D2T2-A1 | Same as CW |
| D2T2-B | CW outside topology |  | - CW node outside topology 2 - ‘CW’ in CW2D and ‘R’ in D2R are different - ‘CW’ in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same - BS communicates with R |  | Case 2-3 (outside topology, DL) Case 2-4 (outside topology, UL) | Same as CW |
| D2T2-C | No CW |  | - No CW Node. - BS communicates with R | Device 2b | N/A | FFS |

When the carrier wave is inside the topology, D2T2-A1 refers to the case that the intermediate node for D2R transmission and the one for R2D transmission are different. On the contrary, D2T2-A2 stands for the case that the intermediate node for D2R transmission and the one for R2D transmission are the same.

Observation 1: Based on RAN1’s agreement, the intermediate node for R2D transmission and D2R transmission could be the same or different.

Since the intermediate node (e.g. a UE) is assigned by the gNB (or the network), the operation and configuration should be under network control. If there are two intermediate nodes to serve a device, there should be alignment for the gNB to make. For example, the resource used between gNB and each intermediate node, and the resource used between each intermediate node and the devices should be allocated without collision or interference. The gNB provides scheduling information for R2D transmission to R2D intermediate node; furthermore, it provides scheduling information for D2R transmission to R2D intermediate node (e.g. to include it in the PRDCH) and D2R intermediate node (e.g. to know where to receive D2R transmission). Additionally, the required information used to assist corresponding R2D transmission should be aware to the R2D intermediate node after receiving at the D2R intermediate node. As a result, RAN2 could study the impact to procedures with different intermediate nodes for R2D and D2R transmission.

Proposal 3:  If there are two intermediate nodes in Topology 2, the gNB provides scheduling information for R2D transmission to R2D intermediate node. In addition, the gNB provides scheduling information for D2R transmission to R2D intermediate node and D2R intermediate node.

- Conclusion

Proposal 1:  For topology 2, the resources for transmissions between the device and the reader should be determined by gNB.

Proposal 2: The gNB provides scheduling information to the intermediate node for the transmissions between the device and the reader.

Observation 1: Based on RAN1’s agreement, the intermediate node for R2D transmission and D2R transmission could be the same or different.

Proposal 3:  If there are two intermediate nodes in Topology 2, the gNB provides scheduling information for R2D transmission to R2D intermediate node. In addition, the gNB provides scheduling information for D2R transmission to R2D intermediate node and D2R intermediate node.

- Reference

[1] 3GPP TR 38.848 v18.0.0 (2023-09), ”Study on Ambient IoT (Internet of Things) in RAN”

[2] RAN1#116bis meeting minute