3GPP TSG-RAN WG2 Meeting #127	R2-2406524

Maastricht, Netherlands, 19 – 23 August 2024

Agenda Item:	8.6.3

Source: 	ASUSTeK

Title:  	Discussion on beam for evaluation of LTM event-triggered reporting

Document for:	Discussion and Decision

- Introduction

RAN2#126 has agreed that the following events are used for triggering L1 measurement reporting for LTM, i.e. beams for event evaluation includes beam(s) of a serving cell and beam(s) of a LTM candidate cell [1]:

Agreements on measurement enhancements for LTM:

…

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

And this document discusses the FFS part that which beam(s) of serving cell and beam(s) of LTM candidate cell are used for LTM event evaluation.

- Discussion

RAN1 also has similar discussion on the events for triggering L1 measurement reporting, but the events are for MIMO, i.e. beams for event evaluation only include beams of the same serving cell. So far, the event for MIMO and beam(s) for event evaluation agreed by RAN1 are [2][3]:

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

Agreement

On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state.

Agreement

Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported

- Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured

The intention of the Event-2 for MIMO is to let network be aware of new beam(s) of a serving cell with a threshold better than the current beam, corresponding to the indicated TCI state, of the serving cell. In other words, the Event-2 for MIMO is designed for the case that the current beam is not the best beam. According to the report of the Event-2 for MIMO, network may change the current beam corresponding to the indicated TCI state accordingly. Or network may keep the current beam considering traffic load of the new beam(s) if the current beam is still good enough.

Besides, Event-2 for MIMO would not be triggered if there exists a new beam with quality better than quality of the current beam, but is not a threshold value better than the current beam. In such case, the current beam would also not be changed to the new beam instantly.

Observation:	Current beam, corresponding to the indicated TCI state, of a serving cell is not always the best beam of the serving cell.

Beam(s) of serving cell

If the beam of the serving cell used for LTM event evaluation also chooses the current beam corresponding to the indicated TCI state, it may result in unnecessary L1 measurement reporting for LTM (and serving cell change) when there exists a good enough new beam in the source serving cell. L1 measurement reporting for LTM (and serving cell change) should be triggered only when there is no good enough new beam in the serving cell. To guarantee L1 measurement reporting for LTM (and serving cell change) happening only when necessary, the beam of the serving cell used for LTM event evaluation should be the best beam of the serving cell.

Proposal 1:	Beam of a serving cell used for evaluation of LTM event-triggered L1 measurement reporting is the best beam of the serving cell.

Besides, LTM does not need to be executed due to poor radio condition of a SCell. Instead, the SCell could be either deactivated or released. LTM should be executed only when radio condition of SpCell for MCG or SCG becomes not acceptable. Therefore, the serving cell used for LTM event evaluation should be the SpCell for MCG or SCG.

Proposal 2:	Serving cell used for evaluation of LTM event-triggered L1 measurement reporting is SpCell for MCG or SCG.

Beam(s) of candidate cell

Similarly, to guarantee L1 measurement reporting for LTM (and serving cell change) happening only when necessary, the beam(s) of a candidate cell used for LTM event evaluation should include the best beam of the candidate cell as the baseline. Considering the concern on ping-pong LTM, whether to use more than one beam of the candidate cell, e.g. N-best beams with N>1, for LTM event evaluation could be FFS.

Proposal 3:	Beam(s) of a candidate cell used for evaluation of LTM event-triggered L1 measurement reporting includes the best beam of the candidate cell.

Proposal 4:	FFS on whether to use N-best beams with N>1 of the candidate cell for evaluation of LTM event-triggered L1 measurement reporting.

- Conclusion

Regarding LTM event-triggered L1 measurement reporting, we have the following observation and proposals:

Observation:	Current beam, corresponding to the indicated TCI state, of a serving cell is not always the best beam of the serving cell.

Proposal 1:	Beam of a serving cell used for evaluation of LTM event-triggered L1 measurement reporting is the best beam of the serving cell.

Proposal 2:	Serving cell used for evaluation of LTM event-triggered L1 measurement reporting is SpCell for MCG or SCG.

Proposal 3:	Beam(s) of a candidate cell used for evaluation of LTM event-triggered L1 measurement reporting includes the best beam of the candidate cell.

Proposal 4:	FFS on whether to use N-best beams with N>1 of the candidate cell for evaluation of LTM event-triggered L1 measurement reporting.

- Reference

- RAN2#126 meeting minute.

- RAN1#116bis meeting minute.

- RAN1#117 meeting minute.