3GPP TSG-RAN WG2 Meeting #127	R2-2406525

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.7.2

Source:			ASUSTeK

Title:	Discussion on DRX enhancements for multi-modality

Document for: 	Discussion and Decision

- Introduction

As specified in the work item [1], to facilitate efficient support for XR application with multi-modality, one potential enhancement is supporting multiple DRX configurations. In this contribution, we would like to further discuss the DRX enhancements in XR.

# 3 Justification This WI aims for enhancements addressing system capacity, efficient and effective mechanisms to meet QoS requirements and lower device power consumption, in the context of the demanding scenarios and traffic characteristics requirements of XR, including also multi-modal applications. This WI aims to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination, see also TR 22.847, TR 23.700 60. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. Related Potential Impacts has been proposed as follows: a) Enhanced RAN Awareness, by signaling from Core Network and/or indication by UE, b) Enhancements User Plane, e.g. Scheduling, LCP, Resource allocation, Discard. c) Support for multiple DRX configurations, without the potentially problematic restrictions of Pre-Rel-19 2nd DRX.

- Discussion

In the XR applications, the XR traffic would have variable data rates and require low latency. XR would have quasi-periodic traffic (with possible jitter) with high data rate in DL (e.g. video steam) and frequent traffic in UL (e.g. pose/control update). On the other hand, XR devices are expected to be mobile and of small-scale, thus having limited battery power resources. It may be needed to reduce the overall UE power consumption when running XR services, and thus extend the battery lifetime of the UE.

However, the current DRX mechanism do not fit well for XR service characteristics (e.g., quasi-periodic periodicity, multiple flows, jitter). The UE would not closely follow the short sleep opportunities to reduce power consumption. On the other hand, the UE would not monitor PDCCH for some data flows arrival, which causes additional traffic delay.

To solve the issues, multiple DRX configurations could be supported for multiple flows of XR. If multiple DRX configurations are supported, the UE could be configured with multiple DRX configurations in a serving cell. Each DRX configuration is associated with a different data flow. And the UE uses more than one of the DRX configurations simultaneously. That is to say, the UE monitors the PDCCH while the drx-onDurationTimer or drx-InactivityTimer is running in any of the activated DRX configurations. How many DRX configurations can be configured/activated at the same time could be further studied.

Proposal 1: Multiple DRX configurations are supported in XR to reduce power consumption and transmission latency.

In addition, the XR characteristics of each data flow may be changed during ongoing XR service. To adapt with variable XR traffic and to avoid too frequent RRC reconfigurations, it would be beneficial for the network to send a L1 or L2 signalling to change the activated DRX configuration(s). For example, for an upcoming type of data flow for a UE, the NW could notify the UE with activating the corresponding DRX configuration. The L1 or L2 signalling could activate, deactivate or switch DRX configuration(s). This would allow dynamic adaptation of the multiple DRX configurations in response to change of XR traffic characteristic.

Proposal 2: To adapt the DRX mechanism with variable XR traffic, the network could send a L1/L2 signalling to change the activated DRX configuration(s).

- Conclusion

In this contribution, we discuss the DRX enhancements for XR multi-modality, and have the following proposals:

Proposal 1: Multiple DRX configurations are supported in XR to reduce power consumption and transmission latency.

Proposal 2: To adapt the DRX mechanism with variable XR traffic, the network could send a L1/L2 signalling to change the activated DRX configuration(s).

- Reference

- RP-240791, “Revised WID on XR (eXtended Reality) for NR Phase 3.”

- 3GPP TS 38.321 V18.1.0