3GPP TSG-RAN WG2 Meeting #127	R2-2406528

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.13.2

Source:	ASUSTeK

Title:	Discussions on relay discovery for multi-hop U2N Relay

Document for:	Discussion and Decision

1	Introduction

According to the objective of the new WID on NR sidelink multi-hop relay [1], relay discovery and (re)selection should be specified to support multi-hop U2N Relay.

In this contribution, we discuss NR sidelink U2N Relay UE threshold conditions for discovery message transmission or forwarding to support multi-hop U2N Relay.

2	Discussion

2.1	Current status of multi-hop U2N Relay in TR 23.700-03 [2]

The following figure (quoted from clause 6.1.1 in [2]) illustrates an example architecture of multi-hop UE-to-Network Relay, where a 5G ProSe Remote UE connects with the NG-RAN via an Intermediate UE-to-Network Relay and a 5G ProSe UE-to-Network Relay.

Figure 1: Example architecture of multi-hop UE-to-Network Relay

Several conclusions related to multi-hop UE-to-Network Relay discovery in clause 8.1 of [2] are quoted below for reference:

- Standalone discovery & link setup/management is considered and both Model A and Model B are supported. - When Model A discovery is performed, the Remote UE may choose the Intermediate Relay based on the announcement message sent by the Intermediate Relay. - When Model B discovery is performed, the Remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay. To perform link management, the DCR message is unicasted between Relays according to the path information included in the message. The path information is an (ordered) list of User Info ID of Relays in the selected path. The Remote UE sends the selected path information to the Intermediate UE-to-Network Relay for communication setup. - The 5G ProSe Direct Discovery message will be extended with an indication that multi-hop relay is supported, along with the hop count and the maximum number of hops. - The maximum number of hops is provisioned by the network to the 5G ProSe Remote UE, Intermediate Relay and UE-to-Network Relays per RSC. - The 5G ProSe Direct Discovery message may be dropped if the maximum number of hops is reached. - The relay path is selected based on e.g. the PC5 signal strength, number of hops between the Remote UE and the UE-to-Network Relay UE, per-hop or cumulative QoS information. The path selection criteria and the support of these metrics can be determined during the normative phase. - Details about remote UE report and additional discovery message handling can be determined in the normative phase.

According to Figure 1, it is clear only the 5G ProSe UE-to-Network Relay needs to be in coverage of NG-RAN and be able to establish a RRC connection with the NG-RAN so as to support multi-hop UE-to-Network Relay. Other Intermediate UE-to-Network Relay(s) can be either in coverage or out of coverage.

Proposal 1: To support multi-hop U2N Relay, only the 5G ProSe U2N Relay needs to be in coverage of NG-RAN and other Intermediate U2N Relay(s) can be either in coverage or out of coverage.

According to the above conclusions captured in [2], both Model A and Model B discovery mechanisms are supported for multi-hop U2N Relay. Figure 2 shows an example of multi-hop U2N Relay discovery procedure with Model B. Basically, a Remote UE may initiate a discovery procedure and broadcast a U2N Relay Discovery Solicitation message. When receiving the U2N Relay Discovery Solicitation message from the Remote UE, a potential Intermediate U2N Relay may forward (information included in) the U2N Relay Discovery Solicitation message to 5G ProSe U2N Relay(s) in proximity. A 5G ProSe U2N Relay may reply with a U2N Relay Discovery Response message when receiving the U2N Relay Discovery Solicitation message from the Intermediate U2N Relay. In response to reception of the U2N Relay Discovery Response message from the 5G ProSe U2N Relay, the Intermediate U2N Relay may also transmit a U2N Relay Discovery Response message to the Remote UE. The Remote UE may then initiate the PC5 RRC connection establishments toward the Intermediate U2N Relay and the 5G ProSe U2N Relay according to the path info included in the received U2N Relay Discovery Response message.

Remote UE

NW

Intermediate U2N Relay

PC5 RRC connection

5G U2N Relay

U2N Relay Discovery Response

(Path info, RSC, hop-count)

U2N Relay Discovery Response

(Path info, RSC, hop-count)

PC5 RRC connection

RRC connection

U2N Relay Discovery Solicitation

(Remote UE User Info, RSC)

U2N Relay Discovery Solicitation

(Remote UE User Info, RSC)

Figure 2. An example of multi-hop U2N Relay discovery with Model B

2.2	NR sidelink U2N Relay UE threshold conditions for discovery message transmission in single-hop U2N Relay

According to clause 5.8.13.3 of Release 18 3GPP TS 38.331 [3], a UE capable of NR sidelink U2N Relay UE operation that is configured by upper layer to transmit the NR sidelink discovery message shall examine if the NR sidelink U2N Relay UE threshold conditions specified in 5.8.14.2 [3] can be met. The UE shall not transmit the NR sidelink discovery message if the NR sidelink U2N Relay UE threshold conditions cannot be met. Basically, the U2N Relay UE examines whether the Reference Signal Received Power (RSRP) measurement of the serving or camping cell is between two threshold values derived by threshHighRelay, hystMaxRelay, threshLowRelay, and/or hystMinRelay. Clause 5.8.14 in [3] is quoted below for reference:

### 5.8.14 NR sidelink U2N Relay UE operation #### 5.8.14.1 General This procedure is used by a UE supporting NR sidelink U2N Relay UE operation configured by upper layers to transmit NR sidelink discovery messages to evaluate AS layer conditions. #### 5.8.14.2 NR sidelink U2N Relay UE threshold conditions A UE capable of NR sidelink U2N Relay UE operation shall: 1> if the threshold conditions specified in this clause were previously not met: 2> if threshHighRelay is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHighRelay by hystMaxRelay if configured; and 2> if threshLowRelay is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshLowRelay by hystMinRelay if configured: 3> consider the threshold conditions to be met (entry); 1> else: 2> if the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshHighRelay if configured; or 2> if the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshLowRelay if configured; 3> consider the threshold conditions not to be met (leave);

According to the above paragraph, the NR sidelink U2N Relay UE threshold conditions includes two cases:

Case 1. the threshold conditions were previously not met; and

Case 2. the threshold conditions were previously met.

For Case 1, the UE shall consider the threshold conditions to be met if the RSRP measurement of the serving or camping cell is below threshHighRelay by hystMaxRelay and above threshLowRelay by hystMinRelay. For Case 2, the UE shall consider the threshold conditions not to be met if the RSRP measurement of the serving or camping cell is above threshHighRelay or below threshLowRelay. Otherwise, it remains to be met. In other words, the RSRP should be between the high RSRP threshold (i.e. threshHighRelay – hystMaxRelay for Case 1 and threshHighRelay for Case 2) and the low RSRP threshold (i.e. threshLowRelay + hystMinRelay for Case 1 and threshLowRelay for Case 2).

Figure 3 illustrates examples of relay UE locations w.r.t. its serving or camping cell. L1 and L2 correspond to the high RSRP threshold and the low RSRP threshold. Relays UE1, UE2, UE3, and UE4 are all within cell coverage. Relay UE1 and Relay UE2 are between L1 and L2 and thus are qualified for sidelink discovery message transmission in single-hop U2N Relay, while Relay UE3 and UE4 are not qualified.

Cell

L2

L1

Relay UE2

Relay UE1

Relay UE3

Remote UE

Cell coverage

Relay UE4

Figure 3. Relay UE locations w.r.t. its serving or camping cell

Observation 1:  In single-hop U2N Relay, a Relay UE is considered to meet the threshold conditions if the RSRP measurement of the serving or camping cell is between the high RSRP threshold and the low RSRP threshold, where the high RSRP threshold and the low RSRP threshold are equal to threshHighRelay – hystMaxRelay and threshLowRelay + hystMinRelay if the threshold conditions were previously not met and equal to threshHighRelay and threshLowRelay if the threshold conditions were previously met.

Observation 2:  Relay UE1 and Relay UE2 (which meet the threshold conditions) are allowed to transmit sidelink discovery message for U2N Relay discovery in single-hop U2N Relay, while Relay UE3 and Relay UE4 (which do not meet the threshold conditions) are not allowed to transmit sidelink discovery message.

As raised in Proposal 1, to support multi-hop UE-to-Network Relay, only the 5G ProSe UE-to-Network Relay needs to be in coverage of NG-RAN and other Intermediate UE-to-Network Relay(s) can be either in coverage or out of coverage. Thus, the conditions for a UE-to-Network Relay to transmit or forward sidelink discovery message for multi-hop UE-to-Network Relay should be re-examined.

Observation 3:  The conditions for a U2N Relay to transmit or forward sidelink discovery message for multi-hop U2N Relay should be re-examined.

2.2.1 	NR sidelink U2N Relay UE threshold conditions for Model B discovery in multi-hop U2N Relay

Figure 4 below illustrates 2 potential U2N relay paths between the Remote UE and the network with Model B discovery.

Cell

L2

L1

Relay UE2

Relay UE1

Relay UE3

Remote UE

Cell coverage

Relay UE4

Path – A

Path – B

Path – A

Path – B

Path – B

Path – A

Path – A

Path – B

Figure 4. Potential U2N relay paths with Model B discovery

These two relay paths can be established as below:

When a Model B discovery procedure is initiated by a Remote UE, the Remote UE may broadcast a U2N Relay Discovery Solicitation message. Then, Relay UE3 (i.e. a potential Relay UE with RSRP below the low RSRP threshold) may forward (information included in) the U2N Relay Discovery Solicitation message received from the Remote UE because only the 5G ProSe U2N Relay needs to be in coverage of NG-RAN. It can be expected that Relay UE1 and Relay UE2 (i.e. a potential Relay UE with the threshold conditions being met) may receive the U2N Relay Discovery Solicitation message from Relay UE3 and reply with a U2N Relay Discovery Response message. It doesn’t seem necessary for Relay UE1 or Relay UE2 to further forward the U2N Relay Discovery Solicitation message from Relay UE3 because Relay UE1 and Relay UE2 are already capable of acting as a 5G ProSe UE-to-Network Relay. Further forwarding the U2N Relay Discovery Solicitation message would derive a new relay path with increased hop count, which should be undesired in terms of traffic latency.

Besides, in response to reception of the U2N Relay Discovery Response message from Relay UE1 or Relay UE2, Relay UE3 may also transmit a U2N Relay Discovery Response message to the Remote UE. On the other hand, Relay UE4 (i.e. a potential Relay UE with RSRP above the high RSRP threshold) should not be allowed to forward a received U2N Relay Discovery Solicitation message.

In view of the above, we have the following proposals for Model B discovery to support multi-hop U2N Relay:

Proposal 2:  A UE capable of multi-hop U2N Relay UE operation shall be allowed to forward a 5G ProSe U2N Relay Discovery Solicitation message received from a Remote UE or other Intermediate U2N Relay UE if the RSRP measurement of the serving or camping cell is below the low RSRP threshold (e.g. Relay UE3 in Figure 4).

Proposal 3:  A UE capable of multi-hop U2N Relay UE operation shall reply with a 5G ProSe U2N Relay Discovery Response message when receiving a 5G ProSe U2N Relay Discovery Solicitation message from a Remote UE or other Intermediate U2N Relay UE if the NR sidelink U2N Relay UE threshold conditions are met (e.g. Relay UE1 and Relay UE2 in Figure 4).

In this situation, the UE capable of multi-hop U2N Relay UE operation does not forward the received 5G ProSe U2N Relay Discovery Solicitation message.

Proposal 4:  A UE capable of multi-hop U2N Relay UE operation shall not be allowed to forward a 5G ProSe U2N Relay Discovery Solicitation message when receiving the 5G U2N Relay Discovery Solicitation message from a Remote UE or other Intermediate U2N Relay UE if the RSRP measurement of the serving or camping cell is above the high RSRP threshold (e.g. Relay UE4 in Figure 4).

2.2.2 	NR sidelink U2N Relay UE threshold conditions for Model A discovery in multi-hop U2N Relay

Figure 5 shows an example of multi-hop U2N Relay discovery procedure with Model A. Basically, a potential 5G ProSe UE-to-Network Relay may initiate a Model A discovery procedure and broadcast a 5G ProSe UE-to-Network Relay Discovery Announcement message. When receiving the 5G ProSe UE-to-Network Relay Discovery Announcement message, a potential Intermediate UE-to-Network Relay may forward (information included in) the 5G ProSe UE-to-Network Relay Discovery Announcement message to 5G ProSe Remote UE(s) in proximity. A 5G ProSe Remote UE receiving the 5G ProSe UE-to-Network Relay Discovery Announcement message may then initiate the PC5 RRC connection establishments toward the Intermediate UE-to-Network Relay and the 5G ProSe UE-to-Network Relay.

RRC connection

Remote UE

NW

Intermediate U2N Relay

PC5 RRC connection

5G U2N Relay

U2N Relay Discovery Announcement

(Path info, RSC, hop-count)

U2N Relay Discovery Announcement

(Path info, RSC, hop-count)

PC5 RRC connection

Figure 5. An example of multi-hop U2N Relay discovery with Model A

Observation 4:  To support multi-hop U2N Relay, a potential 5G ProSe U2N Relay may initiate a Model A discovery procedure and broadcast a 5G ProSe U2N Relay Discovery Announcement message and a potential Intermediate U2N Relay may forward the 5G ProSe U2N Relay Discovery Announcement message received from the potential 5G ProSe U2N Relay.

Figure 6 below illustrates 4 potential U2N relay paths between the Remote UE and the network for Model A discovery, including:

Path-A : Remote UE – Relay UE3 – Relay UE1 – Network

Path-A’: Remote UE – Relay UE3 – Relay UE1 – Relay UE2 – Network

Path-B : Remote UE – Relay UE3 – Relay UE2 – Network

Path-B’: Remote UE – Relay UE3 – Relay UE2 – Relay UE1 – Network

Cell

L2

L1

Relay UE2

Relay UE1

Relay UE3

Remote UE

Cell coverage

Relay UE4

Path – A’

Path – B

Path – B’

Path – A’

Path – A’

Path – B’

Path – A

Path – A

Path – B’

Path – B

Path – B

Path – A

Path – B’

Path – A’

Path – A

Path – A’

Path – B

Path – B’

Figure 6. Potential U2N relay paths with Model A discovery

Path-A and Path-B can be established as below:

Similar to Release 18, Relay UE1 and Relay UE2 (i.e. a potential Relay UE with the threshold conditions being met) should be allowed to initiate a Model A discovery procedure and transmit a 5G ProSe UE-to-Network Relay Discovery Announcement message to support multi-hop UE-to-Network Relay discovery, while Relay UE4 (i.e. a potential Relay UE with RSRP above the high RSRP threshold) should not be allowed to transmit or forward a 5G ProSe UE-to-Network Relay Discovery Announcement message.

However, Relay UE3 (i.e. a potential Relay UE with RSRP below the low RSRP threshold) should be allowed to forward a 5G ProSe UE-to-Network Relay Discovery Announcement message received from other Relay UE because only the 5G ProSe U2N Relay needs to be in coverage of NG-RAN.

In view of the above, we have the following proposals for Model A discovery to support multi-hop U2N Relay:

Proposal 5:  A UE capable of multi-hop U2N Relay UE operation shall be allowed to autonomously transmit a 5G ProSe U2N Relay Discovery Announcement message if the NR sidelink U2N Relay UE threshold conditions are met (e.g. Relay UE1 and Relay UE2 in Figure 6).

Proposal 6:  A UE capable of multi-hop U2N Relay UE operation shall be allowed to forward a 5G ProSe U2N Relay Discovery Announcement message if the RSRP measurement of the serving or camping cell is below the low RSRP threshold (e.g. Relay UE3 in Figure 6).

Proposal 7:  A UE capable of multi-hop U2N Relay UE operation shall not be allowed to transmit or forward a 5G ProSe U2N Relay Discovery Announcement message if the RSRP measurement of the serving or camping cell is above the high RSRP threshold (e.g. Relay UE4 in Figure 6).

Path-A’ and Path-B’ can be established, assuming Relay UE1 and Relay UE2 are allowed to forward a 5G ProSe UE-to-Network Relay Discovery Announcement message received from other Relay UE. As a result, during the Model A discovery procedure, the Remote UE may receive, from Relay UE3, four U2N Relay Discovery Announcement messages, one message for each path, with different hop counts. However, one additional hop is induced in both Path-A’ and Path-B’ because Relay UE2/Relay UE1 forwards the U2N Relay Discovery Announcement message received from Relay UE1/Relay UE2. In this situation, it can be expected that the Remote UE would select the relay path of Path-A or Path-B because of less hop count. Therefore, in our opinion, there is no need for Relay UE1/Relay UE2 to forward the U2N Relay Discovery Announcement message received other Relay UE. This is similar to the situation of the Model-B discovery, where Relay UE1 and Relay UE2 (which are capable of acting as a 5G ProSe UE-to-Network Relay) just replies with a U2N Relay Discovery Response message without further forwarding the received U2N Relay Discovery Solicitation message.

Proposal 8:  RAN2 to discuss and determine whether a UE capable of multi-hop U2N Relay UE operation needs to forward a 5G ProSe U2N Relay Discovery Announcement message received from other Relay UE if the NR sidelink U2N Relay UE threshold conditions are met.

3 	Conclusion

In this contribution, we raise the following observations and proposals:

Observation 1:  In single-hop U2N Relay, a Relay UE is considered to meet the threshold conditions if the RSRP measurement of the serving or camping cell is between the high RSRP threshold and the low RSRP threshold, where the high RSRP threshold and the low RSRP threshold are equal to threshHighRelay – hystMaxRelay and threshLowRelay + hystMinRelay if the threshold conditions were previously not met and equal to threshHighRelay and threshLowRelay if the threshold conditions were previously met.

Observation 2:  Relay UE1 and Relay UE2 (which meet the threshold conditions) are allowed to transmit sidelink discovery message for U2N Relay discovery in R18, while Relay UE3 and Relay UE4 (which do not meet the threshold conditions) are not allowed to transmit sidelink discovery message.

Observation 3:  The conditions for a U2N Relay to transmit or forward sidelink discovery message for multi-hop U2N Relay should be re-examined.

Observation 4:  To support multi-hop U2N Relay, a potential 5G ProSe U2N Relay may initiate a Model A discovery procedure and broadcast a 5G ProSe U2N Relay Discovery Announcement message and a potential Intermediate U2N Relay may forward the 5G ProSe U2N Relay Discovery Announcement message received from the potential 5G ProSe U2N Relay.

Proposal 1:  To support multi-hop U2N Relay, only the 5G ProSe U2N Relay needs to be in coverage of NG-RAN and other Intermediate U2N Relay(s) can be either in coverage or out of coverage.

Proposal 2:  A UE capable of multi-hop U2N Relay UE operation shall be allowed to forward a 5G ProSe U2N Relay Discovery Solicitation message received from a Remote UE or other U2N Relay UE if the RSRP measurement of the serving or camping cell is below the low RSRP threshold (e.g. Relay UE3 in Figure 4).

Proposal 3:  A UE capable of multi-hop U2N Relay UE operation shall reply with a 5G ProSe U2N Relay Discovery Response message when receiving a 5G ProSe U2N Relay Discovery Solicitation message from a Remote UE or other U2N Relay UE if the NR sidelink U2N Relay UE threshold conditions are met (e.g. Relay UE1 and Relay UE2 in Figure 4).

In this situation, the UE capable of multi-hop U2N Relay UE operation does not forward the received 5G ProSe U2N Relay Discovery Solicitation message.

Proposal 4:  A UE capable of multi-hop U2N Relay UE operation shall not be allowed to forward a 5G  ProSe U2N Relay Discovery Solicitation message when receiving the 5G U2N Relay Discovery Solicitation message from a Remote UE or other U2N Relay UE if the RSRP measurement of the serving or camping cell is above the high RSRP threshold (e.g. Relay UE4 in Figure 4).

Proposal 5:  A UE capable of multi-hop U2N Relay UE operation shall be allowed to autonomously transmit a 5G ProSe U2N Relay Discovery Announcement message if the NR sidelink U2N Relay UE threshold conditions are met (e.g. Relay UE1 and Relay UE2 in Figure 6).

Proposal 6:  A UE capable of multi-hop U2N Relay UE operation shall be allowed to forward a 5G ProSe U2N Relay Discovery Announcement message if the RSRP measurement of the serving or camping cell is below the low RSRP threshold (e.g. Relay UE3 in Figure 6).

Proposal 7:  A UE capable of multi-hop U2N Relay UE operation shall not be allowed to transmit or forward a 5G ProSe U2N Relay Discovery Announcement message if the RSRP measurement of the serving or camping cell is above the high RSRP threshold (e.g. Relay UE4 in Figure 6).

Proposal 8:  RAN2 to discuss and determine whether a UE capable of multi-hop U2N Relay UE operation needs to forward a 5G ProSe U2N Relay Discovery Announcement message received from other Relay UE if the NR sidelink U2N Relay UE threshold conditions are met.

- References

[1] RP-241609, New WID on NR sidelink multi-hop relay

[2] TR 23.700-03-100, Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 2 (Release 19)

[3] TS 38.331-i20, NR; RRC protocol specification (Release 18)