3GPP TSG-RAN WG2 Meeting #127	R2-2406529

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.13.3

Source:	ASUSTeK

Title:	Discussions on the L2 Intermediate U2N Relay in multi-hop L2 U2N Relay

Document for:	Discussion and Decision

1	Introduction

According to the objective of the new WID on NR sidelink multi-hop relay [1], control plane procedures should be specified to support multi-hop L2 U2N Relay.The following figure (quoted from clause 6.1.1 in [2]) illustrates an example architecture of multi-hop UE-to-Network Relay, where a 5G ProSe Remote UE connects with the NG-RAN via an Intermediate UE-to-Network Relay and a 5G ProSe UE-to-Network Relay.

Figure 1: Example architecture of multi-hop UE-to-Network Relay

In this contribution, we discuss the protocol stack of an L2 Intermediate U2N Relay and sidelink RRC messages forwarding between the L2 U2N Remote UE and the L2 U2N Relay for multi-hop L2 U2N Relay via the L2 Intermediate U2N Relay.

2	Discussion

2.1	Protocol Architecture for L2 UE-to-Network Relay

The following 2 figures of protocol stacks for single-hop L2 U2N Relay are quoted from [3]:

Figure 2: User plane protocol stack for L2 UE-to-Network Relay

Figure 3: Control plane protocol stack for L2 UE-to-Network Relay

Intermediate U2N Relay

To support multi-hop L2 U2N Relay, it is clear that a PC5-SRAP sublayer is needed in an L2 Intermediate UE-to-Network Relay, as shown below, so as to forward UL/DL data and signaling between the L2 Remote UE and the L2 U2N Relay UE:

Figure 4: Protocol stack in L2 Intermediate UE-to-Network Relay

For uplink traffic, unlike there may be multiple target Remote UEs in L2 U2U Relay, the gNB is the sole destination of SRAP PDUs, which is known to the L2 Intermediate UE-to-Network Relay. Thus, when receiving an SRAP PDU from an L2 U2N Remote UE or other relay UE, the L2 Intermediate UE-to-Network Relay should be able to map the end-to-end Uu Radio bearer of the L2 U2N Remote UE to an egress PC5 Relay RLC channel according to the UE ID of the L2 U2N Remote UE and BEARER ID of the end-to-end Uu Radio bearer in the SRAP header. Therefore, in our opinion no extra information needs to be added in the SRAP header for supporting multi-hop L2 U2N Relay. Similarly, for downlink traffic, no extra information is needed in the SRAP header, either. In other words, the legacy L2 U2N SRAP PDU header can be reused for multi-hop L2 U2N Relay.

Proposal 1:  A PC5-SRAP sublayer is included in an L2 Intermediate U2N Relay to support multi-hop L2 U2N Relay. And, the legacy SRAP header can be reused for multi-hop L2 U2N Relay.

2.2	Sidelink RRC messages exchanged between L2 Remote UE and L2 U2N Relay UE

The following 3 sidelink RRC procedures are supported in single-hop L2 U2N Relay [4]:

#### 5.8.9.8 Remote UE information ##### 5.8.9.8.1 General Figure 5.8.9.8.1-1: Remote UE information This procedure is used by the L2 U2N Remote UE in RRC_IDLE/RRC_INACTIVE to inform about the required SIB(s) /posSIB(s), provide Paging related information to the connected L2 U2N Relay UE, request the SFN-DFN offset from the connected L2 U2N Relay UE, and trigger L2 U2N Relay UE in RRC_IDLE/RRC_INACTIVE to enter RRC_CONNECTED during indirect path addition/change in MP operation. This procedure is also used by the L2 U2U Remote UE to send end-to-end PC5 connection release/failure related information to L2 U2U Relay UE. … #### 5.8.9.9 Uu message transfer in sidelink ##### 5.8.9.9.1 General Figure 5.8.9.9.1-1: Uu message transfer in sidelink The purpose of this procedure is to transfer Paging message and System Information from the L2 U2N Relay UE to the L2 U2N Remote UE in RRC_IDLE/RRC_INACTIVE. #### … #### 5.8.9.10 Notification Message ##### 5.8.9.10.1 General Figure 5.8.9.8.1-1: Notification message in sidelink This procedure is used by a U2N Relay UE to send notification to the connected U2N Remote UE, or used by a L2 U2U Relay UE to send notification to the L2 U2U Remote UE for an end-to-end PC5 connection when condition(s) as specified in 5.8.9.10.2 is met for the hop between the L2 U2U Relay UE and the peer L2 U2U Remote UE. …

As described above, the L2 U2N Remote UE may transmit the RemoteUEInformationSidelink message to the connected L2 U2N Relay UE to inform about the required SIB(s) and provide the Paging related information. And, the L2 U2N Relay UE may transfer System Information or Paging Record to the L2 U2N Remote UE in a UuMessageTransferSidelink message. Besides, a U2N Relay UE may send the NotificationMessageSidelink message to the connected U2N Remote UE to indicate Uu RLF, handover, cell reselection, or Uu RRC failure. These 3 sidelink RRC messages are all transmitted on SL-SRB3 established between the L2 U2N Remote UE and the L2 U2N Relay UE.

Observation 1:  In single-hop L2 U2N Relay, the L2 U2N Remote UE may send the RemoteUEInformationSidelink message to the L2 U2N Relay UE and receive the UuMessageTransferSidelink message and the NotificationMessageSidelink message from the L2 U2N Relay UE.

In our opinion, these 3 sidelink RRC messages should also be applicable to multi-hop L2 U2N Relay.

Proposal 2:  The RemoteUEInformationSidelink message, the UuMessageTransferSidelink message, and the NotificationMessageSidelink message are applicable to multi-hop L2 U2N Relay.

In case of multi-hop L2 U2N Relay, sidelink RRC messages need to be exchanged between the L2 U2N Remote UE and the L2 U2N Relay via the Intermediate U2N Relay(s). Thus, a mechanism should be specified for the Intermediate U2N Relay(s) to forward sidelink RRC messages between the L2 U2N Remote UE and the L2 U2N Relay.

Therefore, we propose the following:

Proposal 3: RAN2 to specify a mechanism for the Intermediate U2N Relay(s) to forward sidelink RRC messages between the L2 U2N Remote UE and the L2 U2N Relay. The mechanism details are FFS.

3 	Conclusion

In this contribution, we raise the following observation and proposals:

Observation 1:  In single-hop L2 U2N Relay, the L2 U2N Remote UE may send the RemoteUEInformationSidelink message to the L2 U2N Relay UE and receive the UuMessageTransferSidelink message and the NotificationMessageSidelink message from the L2 U2N Relay UE.

Proposal 1:  A PC5-SRAP sublayer is included in an L2 Intermediate U2N Relay to support multi-hop L2 U2N Relay. And, the legacy SRAP header can be reused for multi-hop L2 U2N Relay.

Proposal 2:  The RemoteUEInformationSidelink message, the UuMessageTransferSidelink message, and the NotificationMessageSidelink message are applicable to multi-hop L2 U2N Relay.

Proposal 3: RAN2 to specify a mechanism for the Intermediate U2N Relay(s) to forward sidelink RRC messages between the L2 U2N Remote UE and the L2 U2N Relay. The mechanism details are FFS.

- References

[1] RP-241609, New WID on NR sidelink multi-hop relay

[2] TR 23.700-03-100, Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 2 (Release 19)

[3] TS 38.300-i20, NR; NR and NG-RAN Overall Description; Stage 2 (Release 18)

[4] TS 38.331-i20, NR; RRC protocol specification (Release 18)