3GPP TSG-RAN WG2 Meeting #127	R2-2406532

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.6.2

Source: 	OPPO

Title:  	Discussion on open issues for inter-CU LTM

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to provide our considerations on open issues for inter-CU LTM.

Discussion

LTM preparation

During last RAN2 meeting, the number of reference configuration was discussed but not no consensus was made. There are two alternatives on the table for the provision of reference configuration:

- Alt1: Single reference configuration as R18 LTM.

- Alt2: Per CU reference configuration.

For per CU reference configuration, it yields benefits only under certain assumptions including: 1) There is a significant variation in the candidate cell configurations under each CU, relying on a single reference configuration may lead to a lot of full configurations. 2) At least two candidate cell configurations are provided by a CU, i.e., the more distributed the candidate cells are, the less benefits from per CU reference configuration becomes.

In other scenarios, the advantage of providing per CU reference configuration may not be clear and it is quite likely that one reference configuration shared among multiple CUs might be enough. In addition, single reference configuration has already been applied for R18 inter-SN SCPAC which also involves multiple CUs. Since it is hard to evaluate which alternative is better for different cases, it is preferred to follow R18 LTM design for simplicity, i.e., the UE is configured with a single configuration for each LTM-Config.

Single reference configuration is supported for inter-CU LTM.

Another issue concerning the LTM configuration is how to provide the LTM-CSI-ResourcConfig used for candidate cell monitoring and the solutions for further discussion include the follows:

- Alt1: One common LTM-CSI-ResourcConfig managed by the source CU.

- Alt2: Per CU LTM-CSI-ResourcConfig.

As agreed in last RAN2 meeting, the source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement. Furthermore, it was also agreed that the LTM candidate ID is unique across all the participating gNB-CUs, which also facilitates the reuse of R18 LTM solution on L1 measurement configuration. Therefore, it is confirmed that Alt1 is the desirable approach for LTM-CSI-ResourcConfig provision.

Stage-2 signaling flows and procedures 5. The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

It is confirmed that a common LTM-CSI-ResourceConfig managed by the source CU is configured for inter-CU LTM.

Inter-MN LTM with DC

As specified in WID, inter-MN LTM with SN unchanged or released is supported.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

For inter-MN LTM with SN unchanged, it is unclear whether the SCG configuration can be included in the candidate cell configuration. In legacy inter-MN HO without SN change or CHO with SCG, the RRC reconfiguration message can contains an MCG configuration and possibly an SCG configuration. It means that the handling of SCG during MCG change may include the following three cases:

- Case1: The SCG configuration is unchanged, i.e., the SCG configuration is not contained in the target cell configuration.

- Case2: The SCG configuration is changed and the cell components of SCG keep the same, i.e., the SCG configuration contained in the target cell configuration is applied for addition/modification/release of SCG bearer and SCG RLC bearer of split bearer(s) as well as configuration changes for SN terminated MCG bearers.

- Case3: The SCG configuration is changed and the cell components are also changed, i.e., the SCG configuration contained in the target cell is applied for change of PSCell or addition/modification/release of SCG SCells.

For inter-MN LTM with SN unchanged, the three cases for SCG handling can be also supported with the SCG configuration included/not included in the candidate cell configuration.

For SCG handling during inter-MN LTM with SN unchanged, the following cases are supported:

  - The SCG configuration is unchanged.

  - The SCG configuration is changed and the cell components of SCG keep the same, e.g., addition/modification/release of SCG bearer and SCG RLC bearer of split bearer(s) as well as configuration changes for SN terminated MCG bearers can be performed.

  - The SCG configuration is changed and the cell components are also changed, e.g., change of PSCell and addition/modification/release of SCG SCells can be performed.

For inter-MN LTM with SN unchanged, the candidate cell configuration includes an MCG configuration and possibly an SCG configuration.

In addition, it is specified that the SN can be unchanged or released during inter-CU MN LTM, there might be two understandings for the SN handling in this scenario:

- Understanding1: The same SN handling is applied for all inter-CU MN LTM procedures, i.e., always unchanged or released.

- Understanding2: A mixture of different SN handlings is supported, i.e., for some candidate MNs, the SN needs to be kept, for other candidate MNs, the SN needs to be released.

Given that different understandings may lead to different UE behaviours and signalling design, further clarification on the scenario is required.

For inter- MN LTM with DC configured, RAN2 confirms how to support the SN handling from the following alternatives:

  - The same SN handling is configured for inter-MN candidate cells, e.g., always unchanged/released.

  - Different SN handlings can be configured for inter-MN candidate cells, e.g., either unchanged or released.

Inter-CU SN LTM

For inter-CU SN LTM, it is quite similar to inter-SN subsequent CPAC procedure specified in R18. For inter-SN SCPAC, it can be initiated either by MN or SN and the configuration can only be provided by MN via SRB1 due to the involvement of MN. The same principle can be simply followed for inter-CU SN LTM since the two procedures are essentially the same.

The configuration of inter-CU SN LTM can be initiated by MN or SN.

The inter-CU LTM can only be configured via SRB1, i.e., in MN format.

Security key update of SN is required during inter-CN SCPAC execution. In order to support subsequent security key update, a list of sk-counter is configured for each SN. The UE selects the first unused sk-counter from the list associated with the securityCellSetID of the target cell to derive a new S-KgNB when performing inter-SN SCPAC. This security update mechanism can be also reused for inter-CU SN LTM.

For inter-CU LTM when CU is acting as SN, the security update mechanism for SCPAC is followed.

Coexistence issue

For various types of LTM configurations, further discussion is needed on the co-existence issue. Our views on the issue is provided as follows.

|  | Inter-MN LTM | Intra-MN LTM |
| --- | --- | --- |
| Inter-SN LTM | - Not supported as per WID | - The configurations can be stored by the UE at the same, i.e., co-existence is supported. But the LTM-config configured in one node might be impacted by the LTM execution at the other node as analysed below. |
| Intra-SN LTM | - The configurations can be stored by the UE at the same, i.e., co-existence is supported. But the LTM-config configured in one node might be impacted by the LTM execution at the other node as analysed below. | - Follow R18 principle |

For intra-MN LTM and inter-SN LTM

- The execution of intra-CU MCG LTM may lead to SCG release, in this case, the inter-CU SCG LTM configuration needs to be released.

- The execution of inter-SN LTM may lead to the update of MCG configuration, in this case, the intra-CU MCG LTM configuration might be invalid. For example, the bearer type change is performed during inter-CU SCG LTM, e.g., from MCG bearer to SCG bearer, but the candidate cells for intra-CU MCG LTM are prepared based on the context that it is an MCG bearer. The bearer type change is not expected during intra-CU SCG LTM, and if intra-CU SCG LTM is executed after inter-SN LTM, it will lead to error bearer type change. To handle the issue, it is preferred to rely on the NW to guarantee the validity of LTM configuration, e.g., the LTM configuration can be released or updated by explicit NW signalling.

For inter-MN LTM and intra-SN LTM

- The execution of inter-CU MCG LTM may lead to SCG release, in this case, the inter-CU SCG LTM configuration needs to be released. In addition, if SCG configuration is included in inter-MN LTM candidate cell configuration, the intra-CU SCG LTM might be invalid due to the update of SCG configuration as discussed above.

Based on the analysis above, we have the following proposals:

The following co-existence cases are supported:

  - Inter-MN LTM and intra-SN LTM.

  - Intra-MN LTM and inter-SN LTM.

If the SCG is released at MCG LTM execution, the SCG LTM configuration is released.

- Obervation1  The intra-CU LTM configuration configured in one node (i.e., MN or SN) might be invalid due to the execution of inter-CU LTM at the other node (i.e., SN or MN).

It is up to the NW to guarantee the validity of LTM configuration when the source cell configuration is updated.

Conclusion

Based on the discussion in section2, we have following proposals:

Proposal 1	Single reference configuration is supported for inter-CU LTM.

Proposal 2	It is confirmed that a common LTM-CSI-ResourceConfig managed by the source CU is configured for inter-CU LTM.

Proposal 3	For SCG handling during inter-MN LTM with SN unchanged, the following cases are supported:

a.	The SCG configuration is unchanged.

b.	The SCG configuration is changed and the cell components of SCG keep the same, e.g., addition/modification/release of SCG bearer and SCG RLC bearer of split bearer(s) as well as configuration changes for SN terminated MCG bearers can be performed.

c.	The SCG configuration is changed and the cell components are also changed, e.g., change of PSCell and addition/modification/release of SCG SCells can be performed.

Proposal 4	For inter-MN LTM with SN unchanged, the candidate cell configuration includes an MCG configuration and possibly an SCG configuration.

Proposal 5	For inter- MN LTM with DC configured, RAN2 confirms how to support the SN handling from the following alternatives:

a.	The same SN handling is configured for inter-MN candidate cells, e.g., always unchanged/released.

b.	Different SN handlings can be configured for inter-MN candidate cells, e.g., either unchanged or released.

Proposal 6	The configuration of inter-CU SN LTM can be initiated by MN or SN.

Proposal 7	The inter-CU LTM can only be configured via SRB1, i.e., in MN format.

Proposal 8	For inter-CU LTM when CU is acting as SN, the security update mechanism for SCPAC is followed.

Proposal 9	The following co-existence cases are supported:

a.	Inter-MN LTM and intra-SN LTM.

b.	Intra-MN LTM and inter-SN LTM.

Proposal 10	If the SCG is released at MCG LTM execution, the SCG LTM configuration is released.

Obervation1  The intra-CU LTM configuration configured in one node (i.e., MN or SN) might be invalid due to the execution of inter-CU LTM at the other node (i.e., SN or MN).

Proposal 11	It is up to the NW to guarantee the validity of LTM configuration when the source cell configuration is updated.