3GPP TSG-RAN WG2 Meeting #127	                                    R2-2406533

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.6.3

Source: 	OPPO

Title:  	Discussion on event-triggered L1 measurement reporting

Document for:	Discussion and Decision

Introduction

In the RAN#102 meeting, a new WID on further enhancement of NR mobility was agreed, and one of the objectives is to study the measurement related enhancements including event-triggered L1 measurement reporting:

Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

In this contribution, we discuss the open issues on event-triggered L1 measurement reporting, including the event design and configuration.

Discussion

In previous meetings, event-based L1 measurement reporting has been studied and the following agreements were reached:

Agreements on measurement enhancements for LTM(RAN2#126):

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

Agreements on measurements (RAN2#125BIS):

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

L1 event design

For event-triggered L1 measurement reporting, similar principle as in event-triggered L3 measurement reporting can be followed. That is, once the LTM candidate cell configuration is provided, UE keeps monitoring the configured candidate cells, and the measurement result is reported when the criteria of an LTM measurement event are met.

And regarding the granularity of the L1 event, there are two alternatives:

- Beam level event

Rel-18 LTM L1 measurement reporting is a beam level measurement reporting which relies on legacy CSI reporting framework. For the L1 event design, as the event is designed for L1 measurement reporting, a beam level event framework is better aligned with current L1 measurement reporting granularity.

- Cell level event

Existing L3 measurement event mainly focuses on cell level measurement evaluation since the reported measurement results are used to assist NW to make the decision for handover. L1 event-triggered measurement reporting is also aiming to assist source cell to trigger a LTM cell switch. A cell level event can also work for triggering the L1 reporting. While extra latency may be caused when consolidating a cell level result, which may delay the reporting.

It is agreed to use a beam level measurement result for event evaluation as a baseline since LTM relies on the beam level measurement reporting. And it is still open on whether the cell level measurement can be used for event evaluation.

Currently, L3 measurement event relies on cell level measurement, we see no need to introduce a redundant L1 cell level event. Thus, we understand beam level event for L1 measurement reporting is sufficient for R19 measurement enhancement.

RAN2 confirm the L1 event for L1 measurement reporting only supports beam level event.

L1 event evaluation

To design the L1 event, L3 measurement event was considered as a baseline, e.g. the beam condition for both serving cell and candidate cell are considered for the event design. And the following events were agreed in last meeting

Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

And there are still some open issues regarding the event evaluation:

- Issue 1: FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation.

The first issue is what beam(s) can be used for event evaluation for both serving and candidate cells. In R19 MIMO WID, similar discussed has initiated and the following consensus were reached.

For LTM event evaluation, we understand one way is to follow the scheme-1 or scheme-2 to determine for serving beam, i.e. RS for serving beam is the QCL RS in the indicated TCI state or the SSB which is QCLed with the QCL RS in the indicated TCI state. Another option is to rely the best beam to determine the serving beam quality. In most cases, the indicated TCI state refer to the best beam of the cell, while there may be the case that beam failure is detected in serving cell, i.e. the serving beam is not the best beam. There is no necessity to perform a cell switch to recovery the beam failure. Thus, it is not appropriate to rely entirely on the beam QCL to indicated beam to determine an event related to cell switch scenario.

For both serving cell and candidate cell, the best beams are used for L1 event evaluation.

- Issue 2: FFS on the need of Event LTM1.

On the need of event LTM1, our understanding is that event LTM1 benefits for measurement overhead reduction while it is out of the scope of R19 mobility WID. At this stage, we can focus on the event LTM2-5 design, and come back to event LTM1 if time allows.

A1 event is not considered for LTM L1 event.

Event configuration

In last meeting, RAN2 have an initial discussion on the measurement event configuration, and the following agreement were reached:

Agreements on measurements:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

In R16/R17 CHO/CPAC, the execution conditions of each candidate cell are all configured by source serving cell, since source serving node have a good knowledge of the candidate cell measurement. And till R18 SCPAC, as subsequent execution procedure is supported, we allow the candidate SNs to provide the subsequent execution conditions as it would be easier to let the candidate SN generate the execution conditions for the candidate PSCells selected by the candidate SN, based on the candidate SCG MeasConfig.

For inter-CU LTM, we think similar principle should be followed to allow each candidate CU to generate the L1 event configuration for each candidate cell to support subsequent LTM L1 reporting. The reason is that after the serving cell change, the event configurations for event-triggered L1 reporting evaluation should use different threshold/parameters for the events given that the radio propagations between the new serving cell and its neighbours are different from those corresponding to previous source serving cell. And the L1 event configuration should be contained in the candidate cell configuration accordingly since UE will only use it when the candidate cell becomes serving cell.

L1 event configuration is configured by each serving/candidate cell.

L1 event configuration is included in LTM candidate cell configuration, i.e. in CSI report configuration inside the RRC container of each candidate cell.

Furthermore, whether the L1 event are configured for all candidate cells:

- Alternative 1: common L1 event configuration

one alternative is to use a common L1 event configuration, for example, when evaluate the beam level event, the RS may include the beams of all the LTM candidate cells. As shown below, the L1 event configuration is provided by each candidate cell which will be applied upon UE executed to that cell. And the L1 event configuration is same for all candidate cells.

- Alternative 2: per candidate cell L1 event configuration

Per candidate cell L1 event configuration gives more flexibility to support unidirectional subsequent LTM procedure, which also allow NW to release/modify the LTM candidate cell without reconfigure the CSI resource set associated to event triggered CSI report configuration.

RAN2 further discuss the granularity of L1 event configuration, i.e., common or per candidate cell.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	RAN2 confirm the L1 event for L1 measurement reporting only supports beam level event.

Proposal 2	For both serving cell and candidate cell, the best beams are used for L1 event evaluation.

Proposal 3	A1 event is not considered for LTM L1 event.

Proposal 4	L1 event configuration is configured by each serving/candidate cell.

Proposal 5	L1 event configuration is included in LTM candidate cell configuration, i.e. in CSI report configuration inside the RRC container of each candidate cell.

Proposal 6	RAN2 further discuss the granularity of L1 event configuration, i.e., common or per candidate cell.

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx