3GPP TSG-RAN WG2 Meeting #127	                                    R2-2406534

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.6.4

Source: 	OPPO

Title:  	Discussion on event-triggered L1 measurement reporting

Document for:	Discussion and Decision

Introduction

In the RAN#102 meeting, a new WID on further enhancement of NR mobility was agreed, and one of the objectives is to study the measurement related enhancements including event-triggered L1 measurement reporting:

Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

In this contribution, we discuss the UL signalling used to carry the measurement results for L1 event triggered reporting.

Discussion

To reduce the CSI reporting overhead due to LTM candidate cell evaluation, it is agreed to study the event-based L1 measurement reporting in Rel-19. And it is questionable that which signalling/format is used for the event triggered reporting.

In R19 MIMO WID, similar discussion for UE initiated beam reporting has been triggered in last meeting and the following working assumption was agreed:

Working Assumption

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

- FFS: details on the dedicated RRC signaling

- Above applies at least for the single CC case.

Both mode-A and mode-B rely on two-step UCI based reporting. UE sends one-bit indication in PUCCH to request the reosurce for UCI or notify a UCI-based reporting.

For Rel-19 mobility, one way to transmit the event triggered report is to follow MIMO-like way, i.e. reuse existing CSI reporting for event triggered L1 measurement reporting. The gNB provides preconfigured physical layer resource, for example PUCCH resources and the UE can transmit the UCI in the preconfigured PUCCH resource.

- UCI based L1 measurement reporting has been introduced since R15, UE reports the beam measurement result through PUCCH or PUSCH. Reusing the UCI based L1 measurement reporting can bring gains on latency, which is very important for providing a real-time measurement.

- While UCI requires pre-configured PUCCH resources and NW should be aware of the UCI format contained in PUSCH/PUCCH, which requires NW to pre-configure additional dedicated resources for the coming UCI indication. With such 2-step UCI, the benefit of latency becomes less obvious. Furthermore, the format of UCI is fixed, that is, limited measurement reuslt can be deliveried to NW with the measurement report carirerd in UCI.

Alternatively, UE can report the measurement result through a MAC CE message. When the event is triggered, UE generates the MAC CE for L1 measurement reporting.

- For MAC CE based L1 measurement reporting, the most significant benefit is the flexible size of MAC CE. By comparing with UCI, larger amount of measurements can be reported to NW within one MAC CE, which can provide an overall cognition of the LTM candidate cells/beams.

- Another advantage of MAC CE based L1 measurement reporting is the less spec impact on UL grant acquisition. As we know, for event triggered L1 reporting, NW may not be able to predict the exactly timing of the triggered event, that is, NW may not be able to provide a precise UL grant for such report. Legacy MAC spec already supports UE to request a UL grant for the MAC CE transmission dynamically, i.e. trigger SR in case there are no available UL resources. And MAC CE based L1 measurement report can be transmitted with any available UL resources (i.e. CG, DG or UL grant indicated in RAR) without limitation.

Based on the discussion above, we think MAC CE is the appropriate format for L1 event triggered reporting.

MAC CE is used as UL signalling that conveys the event triggered L1 measurement report.

Regarding to the content of the reporting, R18 candidate cell L1 measurement report includes the L1-RSRP and SSBRI for the configured SSBs. Similar content can be also applied for event triggered L1 measurement reporting and as this is RAN1 scope, we could leave it to RAN1 to decide.

It is up to RAN1 to determine the content for measurement results of the event triggered L1 measurement report.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	MAC CE is used as UL signalling that conveys the event triggered L1 measurement report.

Proposal 2	It is up to RAN1 to determine the content for measurement results of the event triggered L1 measurement report.

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx