3GPP TSG-RAN WG2 Meeting #127                                           	R2-2406536

Maastricht, 19.-23.08.2024

Source:	Telit Communications S.p.A., Novamint , Sateliot , Thales

Title:	Further considerations on S&F operation from device perspective

Agenda Item:	8.9.2

Document for:	Discussion and Decision

# Introduction

The present document is a revision of R2-2404979. It was updated to reflect the actual discussion progress within RAN2 and also parts from SA2 discussions where relevant are additionally reflected and providing some more justification examples for the various proposals. However, the proposals itself have been kept according to their core part.

In the last RAN2 meeting following agreements were made related to S&F Satellite operation.

Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

Even though in many areas RAN2 needs to wait until discussion in other groups has further progressed, some important decisions were already made especially by SA2, which are reflected hereafter:

Extract of SA2 [2] conclusions on KI#2 (Store and Forward):

1)	When feeder link is not available and the network supports S&F operation, the network shall be able to inform UE(s) whether S&F Satellite operation is applied, (e.g. eNB broadcast support of S&F operation as part of System Information).

NOTE 1:	The trigger for the eNB to broadcast support of S&F operation is based on the decision of RAN WGs. From system perspective the expectation is that if the network does not support S&F operation and the feeder link is not available then eNB switches off and does not broadcast any signal.

……

7) The MME may indicate to UE the estimated delivery time in NAS messages (Attach accept or TAU accept message or service accept). How UE uses this information is left for UE implementation.

# Discussion

S&F will be an important feature especially at the beginning for the deployments with lower number of satellites and a limited number of ground stations but also in general for delay tolerant traffic. As satellite deployment evolves or also depending on satellite position (whether feeder-link is in reach) a satellite will likely support both transfer methods, namely, S&F and real time (RT).

It is important for the UE to know and especially the application on the UE whether S&F is supported as sole transfer method or alongside with normal RT traffic.

The knowledge on S&F is important also for applications on the device, e.g. whether to use TCP/IP or UDP protocol for the transfer.

Proposal 1: UE should receive an indication whether S&F satellite operation is activated.

Proposal 2: UE should receive an indication whether real-time is momentarily supported by the satellite.

It needs to be discussed whether this could also be done with the existing cellBarred and cellBarredNTN. (please note that the Satellite community do not believe we should use cellBarred or cellBarredNTN for this purpose)

S&F is especially suited for delay-tolerant traffic, however additional indication on expected delivery delay would be important for the device.

Delivery delay is at least the time it requires to reach the next feeder link but may even be larger if not all data can be offloaded during visibility of the feeder link from the satellite. Hence an upper boundary indication of S&F expected delivery time is important in general for the device.

If both transfer methods would be supported by a satellite, the UE could also use the expected delivery time to evaluate whether to use S&F transfer or RT, depending also on the service needs.

Proposal 3: For S&F Satellite operation expected maximum delivery time should be indicated to the UE.

In case both methods are available the UE should be able to decide whether S&F Satellite operation is sufficient for the provided data.

Proposal 4: In case both transfer methods are available, UE should be able to decide/ indicate whether data transfer should be performed by S&F or RT.

Finally it is still up to the satellite network e.g. in case that S&F was indicated as sufficient to perform an earlier delivery than indicated by e.g. using RT traffic or forwarding via ISL. Network decision may depend on remaining storage capabilities but no need to specify as it is up to network when indicated maximum delivery time is still met.

As depending on satellite position RT traffic or depending on exceeding capabilities also unavailability of the respective methods should be possible to indicate.

Proposal 5: Satellite should be able to indicate (broadcast) the duration during which  S&F or RT operation are respectively available or unavailable.

# Conclusion

Proposal 1: UE should receive an indication whether S&F is supported by the satellite.

Proposal 2: UE should receive an indication whether RT is momentarily supported by the satellite.

Proposal 3: For S&F Satellite operation expected maximum delivery time should be indicated to the UE.

Proposal 4: In case both transfer methods are available, UE should be able to decide/ indicate whether data transfer should be performed by S&F or RT.

Proposal 5: Satellite should be able to indicate (broadcast) the duration during which S&F or RT operation are respectively available or unavailable.

# Reference

[1] R2-2403733 Report from Break-Out Session on NR NTN and IoT NTN

[2] TR 23.700-29 v1.0.0 [FS_5GSAT_Ph3_Arch]