3GPP TSG-RAN WG2 Meeting #127          	          R2-2406538

Maastricht, Netherlands, August 19th –23rd, 2024

Agenda Item:	8.1.3

Source: 	Fujitsu

Title: 	Discussion on NW side Data Collection for Beam Management

WID/SID:	NR_AIML_air

Document for:	Discussion and Decision

Introduction

In last RAN2#126 meeting [1], there were discussions on the NW side data collection of beam management use cases and some agreements were achieved below:

Agreements

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting.

In this contribution, we would like to discuss the detailed solutions for NW side data collection of beam management use cases for model training.

Discussion

Logging Data Collection and Reporting

According to the agreements list above, one remaining issue for NW-side BM data collection is the details on logging and reporting data collected by multiple time instance. The key aspects need to be further discussed are listed here: 1) The details of L1 measurement result or other data collection; 2) The configuration on multiple time instance data logging; 3) The details of logged data reporting.

As for the first aspect, since L1 measurement result and other data related parameters for data collection are determined by RAN1, therefore, it is suggested that RAN2 wait for RAN1 further input at this stage.

Proposal 1 For the detailed parameters of L1 data collection, RAN2 waits for the RAN1 inputs.

As for the logging of data from multiple time instances, we believe both periodic and event-based schemes are possible. For beam management use cases, periodic data logging is probably sufficient for most cases if the scenario is kept static because the number of beams for Set B has been proven to be small in Rel-18, the periodicity can be configured with a wide range to support different Set A/B association. On the other hand, event-triggered scheme should also be supported in case that there is instant change of the scenario, and the periodicity cannot cover the change.

Proposal 2 For data collection of beam management, the type for data logging can be periodical and event triggered.

And for the reporting of logged data, events can be defined to trigger the reporting from both NW and UE perspective, e.g., from NW’s point of view, to avoid data overflow or discarding, the data must be reported if the maximum number of recording entry has been reached; from UE’s point of view, the logged data must be reported if the allocated local storage has reached the limit.

Proposal 3 The event-based reporting is supported for logging scheme.

Proposal 4 The event triggering for logging scheme can be:

- The configured maximum entries for data collection are achieved.

- The storage for data collection at UE side is above/below the threshold.

In terms of the potential issue that single RRC message is not sufficient for logged data reporting, since the reporting can be configured by NW, therefore, NW can tailor its configuration to avoid such issue by its own judgement, e.g., NW can raise the threshold for data filtering or the maximum bits to be reported.

Proposal 5 The NW can raise the threshold for data filtering or configure the maximum bits for the data collection to avoid the case when single RRC message is not sufficient.

Assistance Information

To assist the network to categorize the collected data, it is important for the network to be aware of the related assistance information (e.g., additional conditions) when the data is generated and stored. There are UE-side and NW-side additional conditions. To better differentiate the characteristics of data, both additional conditions should be identified.

Observation 1 To categorize the data, it is necessary to consider both UE-side and NW-side assistance information.

In RAN1#116bis meeting, one solution was agreed for UE-side model training with the transmission of NW-sided additional conditions implicitly in term of NW associated ID(s) from network to UE. The AI/ML models will be developed at UE side based on the collected data corresponding to the NW-sided additional conditions/associated ID(s), or both UE-side and NW-side additional conditions/associated ID(s). The details can refer to the following agreements.

Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

Similar principle can be applied to NW-side data collection for model training. With the UE-side and NW-side assistance information (e.g., additional conditions/associated ID(s)), the network will develop the AI model specific for these additional conditions/associated ID(s). One possible solution is that the network can send the NW-side assistance information to the UE. When the UE stores the collected data, it also includes the related NW-side assistance information. Additionally, the UE can also provide the corresponding UE-side assistance information with the collected data. The UE reports the collected data, NW-side assistance information and NW-side assistance information as the integrated data statistics.

Figure 1 UE- based solution

In option 2, the UE can only report UE-side assistance information together with the reported data. When receiving the data from the UE, the network can complement the corresponding NW-side assistance information.

Figure 2 UE- and NW-based solution

In option 3, it is also possible for the UE to provide UE-side assistance information to the network in advance. The UE can report the data as without any UE-side or NW-side assistance information. When receiving the data from UE, the network complements both the UE-side and NW-side assistance information.

Fig 3 NW-based solution

It is NW-side data collection, the NW needs to filter the data according to both NW and UE-side assistance information, however, the NW has no prior knowledge of UE-side assistance information, therefore in order to facilitate the aforementioned 3 options, the key aspect needs to be specified is the transmission of UE-side assistance information (e.g., additional conditions) from UE to NW. There are basically two solutions: 1) UE provides to the NW the UE-side assistance information (e.g., additional conditions) together with the data collection reporting; 2) The NW sends the UE-side assistance information (e.g., additional conditions) filtering configuration to the UE. Both solutions are reasonable, so we support to have both of the two proposals below:

Proposal 6 For NW-side data collection of beam management, UE provides to the NW the UE-side assistance information (e.g., additional conditions) together with the data collection reporting.

Proposal 7 For NW-side data collection of beam management, The NW sends the UE-side assistance information (e.g., additional conditions) filtering configuration to the UE.

Conclusion

In this contribution, we discuss the potential solutions for the NW-side data collection for model training of beam management. The following observation and proposals are considered:

Proposal 1 For the detailed parameters of L1 data collection, RAN2 waits for the RAN1 inputs.

Proposal 2 For data collection of beam management, the type for data logging can be periodical and event triggered.

Proposal 3 The event-based reporting is supported for logging scheme.

Proposal 4 The event triggering for logging scheme can be:

- The configured maximum entries for data collection are achieved.

- The storage for data collection at UE side is above/below the threshold.

Proposal 5 The NW can raise the threshold for data filtering or configure the maximum bits for the data collection to avoid the case when single RRC message is not sufficient.

Observation 1: To categorize the data, it is necessary to consider both UE-side and NW-side assistance information.

Proposal 6 For NW-side data collection of beam management, UE provides to the NW the UE-side assistance information (e.g., additional conditions) together with the data collection reporting.

Proposal 7 For NW-side data collection of beam management, The NW sends the UE-side assistance information (e.g., additional conditions) filtering configuration to the UE.

Reference

[1] R2_126_ChairNotes_Final, 3GPP TSG-RAN WG2 Meeting #126, Fukuoka, Japan May 20th – 24th, 2024.