3GPP TSG-RAN WG2 Meeting #127	R2-2406543

Maastricht, Netherlands, August 19th –23rd, 2024

Agenda item:	8.2.5

Source: 	Fujitsu

Title: 	Discussions on topology 2 related aspects

Document for:	Discussion and decision

1. Introduction

According to the New SID [1], the overall objective shall be to study a harmonized air interface design with minimized differences for Ambient IoT for Topologies 1 and 2.

This contribution will provide more consideration on Ambient IoT Topology 2.

2. Discussion

According to TR38.848, Topology 2 is shown as below:

Figure 4.2.1.2-1: Topology 2 [2]

In Topology 2, the Ambient IoT device communicates bidirectionally with an intermediate node between the device and the base station. In this topology, the intermediate node can be a relay, IAB node, UE, repeater, etc. which is capable of Ambient IoT. The intermediate node transfers Ambient IoT data and/or signalling between the BS and the Ambient IoT device. According to the SID, for Topology 2, UE acts as intermediate node under NW control is in the scope of the SI.

Observation 1: Topology 2 where UE acts as intermediate node under NW control is in the scope of the SI.

Architecture

In Topology 2, a UE can act as intermediate node under NW control. The UE will communicate with an Ambient IoT device using the air interface, as specified in TR 38.769 [3] and communicate with the network, RAN or CN using the existing interface with limited enhancements.

Figure 1: Architecture

Proposal 1: In Topology 2, a UE as an intermediate node under NW control comprises UE/MT functionality and reader functionality.

Observation 2: The UE as intermediate node communicates with an Ambient IoT device using the air interface, as specified in TR 38.769.

For the UE as intermediate node, communication with the network, e.g. gNB or CN, needs further study in RAN2.

Communication with the network takes the following aspects into account:

- Notification of UE as intermediate node

The network needs to provide the configuration for reader and thus it is necessary for the network to identify if a UE connects to the network as an intermediate node. This can be notified by the UE as a capability or a status.

- Allocation/management of resource over AIoT Uu interface

The reader communicates with served AIoT devices over the AIoT Uu interface. The used resources should be allocated by the gNB and thus the configuration from the network needs to be considered.

- Inventory/command procedure

Inventory/command procedure should be supported for both topologies. When the UE connects to the network as an intermediate node, related signaling or data transmission, e.g. inventory/command request and corresponding response between the UE and the network should be supported.

Based on these, the communication between the UE as intermediate node and the network including notification of UE as intermediate node, resource allocation and inventory/command procedures will be considered.

Proposal 2: Communication with the network to support UE as a reader/intermediate node should consider the following aspects:

- UE as intermediate node notification

- Allocation/management of resources over AIoT Uu interface

- Inventory/command forwarding

Protocol stack

Based on SA2 solutions [4], the protocol stack for an intermediate node can based on NAS layer or AS layer.

In Figure 2-1, the intermediate UE can be controlled by NAS signaling. The AIoT messages are contained in NAS messages and be forwarded via gNB transparently. If this architecture is selected for the Topology 2 scenario, SA2 and RAN3 will work on the NAS enhancements and RAN2 still needs to consider possible UE configuration by gNB.

Figure 2-2 illustrates the protocol stacks for AS based solution for intermediate UE. The AIoT data are carried by control plane messages in Uu interface. For example, the AIoT data can be contained in RRC messages or MAC CE between the gNB and UE (reader). RAN2 may further study which layer is used for gNB and intermediate UE to forward the AIoT data.

Figure 2-1: NAS based protocol stacks

Figure 2-2: AS based protocol stacks

It is important to coordinate with SA2 and RAN3 on the selection of the above protocols of an intermediate UE for Topology 2. RAN2 should study the pros and cons of the different protocol stacks and make decisions or suggestions to SA2.

Proposal 3: RAN2 studies NAS based and AS based protocol stacks.

Other aspects

Legacy UE supports different RRC states, including RRC idle, inactive and connected state. So, the UE as intermediate node can enter RRC idle state and data transmission is not performed. RAN2 needs to discuss the handling of Reader when the MT/UE enters RRC idle state.

The UE may initiate RRC reestablishment when a failure occurs. During the recovery, the UE cannot communicate with the network and thus the handling of Reader needs to be discussed as well.

Besides, if the UE moves far from the served AIoT devices, or if the UE performs the cell switch, the impact to the Reader and these served AIoT devices can be observed. RAN2 needs to further study the potential issues on this.

Proposal 4: RAN2 to discuss the following issues:

- Handling of Reader when RRC state of the MT/UE changes

- Handling of Reader when a failure or recovery occurs at the MT/UE

- Impact to Reader/served AIoT devices when the MT/UE moves, e.g. performing cell switch

3. Conclusion

In this contribution, we have some discussions on the Ambient IoT Topology 2 and made the following observations and proposals:

Observation 1: Topology 2 where UE acts as intermediate node under NW control is in the scope of the SI.

Proposal 1: In Topology 2, a UE as an intermediate node under NW control comprises UE/MT functionality and reader functionality.

Observation 2: The UE as intermediate node communicates with an Ambient IoT device using the air interface, as specified in TR 38.769.

Proposal 2: Communication with the network to support UE as a reader/intermediate node should consider the following aspects:

- UE as intermediate node notification

- Allocation/management of resources over AIoT Uu interface

- Inventory/command forwarding

Proposal 3: RAN2 studies NAS based and AS based protocol stacks.

Proposal 4: RAN2 to discuss the following issues:

- Handling of Reader when RRC state of the MT/UE changes

- Handling of Reader when a failure or recovery occurs at the MT/UE

- Impact to Reader/served AIoT devices when the MT/UE moves, e.g. performing cell switch

Reference

- RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#103, March 18-21, 2024

- 3GPP TR 38.848: "Technical Specification Group Radio Access Network; Study on Ambient IoT (Internet of Things) in RAN"

- 3GPP TR 38.769: "Study on solutions for Ambient IoT (Internet of Things) in NR"

- 3GPP TR 23.700-13: “Study on Architecture support of Ambient power-enabled Internet of Things”, v0.4.0