3GPP TSG-RAN WG2 Meeting #127	R2-2406545

Maastricht, Netherlands, August 19th –23rd, 2024

Agenda item:	8.6.3

Source: 	Fujitsu

Title: 	Open issues of measurement event evaluation for LTM

Document for:	Discussion and decision

1. Introduction

The following objective of the work item aims at specifying measurement related enhancements for New WID [1] on NR mobility enhancements:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

This contribution will provide more consideration on measurement event evaluation for LTM including the following aspects:

- What beam(s) of the serving cell and neighbouring cell will be used for event evaluation?

- Need of cell level measurement result for event evaluation (if needed, what’s use case and how it works)?

- Need of event LTM1 (if needed, what’s use case)?

- Modelling

- Further details on event configuration signaling design, e.g. how to associate with resource configuration

2. Discussion

2.1 Open issues for event evaluation

At RAN2#126, the following LTM events are supported as the L1 LTM measurement events:

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

However, there are some remaining issues need to be further discussed.

Issue 1: What beam(s) of the serving cell and neighbouring cell will be used for event evaluation?

As discussed, for beam(s) of the serving cell used for event evaluation, there are two options on the table:

- Option 1: RS associated with indicated beam for the serving cell

This option is adopted for event-triggered L1 measurement reporting for MIMO. It is straightforward to use RS associated with indicated beam as the beam of the serving cell for event evaluation since it can represent the radio link quality on serving cell.

However, if the indicated beam is associated with PCI of non-serving cell, LTM cell switch based on event-triggered L1 measurement report cannot be supported. In case of inter-cell beam management, the UE is using the beam from the neighbouring cell associated to the serving cell and thus the RS from the neighbouring cell is used for event evaluation. Event e.g. LTM2/3/5 cannot be satisfied since the quality of beam of serving cell used for event evaluation, i.e. the RS from the neighbouring cell, is good.

- Option 2: any/best beam for the serving cell

In this option, a set of RS(s) can be configured for the serving cell for L1 measurement. The UE can perform ranking to find a beat beam, i.e. RS with best quality in the RS set and use it for event evaluation. The ranking procedure incurs some delay for measurement reporting. So, it is valuable to consider any beam for the serving cell used for event evaluation. The concern on frequent measurement reporting can be solved by e.g. TTT mechanism.

If Option 2 is applied, LTM cell switch scenarios including cell switch after ICBM can be supported.

Considering that LTM events include the event only involving LTM candidate measurement and the measurement on beam of LTM candidate is more important than that on the serving cell, Option 1 is supported. If a set of RS(s) can be configured for the serving cell for L1 measurement, Option 2 can be supported as well.

Proposal 1: For event evaluations, RS associated with indicated beam for the serving cell is used.

Proposal 1bis: For event evaluations, RAN2 to discuss if configured RS of the serving cell can be used.

For beam(s) of the neighbouring cell used for event evaluation, there are two options to be considered:

- Option 1: any/best beam for the LTM candidate cell

In this option, a set of RSs can be configured for one or more LTM candidates for the L1 measurement, as in Rel-18.

The UE can perform ranking to find a beat beam, i.e. RS with best quality in the RS set and use it for event evaluation. The ranking procedure incurs some delay for measurement reporting. So, it is valuable to consider any beam for the serving cell used for event evaluation.

- Option 2: RS associated with the activated TCI state(s) or a subset of configured TCI state(s) for the LTM candidate cell

In this option, if early DL synchronization towards the LTM candidate is supported and TCI state(s) are activated, the RS associated with the activated TCI state(s) can be used as the beam(s) of the neighbouring cell for event evaluation.

Otherwise, the RS associated with a subset of configured TCI state(s) can be used as the beam(s) for event evaluation. The subset of TCI states can be configured by the gNB via RRC signaling.

Option 1 applies to UEs with different UE capabilities while Option 2 requires early DL synchronization even if it is more suitable since the UE will use the indicated beam in the LTM cell switch command, i.e. one of the activated TCI states, for data transmission in the target cell. Therefore, it is better to configure the RS by the network.

Proposal 2: For event evaluations, NW can configure the RS used for candidate cell, from:

- RS associated with activate TCI state(s)

- RS associated with a subset of TCI state(s) in the LTM configuration

- Any/best one of explicitly configured RS

NW can configure the RS explicitly in the event configuration or configure it implicitly. For example, if there are activated TCI state(s) for an LTM candidate, QCLed RS is used for the LTM candidate; otherwise, explicitly configured RS is used.

In addition, RAN2 assumes that the same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. Usually, SSB can be configured for LTM candidates and if CSI-RS can be configured needs more RAN1 information. If beam of LTM candidate used for event evaluation is an SSB, while beam of the serving cell used for event evaluation is a CSI-RS, the SSB which is QCLed with the CSI-RS should be used instead of the CSI-RS.

Proposal 3: The SSB which is QCLed with the CSI-RS should be used so that the same RS type are used for serving and neighbouring cell when event LTM3 and event LTM5 are evaluated.

Issue 2: Need of cell level measurement result for event evaluation

There are some concerns on ping-pong due to beam level measurement for event evaluation and thus there is one remaining issue, i.e. if cell level measurement result can be used for event evaluation. We think that the LTM decision is made by serving gNB/DU, e.g. based on beam specific L1 measurement result. To avoid ping-pong, multiple beams for candidate cell can be considered to make the LTM decision. This can be implemented by including the measurement result for multiple beams for the candidate cell for which the RS triggers L1 measurement reporting, rather than using cell level measurement result for event evaluation. So, cell level measurement result is not used for event evaluation.

Proposal 4: Cell level measurement result for event evaluation is not supported, at least in case of LTM decision made by NW.

Proposal 4bis: RAN2 to discuss whether cell level measurement result for event evaluation is needed for conditional LTM.

Issue 3: Need of event LTM1

In addition to event LTM2~LTM5, RAN2 is discussing if event LTM1 needs to be supported as one of the L1 LTM measurement events, i.e. beam of serving cell becomes better than absolute threshold. If event LTM1 is supported, the UE provides the L1 measurement result to the network when the serving cell is good enough. Based on the reporting, the NW can determine to stop early UL synchronization towards the LTM candidate without valid TA and thus the interruption to serving cell can be mitigated.

Proposal 5: Event LTM1 is supported.

Issue 4: Modelling

Regarding the framework for event-triggered L1 measurement reporting, one possibility is to reuse the framework of the event-triggered L3 measurement reporting. For event-triggered L3 measurement reporting, it involves measurement configuration, performing of measurement, measurement report triggering and measurement reporting. Similarly, event-triggered L1 measurement reporting can include L1 measurement configuration, performing of L1 measurement, measurement report triggering and measurement reporting.

Observation: Event-triggered L1 measurement reporting takes L3 measurement as baseline, comprising L1 measurement configuration, performing of L1 measurement, measurement report triggering and L1 measurement reporting.

L1 measurement configuration should be provided by RRC signalling, as legacy. However, for other aspects, the following modelling methods should be discussed, as shown in the figure below:

- Modelling 1:

  - MAC layer: measurement report triggering and L1 measurement reporting

  - Physical layer: performing of L1 measurement

- Modelling 2:

  - MAC layer: L1 measurement reporting

  - Physical layer: performing of L1 measurement and measurement report triggering

Figure 1: Modelling of event-triggered L1 measurement reporting

For Modelling 1, measurement report triggering can reuse the events for L3 measurement as much as possible. In this model, other metrics in addition to L1 measurement results can be considered, e.g. a timer or counting of some bad indications provided by the PHY layer for a serving cell.

For Modelling 2, measurement report triggering can take into account an immediate L1 measurement result of serving cell(s) or the candidate cell(s).

Because the Modelling of event-triggered L1 measurement reporting has impact to the RAN1/RAN2 work split, it is better to discuss this issue first. Modelling 1 is more aligned with the objectives in WID, i.e. led by RAN2. So, we slightly prefer Model 1.

Proposal 6: RAN2 to agree event evaluation for L1 measurement reporting is performed at MAC layer.

In addition, RAN2 also needs to discuss the level at which the event evaluation is performed. If RS set is configured for L1 measurement, the UE performs event evaluation per RS set. Otherwise, the UE performs event evaluation per candidate cell.

Proposal 7: For each RS set or each candidate cell, the MAC entity performs event evaluation to determine if the LTM event is satisfied or not for the RS set or for the candidate cell.

2.2 Further details on event configuration signaling design

In Rel-18, L1 measurement reporting via UCI is supported. To support this, the network configures both measurement resources and reporting resources by RRC signalling.

Figure 2: configuration of L1 measurement and reporting

As shown in Figure 2, the IE LTM-CSI-ReportConfig is used to configure report on the cell in which the LTM-CSI-ReportConfig is included while the IE LTM-CSI-ResourceConfig (included in LTM-Config) defines a group of one or more CSI resources for one or more LTM candidate configurations. To associate the RS configuration and reporting configuration, the ID for RS configuration, i.e. LTM-CSI-ResourceConfigId is included in each reporting configuration.

For event configuration, RAN2 has agreed that L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in the LTM configuration via RRC signaling. There can be different options for signaling design to implement this. For example, when RS(s) for L1 measurement are provided explicitly, a similar mechanism as in Rel-18 can be reused, i.e. RS resource ID (e.g. LTM-CSI-ResourceConfigId) is included in the event configuration. If RS for L1 measurement is not provided explicitly by RRC signaling, the UE performs event evaluation using RS associated with TCI state (s) and thus TCI state ID(s) is included in the event configuration for association.

Alternatively, L3 measurement like configuration is used for L1 measurement. In this mechanism, RS resource ID is used to identify the L1 measurement RS and a reporting configuration ID is introduced to define the event configuration or reporting configuration. An additional L1 measurement ID is used to associate the RS resource ID and the reporting configuration ID.

Proposal 8: RAN2 to discuss the following options to associate event configuration with resource configuration:

    - Option 1: RS resource ID is included in the event configuration

    - Option 2: L3 measurement style

3. Conclusion

In this contribution, we have some discussions on measurement event evaluation for LTM and made the following observation and proposals:

Open issues for event evaluation

Proposal 1: For event evaluations, RS associated with indicated beam for the serving cell is used.

Proposal 1bis: For event evaluations, RAN2 to discuss if configured RS of the serving cell can be used.

Proposal 2: For event evaluations, NW can configure the RS used for candidate cell, from:

- RS associated with activate TCI state(s)

- RS associated with a subset of TCI state(s) in the LTM configuration

- Any/best one of explicitly configured RS

Proposal 3: The SSB which is QCLed with the CSI-RS should be used so that the same RS type are used for serving and neighbouring cell when event LTM3 and event LTM5 are evaluated.

Proposal 4: Cell level measurement result for event evaluation is not supported, at least in case of LTM decision made by NW.

Proposal 4bis: RAN2 to discuss whether cell level measurement result for event evaluation is needed for conditional LTM.

Proposal 5: Event LTM1 is supported.

Observation: Event-triggered L1 measurement reporting takes L3 measurement as baseline, comprising L1 measurement configuration, performing of L1 measurement, measurement report triggering and L1 measurement reporting.

Proposal 6: RAN2 to agree event evaluation for L1 measurement reporting is performed at MAC layer.

Proposal 7: For each RS set or each candidate cell, the MAC entity performs event evaluation to determine if the LTM event is satisfied or not for the RS set or for the candidate cell.

Further details on event configuration signaling design

Proposal 8: RAN2 to discuss the following options to associate event configuration with resource configuration:

    - Option 1: RS resource ID is included in the event configuration

    - Option 2: L3 measurement style

Reference

- RP-241515, “Revised Work Item: NR mobility enhancements Phase 4”, RAN#104, Jun 17-20, 2024