3GPP TSG-RAN WG2 Meeting #127	R2-2406546

Maastricht, Netherlands, August 19th –23rd, 2024

Agenda item:	8.6.4

Source: 	Fujitsu

Title: 	Open issues of measurement reporting procedure for LTM

Document for:	Discussion and decision

1. Introduction

The following objective of the work item aims at specifying measurement related enhancements for New WID [1] on NR mobility enhancements:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

This contribution will provide more consideration on the measurement reporting procedure, including:

- Information in the measurement report

- MAC CE or UCI for measurement report

- Need of measurement reporting once leaving condition is met

- Other aspects

2. Discussion

Container

In Rel-18 LTM, the IE LTM-CSI-ReportConfig is used to configure a periodic, semi-persistent or aperiodic L1 measurement report. The resources are pre-configured or triggered by the network. This cannot be applied for the event-triggered L1 measurement reporting because the network cannot predict when and how many the resources will be used by the event-triggered L1 measurement reporting. If UCI is used for the event-triggered L1 measurement reporting, the resources may be wasted since the L1 measurement report is not transmitted when the condition is not satisfied. Instead, the resource for the L1 measurement report can be e.g. requested by the UE and thus the resource waste can be avoided if MAC CE is used. Also, the variable size of the L1 measurement report can be supported. So, a new MAC CE should be defined for the event-triggered L1 measurement reporting.

Proposal 1: A new MAC CE is introduced for event-triggered L1 measurement reporting.

Information in the measurement report

Regarding the content of the MAC CE, from RAN2 point of view, some information related to the event which triggers the L1 measurement reporting needs to be included in the measurement report. With the information, the network can associate with the measurement results with event configuration, e.g. event type, related threshold(s) and offset and to make decision including LTM cell switch.

Proposal 2: From RAN2 perspective, event ID-like information is included in the MAC CE for L1 measurement reporting.

In addition, at least the measurement results should be included. The measurement quantity to be reported and if only single measurement quantity to be reported need to be discussed in RAN1. Also, how to identify a measurement result, e.g. for which RS or for which candidate cell, should be in the scope of RAN1.

In Rel-18 LTM, the IE LTM-CSI-ReportConfig is also used to configure the content of L1 measurement report, e.g. the number of reported cells, the number of reported RS per cell. If such consideration for event-triggered L1 measurement reporting is needed, RAN1 will discuss this.

Therefore, although a MAC CE is used as the container of event-triggered L1 measurement report, the details of the content need some RAN1 inputs.

Proposal 2bis: Regarding the content of the MAC CE, RAN1 input is required.

Support of report on leaving

For L3 measurement reporting, entering condition and leaving condition are defined and measurement reporting on leaving condition can be configured by the gNB. With this, the network can make decision on LTM cell switch or early synchronization based on the newly received L1 measurement results. So, ping-pong or too early synchronization can be mitigated. We propose that a similar mechanism can be applied for event triggered L1 measurement reporting.

Proposal 3: L1 Measurement reporting on leaving condition can be supported.

Other aspects

For L3 measurement reporting, to address transmission failure of the L3 measurement report, a timer T312 can be configured. The timer T312 can be started if the L3 measurement report is triggered during T310 is running. Failure reporting or connection re-establishment procedure is initiated when the timer expired.

For L1 measurement reporting, RAN2 has agreed that the event triggered L1 measurement reporting should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization

- Select the target beam/cell and trigger LTM cell switch procedure

A similar mechanism can be used for the L1 measurement reporting triggered by the events defined for LTM decision. With this, the UE can take some actions when the transmission of the event triggered L1 measurement report fails.

Proposal 4: If the event is configured for LTM decision, RAN2 considers T312-like mechanism.

These LTMx events can also be the configured to select the candidate beam/cell to trigger early synchronization. Then, if the quality of candidate cell changes, the activated TCI states could be deactivated. Depending on the quality of the candidate cell, the network may or may not transmit the PDCCH order to trigger early TA acquisition again e.g. when the TA becomes invalid. So, multiple or periodic reporting after the event-triggered L1 measurement reporting needs to be considered.

Proposal 5: If the event is configured for early synchronization decision, multiple or periodic reporting after the event-triggered reporting is considered.

3. Conclusion

In this contribution, we have some discussions on potential measurement enhancements for LTM and made the following proposals:

Proposal 1: A new MAC CE is introduced for event-triggered L1 measurement reporting.

Proposal 2: From RAN2 perspective, event ID-like information is included in the MAC CE for L1 measurement reporting.

Proposal 2bis: Regarding the content of the MAC CE, RAN1 input is required.

Proposal 3: L1 Measurement reporting on leaving condition can be supported.

Proposal 4: If the event is configured for LTM decision, RAN2 considers T312-like mechanism.

Proposal 5: If the event is configured for early synchronization decision, multiple or periodic reporting after the event-triggered reporting is considered.

Reference

- RP-241515, “Revised Work Item: NR mobility enhancements Phase 4”, RAN#104, June 17-20, 2024