3GPP TSG-RAN WG2 Meeting #127	R2-2406549

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.7.5

Source: 	Fujitsu

Title: 	Discussions on RLC enhancements

Document for:	Discussion and decision

Introduction

On RLC retransmission enhancement in XR, RAN2 has made the following agreements in RAN2#126 meeting on avoiding unnecessary retransmissions [1]:

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned

For ensuring timely RLC retransmissions, the following agreements are reached:

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

In this contribution, we would like to discuss the potential enhancements to achieve timely RLC retransmissions and to avoid unnecessary RLC retransmissions for XR.

Discussion

Ensuring timely RLC retransmission(s) for XR:

As agreed by RAN2, the following options will be considered:

- Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered)

- Retransmission based on enhanced status report

- Retransmission based on enhanced polling

Among these options, autonomous retransmission can achieve high reliability for XR traffics in the small PDB. In the last meeting, some companies think that the overhead of autonomous retransmission is large, and they have concern on the system capacity. From our perspective, autonomous retransmission can be applied in certain cases to save the radio resource occupation. For example, the trigger of autonomous retransmission can be e.g., when the remaining time of the data becomes lower than a threshold, or when the network configures the UE to apply autonomous retransmission. When PDB is reached, the autonomous retransmission in RLC should be stopped. Therefore, the impact of autonomous retransmission to the system capacity can be acceptable.

On the other hand, the options of retransmission based on enhanced status report or enhanced polling also have overhead, since the UE would transmit polling and/or the network would transmit the status reports more frequently with configured small value of pollPDU or pollByte or t_reassembly timer. The transmission of polling and status reports would delay the transmission of data, which also has impact on the system capacity.

Observation 1: Autonomous retransmission has acceptable impact on system capacity with proper triggers.

In the options of retransmission based on enhanced status report or enhanced polling, the round-trip-time of an RLC retransmission may still be long since there may be HARQ retransmissions of the status report. From this point of view, retransmission based on enhanced status report or enhanced polling may not be so fast to achieve timely RLC retransmission within PDB.

Observation 2: Retransmission based on enhanced status report or enhanced polling may not achieve timely RLC retransmission within PDB.

Based on the above observations, autonomous retransmission may achieve better performance than retransmission based on enhanced status report or enhanced polling. So, we propose to adopt autonomous retransmission to achieve timely RLC retransmission, i.e., the transmitting side of an AM RLC entity can perform retransmission even if it does not receive the negative acknowledgement. By this way, the number of RLC retransmission(s) is increased in the PDB compared with the current scheme, and thus the reliability of the XR traffic can be ensured.

Proposal 1: RAN2 adopts autonomous retransmissions to achieve timely RLC retransmission in RLC AM.

Proposal 1a: Autonomous retransmission can be activated/deactivated by the network or can be triggered when the remaining time of the data becomes lower than a threshold.

The autonomous RLC retransmission(s) can occur in the consecutive slots or can have an interval. If positive acknowledgement is received by the transmitting side of the AM RLC entity, these autonomous RLC retransmission(s) can be stopped. If negative acknowledgement is received by the transmitting side of the AM RLC entity, the transmitting side of the AM RLC entity can continue the autonomous RLC retransmissions.

The number of the autonomous RLC retransmissions can be configured by the network or can be specified in RLC. In our opinion, these autonomous RLC retransmissions should not be counted into RETX_COUNT, so that RETX_COUNT would not easily reach the maximum retransmission number (maxRetxThreshold) and RLF would not be initiated.

Proposal 2: The number of the autonomous RLC retransmissions can be configured by the network or can be specified in RLC.

Proposal 3: The autonomous RLC retransmissions are not counted into RETX_COUNT.

Avoiding unnecessary retransmissions:

In RAN2#126 meeting, RAN2 have extensive discussions on the following approaches and agrees to adopt one of them to avoid unnecessary RLC AM retransmissions:

1. Rx initiated approach

2. Tx initiated approach

For Rx initiated approach, there is an FFS on how Rx side determines that an SDU should be abandoned. In our opinion, there could be following options:

- Option 1: a timer is used in the Rx side to determine the abandoned RLC SDU.

- Option 2: Rx PDCP indicates to Rx RLC about the abandoned RLC SDU.

In Option 1, for example, the timer associated with an RLC SDU can be started when another RLC SDU with a higher SN is received in the Rx side. If the RLC SDU is received, the timer associated with it can be stopped if the timer is running. When the timer expires, Rx side can consider the associated RLC SDU as abandoned.

In Option 2, Rx PDCP can determine the abandoned RLC SDU e.g., when t-Reordering timer expires, or when a new timer similar to PDCP discard timer expires, or when a PDCP SN gap report is received from Tx PDCP if the PDCP SN gap report can indicate the discarded PDCP SDUs which has been submitted by Tx RLC to lower layers. When Rx PDCP determines the abandoned RLC SDU, it can inform Rx RLC of the abandoned RLC SDU.

Proposal 4: for Rx initiated approach, Rx side determines the abandoned RLC SDU based on a timer or Rx PDCP indication.

For Tx initiated approach, a new RLC SN gap report should be transmitted from the Tx side to the Rx side. In the RLC SN gap report, the obsolete RLC SDUs will be indicated, so that Rx side can advance its receiving window. In the online discussions, many companies do not prefer the Tx initiated approach due to its complexity and big impact to standard. From our perspective, Rx initiated approach seems simpler.

Proposal 5: Rx initiated approach is adopted for avoiding unnecessary retransmissions.

Conclusion

In this contribution, we have discussed the RLC enhancements to achieve timely RLC retransmission and to avoid unnecessary retransmissions. We have the following observations and proposals:

Observation 1: Autonomous retransmission has acceptable impact on system capacity with proper triggers.

Observation 2: Retransmission based on enhanced status report or enhanced polling may not achieve timely RLC retransmission within PDB.

Proposal 1: RAN2 adopts autonomous retransmissions to achieve timely RLC retransmission in RLC AM.

Proposal 1a: Autonomous retransmission can be activated/deactivated by the network or can be triggered when the remaining time of the data becomes lower than a threshold.

Proposal 2: The number of the autonomous RLC retransmissions can be configured by the network or can be specified in RLC.

Proposal 3: The autonomous RLC retransmissions are not counted into RETX_COUNT.

Proposal 4: for Rx initiated approach, Rx side determines the abandoned RLC SDU based on a timer or Rx PDCP indication.

Proposal 5: Rx initiated approach is adopted for avoiding unnecessary retransmissions.

References

- R2-2405706, RAN2#126 Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei)