3GPP TSG-RAN WG2 Meeting #127	R2-2406554

Maastricht, Netherlands, Aug 19th -23rd, 2024

Agenda Item:	7.15.2

Source:	NEC

Title:	TP for SL CA in TS 38.300

Document for:	Discussion, Decision

Introduction

In this contribution, we will discuss some remaining issues in current spec for SL enhancement.

Discussion

Technically speaking, fo SL CA carrier selection, it should be performed at MAC layer for all of the broadcast, groupcast and unicast service. The difference between unicast and groupcast/broadcast is that for broadcast and groupcast, the configured carriers are coming from the UE’s upper layer only, which is V2X layer. But for unicast, the configured carriers should not only consider the UE’s upper layer configuration, but also consider peer UE’s SL CA related capability and carrier configuration. That is why SL CA for unicast is not applied until the carrier configuration signalling is complete.

However, besides the difference, carrier selection for broadcast, groupcast and unicast services should all be performed in MAC layer, which is quite clear interpretation. However, according to current specification, in TS 38.300, it is only mentioned that for groupcast and broadcast, carrier selection is performed at MAC layer. For unicast, which layer is performing carrier selection is not mentioned, as captured in the below table.

TS 38.300 For groupcast and broadcast, carrier selection is performed at MAC layer, depending on the CBR of the configured carriers and logical channel priority. In order to avoid frequent switching across different carriers, the UE may keep using a carrier already selected for transmission, if the measured CBR on this carrier is lower than a (pre)configured threshold. For a UE using mode 2 resource allocation, logical channel prioritization is performed for a sidelink resource on a carrier depending on the CBR measured on the carrier and the logical channel priority, as specified in TS 38.321 [6]. For unicast, SL CA related capability is exchanged between the TX UE and RX UE, and the TX UE delivers the carrier configuration to the RX UE in PC5-RRC. For unicast, carrier selection and logical channel prioritization is performed similar to groupcast and broadcast among the carriers configured in the carrier configuration. SL CA for unicast is not applied until the carrier configuration signalling is complete. When the TX UE detects carrier failure on a specific carrier, carrier (re)selection is performed by excluding the failed carrier, as specified in TS 38.321 [6] and a new carrier configuration is sent to the RX UE as specified in TS 38.331 [12].

On the other hand, in MAC spec, for the section of carrier selection, which is 5.22.1.11, it does not mention whether this section is applicable to all cast types. So whether unicast carrier selection is also perform at MAC layer is still unclear in the specification.

Therefore, it is suggested to explicitly mention that for unicast, carrier selection is also performed at MAC layer.

It is suggested to explicitly mention that for unicast, carrier selection is also performed at MAC layer, in TS 38.300.

Correspondingly, it is suggested to adopt the TP in the annex.

Proposal 2	It is suggested to adopt the TP in the annex

Conclusion

In this contribution, we have discussed the remaining issue in TS 38.300 for sidelink carrier aggregation, the brunch of proposals are given in the following:

Proposal 1	It is suggested to explicitly mention that for unicast, carrier selection is also performed at MAC layer, in TS 38.300.

Proposal 2	It is suggested to adopt the TP in the annex

Annex

### TP1 ### 16.9.10 Sidelink CA Carrier aggregation (CA) in sidelink is supported for mode 2 and in V2X case only. It applies to both in coverage UEs and out of coverage UEs. Each resource pool (pre)configured for sidelink is associated to a single carrier. A UE using mode 2 resource allocation performs carrier (re)selection and may select one or more carriers used for sidelink. The carrier(s) that can be used for transmitting data are provided by the V2X layer per QoS flow, and LCP ensures that data from a SLRB is transmitted on a carrier for which all mapped QoS flows are allowed to use the carrier. For groupcast and broadcast, when the V2X layer provides multiple carriers in QoS flow to carrier mapping information to the AS, TX profile is used to indicate whether the transmission corresponding to the QoS flow is backward compatible or not. When backward compatibility is needed, the TX UE uses only the legacy carrier without PDCP duplication, or uses PDCP duplication with at least the legacy carrier. A UE assumes backward compatibility is needed for the QoS flow if there is no associated TX profile. For groupcast and broadcast, carrier selection is performed at MAC layer, depending on the CBR of the configured carriers and logical channel priority. In order to avoid frequent switching across different carriers, the UE may keep using a carrier already selected for transmission, if the measured CBR on this carrier is lower than a (pre)configured threshold. For a UE using mode 2 resource allocation, logical channel prioritization is performed for a sidelink resource on a carrier depending on the CBR measured on the carrier and the logical channel priority, as specified in TS 38.321 [6]. For unicast, SL CA related capability is exchanged between the TX UE and RX UE, and the TX UE delivers the carrier configuration to the RX UE in PC5-RRC. For unicast, carrier selection and logical channel prioritization is performed at MAC layer, similar to groupcast and broadcast among the carriers configured in the carrier configuration. SL CA for unicast is not applied until the carrier configuration signalling is complete. When the TX UE detects carrier failure on a specific carrier, carrier (re)selection is performed by excluding the failed carrier, as specified in TS 38.321 [6] and a new carrier configuration is sent to the RX UE as specified in TS 38.331 [12]. Sidelink packet duplication is supported for sidelink CA and is performed at PDCP layer. For sidelink packet duplication for transmission, a PDCP PDU is duplicated at the PDCP entity. The duplicated PDCP PDUs of the same PDCP entity are submitted to two different RLC entities and associated to two different sidelink logical channels respectively. The duplicated PDCP PDUs of the same PDCP entity are only allowed to be transmitted on different sidelink carriers. For a SL DRB, sidelink packet duplication is either (pre)configured in the bearer configuration if the TX profile indicates backward compatibility is not needed, or is decided by the TX UE if the TX profile indicates backward compatibility is needed. For applicable SL SRBs, whether to use duplication is decided by the TX UE. In unicast, the TX UE sends the duplication configuration to the RX UE in PC5-RRC. There are specified logical channel identities which apply to the sidelink logical channel used for sidelink packet duplication exclusively as specified in TS 38.321 [6].