3GPP TSG-RAN WG2 Meeting #127                                                                    R2-2406555

Maastricht, NL, 19th – 23rd Aug, 2024

Source:	CATT

Title:	Further Consideration on DRX SFN Counter Initialization

Agenda Item:	7.5.3

Document for:	Discussion and Decision

Introduction

In RAN2#125 meeting, DRX SFN counter initialization was discussed with the following agreement:

=> The intention is that the UE should initialize this counter at the same time as checking whether the SFN is in the first part of second part of HFN. FFS about the TP

In RAN2#126 meeting, regarding to the TP, following conclusion was achieved:

=> Remove the sentence “in the first symbol of the slot immediately after the first PUSCH transmission which contains the RRCReconfigurationComplete message of the RRC (re-) configuration as specified in TS 38.331 [5]” from the procedure text. Adopt the TP for TS 38.321 for DRX_SFN_COUNTER initialization. FFS HO case

In this contribution, further considerations on DRX SFN counter initialization will be given based on some special cases, including mobility in RRC_CONNECTED.

Discussion

Timeline of DRX SFN counter initialization

Figure 1 Timeline of DRX SFN counter initialization

As shown in Figure 1:

- T is the time when the gNB sends RRC message with non-integer DRX configuration.

- T1 is the time when the UE receives RRC message with non-integer DRX configuration.

- T’ is the time when the UE initialize DRX SFN counter.

For normal RRC (re)-configuration case, the time gap between T1 and T’ is limited and can be ignorable. Therefore, the procedure for TS 38.321 for DRX SFN counter initialization is [1]:

(Text omitted) 1> if the drx-NonIntegerLongCycleStartOffset is configured: 2> increment DRX_SFN_COUNTER by 1 in the first symbol of a slot in which SFN changes to 0; 2> if DRX is (re-)configured by RRC: 3> if drx-TimeReferenceSFN is included in the RRC (re-)configuration which is received during the first half of a hyper frame (i.e., SFN is between 0 and 511): 4> set DRX_SFN_COUNTER to 1. 3> else: 4> set DRX_SFN_COUNTER to 0.

Mobility in RRC_CONNECTED

Mobility in RRC_CONNECTED includes normal HO case, Conditional HO case, and LTM case. In this section, we will analyze DRX SFN counter initialization issues with these special cases.

- Normal HO case

When the source cell configures non-integer DRX configuration to a UE, the target gNB may keep reconfigure non-integer DRX configuration with some updated configurations. Or when the source cell doesn’t support non-integer DRX configuration, the target gNB may configure non-integer DRX configuration if it supports this feature. Therefore, it is possible that the target gNB may include non-integer DRX configuration during normal HO case.

During normal HO case, the corresponding time point in figure 1 is:

- T1: The time when the UE receives RRCReconfiguration message which contains non-integer DRX configuration and the spCellConfig with reconfigurationWithSync.

The UE starts timer T304 in T1.

- T’: The time when the UE initialize DRX SFN counter.

According to the following text procedure in TS 38.331 [2], the UE stops timer T304 if HO procedure is successfully completed. Then the UE initializes DRX SFN counter upon acquiring the SFN of that target SpCell as it requires the UE to know the SFN of the respective target SpCell.

5.3.5.3 Reception of an RRCReconfiguration by the UE (Text omitted) 1> if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and when MAC of an NR cell group successfully completes a Random Access procedure triggered above; or, 1> if sl-PathSwitchConfig was included in reconfigurationWithSync included in spCellConfig of an MCG, and when successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE); or, 1> if rach-LessHO was included in reconfigurationWithSync included in spCellConfig of an MCG, and upon indication from lower layers that the RACH-less handover has been successfully completed; or, 1> if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and the RRCReconfiguration message is applied due to an LTM cell switch execution and upon an indication from lower layer that the LTM cell switch execution has been successfully completed: 2> stop timer T304 for that cell group if running; 2> if sl-PathSwitchConfig was included in reconfigurationWithSync: 3> if the sl-IndirectPathMaintain is not included in reconfigurationWithSync: 4> stop timer T420; 4> release all radio resources, including release of the RLC entities and the MAC configuration at the source side; 4> reset MAC used in the source cell; 3> else (sl-IndirectPathMaintain is included): 4> release radio resources on the direct path, including release of the RLC entities and the MAC configuration; 4> reset MAC used in the source cell; 2> if rach-LessHO was included in reconfigurationWithSync and cg-RRC-Configuration was configured: 3> release the uplink grant configured for RACH-less handover; NOTE 2b: PDCP and SDAP configured by the source prior to the path switch that are reconfigured and re-used by target when delta signalling is used, are not released as part of this procedure. 2> stop timer T310 for source SpCell if running; 2> apply the parts of the CSI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the respective target SpCell, if any; 2> apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the respective target SpCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of that target SpCell;

Observation 1: When UE receives RRCReconfiguration message which contains non-integer DRX configuration and the spCellConfig with reconfigurationWithSync, the UE starts timer T304.

Observation 2: The UE initializes DRX SFN counter upon acquiring the SFN of that target SpCell which is not earlier than the time that the UE stops timer T304 when HO procedure is successfully completed.

The time gap between T’ and T1 is not bigger than timer T304. The following is the possible value for timer T304 [2]. Therefore, the time gap between T’ and T1 is not small, but is less than a hyper frame.

t304 ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000}

Observation 3: For normal HO case, the time gap between the time when the UE initializes DRX SFN counter (T’) and the time when the UE receives RRCReconfiguration message which contains non-integer DRX configuration and the spCellConfig with reconfigurationWithSync (T1) is not small, but is less than a hyper frame.

The current procedure for DRX SFN counter initialization is not applicable for normal HO case as it is assumed that there is no time gap between the time when the UE initialize DRX SFN counter (T’) and the time when the UE receives RRC message with non-integer DRX configuration (T1). For example: the UE may receive non-integer DRX configuration with drx-TimeReferenceSFN (T1) during the second half of a hyper frame, and initialize DRX SFN counter at the first half of the next hyper frame (T’). The UE sets DRX_SFN_COUNTER to 0 according to current procedure, while DRX_SFN_COUNTER in the network side is 1. The misalignment of DRX_SFN_COUNTER between the network and the UE exists.

Observation 4: The current procedure for DRX SFN counter initialization is not applicable for normal HO case as it is assumed that there is no time gap between the time when the UE initialize DRX SFN counter (T’) and the time when the UE receives RRC message with non-integer DRX configuration (T1).

To handle the possible misalignment of DRX_SFN_COUNTER between the network and the UE, the possible options are:

- Option 1: non-integer DRX configuration is not configured during HO procedure.

With the option, non-integer DRX configuration can not be configured during HO. Instead, non-integer DRX configuration can be configured via RRCReconfiguration after HO procdure is successfully completed if the target cell want to use non-integer DRX configuration for the UE. With the option, the current procedure for DRX SFN counter initialization can be reused without change. The only specification impact is that the condition when the non-integer DRX configuration is configured should be added. As shown in Text Proposal.

- Option 2: Modify the procedure of DRX SFN counter initilization

With observation 3, the time gap between T’ (the time when the UE initialize DRX SFN counter) and T1 (the time when the UE receives non-integer DRX configuration) is not small, but is less than a hyper frame for normal HO case. Then when SFN between source cell and target cell is synchronous, the possible cases and the initialization of DRX SFN counter are summarized below:

Table 1 initialization of DRX SFN counter with SFN synchronization during normal HO

| Case | T | T1 | T’ | DRX SFN counter |
| --- | --- | --- | --- | --- |
| Case 1 | drx-TimeReferenceSFN is absent with non-integer DRX configuration | T1 is in the first half of a hyper frame | T’ is in the first half of a hyper frame | If SFN of T’ is less than SFN of T1, it means SFN has been wrap around, DRX_SFN_COUNTER is set to 1; Otherwise, DRX_SFN_COUNTER is set to 0. |
| Case 2 | drx-TimeReferenceSFN is absent with non-integer DRX configuration | T1 is in the first half of a hyper frame | T’ is in the second half of a hyper frame | DRX_SFN_COUNTER is set to 0. |
| Case 3 | drx-TimeReferenceSFN is absent with non-integer DRX configuration | T1 is in the second half of a hyper frame | T’ is in the first half of a hyper frame | DRX_SFN_COUNTER is set to 1. |
| Case 4 | drx-TimeReferenceSFN is absent with non-integer DRX configuration | T1 is in the second half of a hyper frame | T’ is in the second half of a hyper frame | If SFN of T’ is less than SFN of T1, DRX_SFN_COUNTER is set to 1; otherwise, DRX_SFN_COUNTER is set to 0. |
| Case 5 | drx-TimeReferenceSFN is included with non-integer DRX configuration | T1 is in the first half of a hyper frame | T’ is in the first half of a hyper frame | If SFN of T’ is less than SFN of T1, DRX_SFN_COUNTER is set to 2; otherwise, DRX_SFN_COUNTER is set to 1. |
| Case 6 | drx-TimeReferenceSFN is included with non-integer DRX configuration | T1 is in the first half of a hyper frame | T’ is in the second half of a hyper frame | DRX_SFN_COUNTER is set to 1. |
| Case 7 | drx-TimeReferenceSFN is included with non-integer DRX configuration | T1 is in the second half of a hyper frame | T’ is in the first half of a hyper frame | DRX_SFN_COUNTER is set to 1. |
| Case 8 | drx-TimeReferenceSFN is included with non-integer DRX configuration | T1 is in the second half of a hyper frame | T’ is in the second half of a hyper frame | If SFN of T’ is less than SFN of T1, DRX_SFN_COUNTER is set to 1; otherwise, DRX_SFN_COUNTER is set to 0. |

When SFN between source cell and target cell is asynchronous, SFN between T’ and T1 are not comparable. Then for this case, it is better for the UE to record the time duration between T’ and T1, e.g. start a timer in T1.

Considering the complexity of option 2, option 1 is preferred, i.e. non-integer DRX configuration is not configured during HO procedure. Furthermore, for PSCell Addition/PSCell change cases with reconfiguration with sync, the time gap between T’ and T1 is same to the normal HO. The similar handling for non-integer DRX configuration can be reused for these two cases.

Proposal 1: Non-integer DRX configuration is not configured during HO, and PSCell Addition/PSCell change cases with reconfiguration with sync.

- Conditional HO, CPAC and LTM cases

During mobility in RRC_CONNECTED, Conditional HO or LTM may be configured and performed. For these cases, the time when the UE initialize DRX SFN counter (T’) is uncertain. Therefore, the time gap between T’ and T1 (the time when the UE receives non-integer DRX configuration) may be larger than a hyper frame. And for CPAC (Conditional PSCell Addition or Change), the similar time gap between T’ and T1 exists.

Observation 5: For CHO, CPAC and LTM, the time gap between the time when the UE initializes DRX SFN counter (T’) and the time when the UE receives RRCReconfiguration message which contains non-integer DRX configuration and the spCellConfig with reconfigurationWithSync (T1) is uncertain, may be larger than a hyper frame.

With observation 5, option 2 is not applicable to Conditional HO, CPAC and LTM cases either.

Proposal 2: Non-integer DRX configuration is not configured for CHO, CPAC and LTM cases.

And for HO case, PSCell Addition/PSCell change cases, Conditional HO, CPAC and LTM case, the RRC message always contains reconfiguration with sync. Therefore, we propose that non-integer DRX configuration is not configured if reconfiguration is contained for the cell group. The corresponding TP is provided in clause 5.

Conclusion

According to the analysis in section 2, we propose:

Observation 1: When UE receives RRCReconfiguration message which contains non-integer DRX configuration and the spCellConfig with reconfigurationWithSync, the UE starts timer T304.

Observation 2: The UE initializes DRX SFN counter upon acquiring the SFN of that target SpCell which is not earlier than the time that the UE stops timer T304 when HO procedure is successfully completed.

Observation 3: For normal HO case, the time gap between the time when the UE initializes DRX SFN counter (T’) and the time when the UE receives RRCReconfiguration message which contains non-integer DRX configuration and the spCellConfig with reconfigurationWithSync (T1) is not small, but is less than a hyper frame.

Observation 4: The current procedure for DRX SFN counter initialization is not applicable for normal HO case as it is assumed that there is no time gap between the time when the UE initialize DRX SFN counter (T’) and the time when the UE receives RRC message with non-integer DRX configuration (T1).

Proposal 1: Non-integer DRX configuration is not configured during HO, and PSCell Addition/PSCell change cases with reconfiguration with sync.

Observation 5: For CHO, CPAC and LTM, the time gap between the time when the UE initializes DRX SFN counter (T’) and the time when the UE receives RRCReconfiguration message which contains non-integer DRX configuration and the spCellConfig with reconfigurationWithSync (T1) is uncertain, may be larger than a hyper frame.

Proposal 2: Non-integer DRX configuration is not configured for CHO, CPAC and LTM cases.

The corresponding TP is provided in clause 5.

Reference

- 3GPP TS 38.321 V18.2.0, Medium Access Control (MAC) protocol specification.

- 3GPP TS 38.331 V18.2.0, NR; Radio Resource Control (RRC); Protocol specification.

Text Proposal

## TP for TS 38.331

<<<<<<<<<<<<<<<<<<<< Start of change >>>>>>>>>>>>>>>>>>>>

6.3.2	Radio resource control information elements

[…]

–	MAC-CellGroupConfig

The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX.

MAC-CellGroupConfig information element

-- ASN1START

-- TAG-MAC-CELLGROUPCONFIG-START

MAC-CellGroupConfig ::=             SEQUENCE {

drx-Config                          SetupRelease { DRX-Config }                                     OPTIONAL,   -- Need M

schedulingRequestConfig             SchedulingRequestConfig                                         OPTIONAL,   -- Need M

bsr-Config                          BSR-Config                                                      OPTIONAL,   -- Need M

tag-Config                          TAG-Config                                                      OPTIONAL,   -- Need M

phr-Config                          SetupRelease { PHR-Config }                                     OPTIONAL,   -- Need M

skipUplinkTxDynamic                 BOOLEAN,

...,

[[

csi-Mask                            BOOLEAN                                                         OPTIONAL,   -- Need M

dataInactivityTimer                 SetupRelease { DataInactivityTimer }                            OPTIONAL    -- Cond MCG-Only

]],

[[

usePreBSR-r16                       ENUMERATED {true}                                               OPTIONAL,   -- Need R

schedulingRequestID-LBT-SCell-r16   SchedulingRequestId                                             OPTIONAL,   -- Need R

lch-BasedPrioritization-r16         ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

schedulingRequestID-BFR-SCell-r16   SchedulingRequestId                                             OPTIONAL,   -- Need R

drx-ConfigSecondaryGroup-r16        SetupRelease { DRX-ConfigSecondaryGroup-r16 }                   OPTIONAL    -- Need M

]],

[[

enhancedSkipUplinkTxDynamic-r16     ENUMERATED {true}                                               OPTIONAL,   -- Need R

enhancedSkipUplinkTxConfigured-r16  ENUMERATED {true}                                               OPTIONAL    -- Need R

]],

[[

intraCG-Prioritization-r17          ENUMERATED {enabled}                        OPTIONAL,    -- Cond LCH-PrioWithReTxTimer

drx-ConfigSL-r17                    SetupRelease { DRX-ConfigSL-r17 }           OPTIONAL,    -- Need M

drx-ConfigExt-v1700                 SetupRelease { DRX-ConfigExt-v1700 }        OPTIONAL,    -- Need M

schedulingRequestID-BFR-r17         SchedulingRequestId                         OPTIONAL,    -- Need R

schedulingRequestID-BFR2-r17        SchedulingRequestId                         OPTIONAL,    -- Need R

schedulingRequestConfig-v1700       SchedulingRequestConfig-v1700               OPTIONAL,    -- Need M

tar-Config-r17                      SetupRelease { TAR-Config-r17  }                                OPTIONAL,    -- Need M

g-RNTI-ConfigToAddModList-r17       SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17       OPTIONAL,    -- Need N

g-RNTI-ConfigToReleaseList-r17      SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfigId-r17     OPTIONAL,    -- Need N

g-CS-RNTI-ConfigToAddModList-r17    SEQUENCE (SIZE (1..maxG-CS-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17    OPTIONAL,    -- Need N

g-CS-RNTI-ConfigToReleaseList-r17   SEQUENCE (SIZE (1..maxG-CS-RNTI-r17)) OF MBS-RNTI-SpecificConfigId-r17  OPTIONAL,    -- Need N

allowCSI-SRS-Tx-MulticastDRX-Active-r17   BOOLEAN                                                           OPTIONAL     -- Need M

]],

[[

schedulingRequestID-PosMG-Request-r17 SchedulingRequestId                                                   OPTIONAL,    -- Need R

drx-LastTransmissionUL-r17          ENUMERATED {enabled}                                                    OPTIONAL     -- Need R

]],

[[

posMG-Request-r17                   ENUMERATED {enabled}                                                    OPTIONAL     -- Need R

]],

[[

drx-ConfigExt2-v1800                SetupRelease { DRX-ConfigExt2-v1800 }                                   OPTIONAL,    -- Need M

additionalBS-TableAllowed-r18       BIT STRING (SIZE (maxNrofLCGs-r18))                                     OPTIONAL,    -- Need R

dsr-ConfigToAddModList-r18          SEQUENCE (SIZE (1..maxNrofLCGs-r18)) OF LCG-DSR-Config-r18              OPTIONAL,    -- Need N

dsr-ConfigToReleaseList-r18         SEQUENCE (SIZE (1..maxNrofLCGs-r18)) OF LCG-Id-r18                      OPTIONAL,    -- Need N

tar-Config-r18                      SetupRelease { TAR-Config-r18  }                                        OPTIONAL     -- Need M

]]

}

DataInactivityTimer ::=         ENUMERATED {s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60, s80, s100, s120, s150, s180}

MBS-RNTI-SpecificConfig-r17 ::=        SEQUENCE {

mbs-RNTI-SpecificConfigId-r17          MBS-RNTI-SpecificConfigId-r17,

groupCommon-RNTI-r17                   CHOICE {

g-RNTI                                 RNTI-Value,

g-CS-RNTI                              RNTI-Value

},

drx-ConfigPTM-r17                      SetupRelease { DRX-ConfigPTM-r17 }                          OPTIONAL,   -- Need M

harq-FeedbackEnablerMulticast-r17      ENUMERATED {dci-enabler, enabled}                           OPTIONAL,   -- Need S

harq-FeedbackOptionMulticast-r17       ENUMERATED {ack-nack, nack-only}                            OPTIONAL,   -- Cond HARQFeedback

pdsch-AggregationFactor-r17            ENUMERATED {n2, n4, n8}                                     OPTIONAL    -- Cond G-RNTI

}

MBS-RNTI-SpecificConfigId-r17 ::= INTEGER (0..maxG-RNTI-1-r17)

LCG-DSR-Config-r18 ::= SEQUENCE {

lcg-Id-r18                      LCG-Id-r18,

remainingTimeThreshold-r18      INTEGER (1..64),

...

}

LCG-Id-r18 ::= INTEGER (0..maxLCG-ID)

-- TAG-MAC-CELLGROUPCONFIG-STOP

-- ASN1STOP

MAC-CellGroupConfig field descriptions

additionalBS-TableAllowed Indicates whether a UE is allowed to utilize the refined buffer size levels, as specified in TS 38.321 [3], for a certain Logical Channel Group. The leftmost bit corresponds to LCG ID=0, second leftmost bit to LCG ID=1 and so on. The UE is allowed to utilize the refined buffer size levels for a Logical Channel Group only when the corresponding bit is set to 1.

allowCSI-SRS-Tx-MulticastDRX-Active Used to control the CSI/SRS transmission during MBS multicast DRX ActiveTime, see TS 38.321 [3].

csi-Mask If set to true, the UE limits CSI reports to the on-duration period of the DRX cycle, see TS 38.321 [3].

dataInactivityTimer Releases the RRC connection upon data inactivity as specified in clause 5.3.8.5 and in TS 38.321 [3]. Value s1 corresponds to 1 second, value s2 corresponds to 2 seconds, and so on.

drx-Config, drx-ConfigExt, drx-ConfigExt2 Used to configure DRX as specified in TS 38.321 [3]. Network only configures drx-ConfigExt or drx-ConfigExt2 when drx-Config is configured. Network does not configure drx-ConfigExt2 when reconfiguration with sync is contained for the cell group.

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

drx-ConfigSL Used to configure additional DRX parameters for the UE performing sidelink operation with resource allocation mode 1, as specified in TS 38.321 [3]. Network only configures this field if sl-ScheduledConfig is configured and drx-Config is configured.

drx-LastTransmissionUL If this field is present, the start of the drx-HARQ-RTT-TimerUL is after the last transmission within a bundle, see TS 38.321 [3].

dsr-ConfigToAddModList List of LCG-specific DSR configurations to add or modify.

dsr-ConfigToReleaseList List of LCG-specific DSR configurations to release.

g-RNTI-ConfigToAddModList List of G-RNTI configurations to add or modify. Up to 8 G-RNTIs can be configured in total in this release based on the UE capability.

g-RNTI-ConfigToReleaseList List of G-RNTI configurations to release.

g-CS-RNTI-ConfigToAddModList List of G-CS-RNTI configurations to add or modify. Up to 8 G-CS-RNTIs can be configured in total in this release based on the UE capability.

g-CS-RNTI-ConfigToReleaseList List of G-CS-RNTI configurations to release.

intraCG-Prioritization Used to enable HARQ process ID selection based on LCH-priority for one CG as specified in TS 38.321 [3].

lch-BasedPrioritization If this field is present, the corresponding MAC entity of the UE is configured with prioritization between overlapping grants and between scheduling request and overlapping grants based on LCH priority, see TS 38.321 [3]. The network does not configure lch-BasedPrioritization with enhancedSkipUplinkTxDynamic simultaneously nor lch-BasedPrioritization with enhancedSkipUplinkTxConfigured simultaneously.

posMG-Request Indicates whether UE is configured to send UL MAC CE for Positioning Measurement Gap Activation/Deactivation Request, as specified in TS 38.321 [3].

schedulingRequestID-BFR-SCell Indicates the scheduling request configuration applicable for BFR on SCell, as specified in TS 38.321 [3].

schedulingRequestID-BFR Indicates the scheduling request configuration (SchedulingRequestConfig) that the UE shall use upon detecting a beam failure on the detection resources configured in failureDetectionSet1 of a serving cell while beam failure is not detected on resources configured in failureDetectionSet2 of the same serving cell.

schedulingRequestID-BFR2 Indicates the scheduling request configuration (SchedulingRequestConfig) that the UE shall use upon detecting a beam failure on the detection resources configured in failureDetectionSet2 of a serving cell while beam failure is not detected on resources configured in failureDetectionSet1 of the same serving cell.

schedulingRequestID-LBT-SCell Indicates the scheduling request configuration applicable for consistent uplink LBT recovery on SCell, as specified in TS 38.321 [3].

schedulingRequestID-PosMG-Request Indicates the scheduling request configuration applicable for Positioning Measurement Gap Activation/Deactivation Request, as specified in TS 38.321 [3].

skipUplinkTxDynamic, enhancedSkipUplinkTxDynamic, enhancedSkipUplinkTxConfigured If set to true, the UE skips UL transmissions as described in TS 38.321 [3]. If the UE is configured with enhancedSkipUplinkTxDynamic or enhancedSkipUplinkTxConfigured with value true, REPETITION_NUMBER (as specified in TS 38.321 [3], clause 5.4.2.1) of the corresponding PUSCH transmission of the uplink grant shall be equal to 1. The network does not configure enhancedSkipUplinkTxDynamic or enhancedSkipUplinkTxConfigured with value true together with numberOfSlotsTBoMS-r17.

tag-Config The field is used to configure parameters for a time-alignment group. The field is not present if any DAPS bearer is configured.

usePreBSR If set to true, the MAC entity of the IAB-MT may use the Pre-emptive BSR, see TS 38.321 [3].

<<<<<<<<<<<<<<<<<<<< End of change >>>>>>>>>>>>>>>>>>>>