3GPP TSG-RAN WG2 Meeting #127                                                        R2-2406556

Maastricht, NL, 19th – 23rd Aug, 2024

Source:	CATT

Title:	Clarification on the Filtering of SL-RSRP for U2U Relay

Agenda Item:	7.9.3

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the details on the filtering parameter of SL-RSRP for U2U relay.

Discussion

In version 18.1.0 of TS38.331, regarding to the filtering parameter for U2U relay, the following was captured and separate filtering parameters are used for SL-RSRP and SD-RSRP.

In version 18.2.0 of TS38.331, sl-FilterCoefficientU2U was deleted and it was further clarified that for integrated discovery of U2U relay, filtering parameter sd-FilterCoefficientU2U used for SD-RSRP can also be used for SL-RSRP when performing U2U Relay Communication with integrated Discovery:

In fact, SL-RSRP is not only used when performing integrated discovery, but also used for other cases, e.g., U2U relay selection/reselection triggers, listed as below:

#### 5.8.17.3 Conditions for selection and reselection of NR sidelink U2U Relay UE ……. NOTE: For relay selection, U2U Remote UE uses SL-RSRP measurements for relay selection trigger evaluation when there is data transmission from peer U2U Remote UE to U2U Remote UE. For relay reselection, U2U Remote UE uses SL-RSRP measurements for relay reselection trigger evaluation when there is data transmission from U2U Relay UE to U2U Remote UE. And in both cases, it is left to UE implementation whether to use SL-RSRP or SD-RSRP for relay (re)selection trigger evaluation in case of no data transmission.

But it is unclear which filtering parameter can be used for SL-RSRP used for cases other than integrated discovery.

Observation 1：For U2U relay, it is unclear which filtering parameter can be used for SL-RSRP for cases other than integrated discovery.

Regarding to the issues mentioned in observation 1, there are three options:

- Option 1: use sd-FilterCoefficientU2U;

- Option 2: use sl-FilterCoefficientRSRP which is defined for U2N Remote UE;

- Option 3: introduce sl-FilterCoefficientU2U;

If a Remote UE can support both U2N and U2U, Option 2 can work. But if the Remote UE can only support U2U, Option 2 cannot work. Hence, Option 2 can be firstly excluded.

Regarding to Option 1 and Option 3, the spec impacts are as below:

If Option 1 is used, the corresponding modification can be as below:

sd-FilterCoefficientU2U Specifies L3 filter coefficient for SD-RSRP measurement results from L1 filter, and for SL-RSRP measurement result from L1 filter.

If Option 3 is used, the corresponding modification can be as below:

SL-RemoteUE-ConfigU2U information element -- ASN1START -- TAG-SL-REMOTEUE-CONFIGU2U-START SL-RemoteUE-ConfigU2U-r18::= SEQUENCE { sl-RSRP-ThreshU2U-r18 SL-RSRP-Range-r16 OPTIONAL, -- Need R sl-FilterCoefficientU2U-r18 FilterCoefficient OPTIONAL, -- Need R sl-HystMinU2U-r18 Hysteresis OPTIONAL, -- Cond SL-RSRP-ThreshU2U sd-RSRP-ThreshU2U-r18 SL-RSRP-Range-r16 OPTIONAL, -- Need R sd-FilterCoefficientU2U-r18 FilterCoefficient OPTIONAL, -- Need R sd-HystMinU2U-r18 Hysteresis OPTIONAL -- Cond SD-RSRP-ThreshU2U } -- TAG-SL-REMOTEUE-CONFIGU2U-STOP -- ASN1STOP SL-RemoteUE-ConfigU2U field descriptions. sl-RSRP-ThreshU2U Indicates the threshold of SL-RSRP for a U2U Remote UE to perform Relay UE selection/ reselection. The U2U remote UE applies the value of this field to evaluate AS layer conditions on direct PC5 link with the peer U2U Remote UE to trigger relay selection, and evaluate AS layer conditions on U2U relay link with U2U Relay UE to trigger relay reselection.. sl-FilterCoefficientU2U Specifies L3 filter coefficient for SL-RSRP measurement results from L1 filter except the integrated Discovery.. sd-RSRP-ThreshU2U ……. sd-FilterCoefficientU2U Specifies L3 filter coefficient for SD-RSRP measurement results from L1 filter, and for SL-RSRP measurement when performing U2U Relay Communication with integrated Discovery..

It is obvious that Option 3 will impact the ASN.1. Considering the chair’s guideline is that RRC ASN.1 changes should be drafted in BC way, hence, Option 1 is slightly preferred.

Proposal 1：For U2U relay, suggest RAN2 to confirm that it is unclear which filtering parameter is used for SL-RSRP for the cases other than integrated discovery.

Proposal 2:  If Proposal 1 is confirmed, suggest RAN2 to perform down-selection between the following two options:

- Option 1: Use sd-FilterCoefficientU2U;

- Option 2: Introduce new parameter sl-FilterCoefficientU2U.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1：For U2U relay, suggest RAN2 to confirm that it is unclear which filtering parameter is used for SL-RSRP for the cases other than integrated discovery.

Proposal 2:  If Proposal 1 is confirmed, suggest RAN2 to perform down-selection between the following two options:

- Option 1: Use sd-FilterCoefficientU2U;

- Option 2: Introduce new parameter sl-FilterCoefficientU2U.