3GPP TSG-RAN WG2 Meeting #127                                                        R2-2406558

Maastricht, NL, 19th – 23rd Aug, 2024

Source:	CATT

Title:	Discussion on SA2 and RAN3 LSs

Agenda Item:	8.7.1.1

Document for:	Discussion and Decision

Introduction

Before RAN2#127 meeting, two LSs[1][2] were sent to RAN2. In this contribution, RAN2 related questions in the SA2 LS on FS_XRM Ph2 (S2-2407351) [1] and RAN3 LS on UL PSI based PDU discarding in NR-DC (R3-243957) are discussed.

Discussion

RAN2 related questions in S2-2407351[1]

The questions in S2-2407351 are cited below, and the questions related to RAN2 are marked in yellow.

- Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension? - Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6? - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server? - Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

- Discussion on Question1

From our point of view, we understand this question is for DL only. Based on the current specification, the periodicity information can already be provided from CN to gNB by TSCAI. Then the gNB can use this information to assist some configuration, such as DRX configuration etc. It is worth mentioning that in the Uu interface, the gNB may use such information like traffic periodicity to facilitate semi-static configurations to the UE. In this sense, the current TSCAI can satisfy the gNB’s requirement. It happens that the periodicity may change sometimes. However, we also think reusing the TSCAI manner to transfer this information from CN to gNB is enough, as such periodicity change should still be in a semi-static manner, instead of a dynamic way that needs a per-packet indication via in-band signaling. To this end, we fail to see the needs of an indication via in-band signaling to indicate the dynamic change of traffic periodicity as asked by SA2.

Proposal 1: Reply to SA2 with the RAN2 understanding that there is no need for indicating periodicity via in-band signalling (i.e. in GTP-U) for the dynamic change of the periodicity. RAN2 understands that reuse of TSCAI remains feasible.

- Discussion on Question2

In the current TS38.331, the definition of the BAT in the field description of the burstArrivalTime is cited below:

UL-TrafficInfo field descriptions

burstArrivalTime Indicates the expected arrival time of the first packet of the Data Burst for the concerned QoS flow. If the UE provides both burstArrivalTime and jitterRange, burstArrivalTime is used as a reference time for the indicated jitter range. If burstArrivalTime is indicated as referenceTime, the indicated time in 10ns unit from the origin is refDays*86400*1000*100000 + refSeconds*1000*100000 + refMilliSeconds*100000 + refTenNanoSeconds. The refDays field specifies the sequential number of days (with day count starting at 0) from 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time). If burstArrivalTime is indicated as referenceSFN-AndSlot, it refers to the UL timing of the closest SFN and slot of the PCell with the indicated number.

jitterRange Indicates the maximum deviation of the arrival time of the first packet of a Data Burst compared to the time indicated with burstArrivalTime and the periodicity of the Data Bursts. lowerBound indicates the negative deviation while upperBound indicates the positive deviation. This field shall only be reported together with the burstArrivalTime or after the burstArrivalTime has been already reported. Value ms0 corresponds to 0 ms, value 0dot5 to 0.5 ms, value ms1 to 1 ms and so on. Value beyondMs7 indicates the jitter bound is higher than 7 ms. Value 0 ms means there is no Data Burst arrival time deviation from the indicated burstArrivalTime.

With the green marked part, the approximate arrival time of the subsequent data burst(s) can be deduced with burstArrivalTime, periodicity and jitterRange parameters. We think this approximate time information is already enough to gNB for scheduling, especially considering that such information via RRC signalling may mainly aim to facilitate scheduling of configured grant. Take one step back, in the LS, the information mentioned in the question is “the possible jitter between the NR-RAN and the Application Server”, which arouses doubt that the accuracy about the time to next burst. It is useful for gNB resource scheduling if this time to next burst information can be provided accurately and early enough, e.g. SA2 should assure that the time of the next burst information is accurate and shall be provided in the first packet of the previous burst. If this is not possible, perhaps reusing of data burst along with jitter information in RRC signaling is already sufficient. Till now, what we discussed is all for UL. For DL, the N2 signaling is used to transfer the burst time and jitter related information from CN to gNB to facilitate gNB’s resource scheduling. This is logically like UL. Hence, we propose:

Proposal 2: From RAN2 point of view, the time to next burst is useful for RAN resource scheduling for both UL and DL only if SA2 can provide it accurately and early enough.

RAN2 related questions in R3-243957 [2]

In R3-243957, the question is cited below:

RAN3 is currently discussing XR DC signaling enhancements for various bearer types.

In case of NR-DC split bearer, MN DU and SN DU congestion is local to each node. Based on congestion situation in MN DU and/or SN DU, each of the nodes independently send PSI-Based SDU Discard Activation/Deactivation MAC CE to UE for activation/deactivation of UL PSI based PDU set discarding of the split bearer. when UE receives MAC CE from both nodes or from a single node, it is not clear how UE will activate/deactivate UL PSI based PDU set discarding when split bearer is configured.

Thus, RAN3 kindly asks RAN2 to clarify the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE.

The PSI based SDU discard with UL split-bearer issue had been discussed in Rel-18. In the chairman note of RAN2-125bis meeting, the below agreed is recorded:

PSI based SDU discard with UL split bearers

R2-2402387    Issues related to PSI-based discard   Qualcomm Incorporated      discussion     Rel-18     NR_XR_enh-Core

Proposal 2.     A UL split-bearer deactivates PSI-based SDU discard once it has received a MAC CE deactivating the discard from either MN or SN.

-     Ericsson doesn’t think this makes sense.  Lenovo thinks it makes sense.  ZTE indicates that we agreed to not optimize this.  Nokia thinks that we would have a problem.  Qualcomm thinks that if we don’t specify anything there will be ambiguity.

=>  No further optimizations will be supported in Rel-18 for UL split-bearer

=>  Noted

R2-2402840    PSI-based discarding on split bearers       Nokia, Nokia Shanghai Bell  discussion     Rel-18     NR_XR_enh-Core

=>  Noted

Proposal 1: On a split bearer configured over two MAC entities, PSI-based discarding is not activated if the MAC CE to activate it has been received over one MAC entity only.

Proposal 2: on a split bearer configured over two MAC entities, if PSI-based discarding is only activated by one MAC entity, the UE considers alternative values of ul-DataSplitThreshold to be in force: 0 if activated by the primary leg, and infinity if activated by the non-primary leg.

Proposal 3: on a split bearer configured over two MAC entities, the UE applies the RRC-configured value of ul-DataSplitThreshold when PSI-based discarding is activated by both the MAC entities, or by neither one of them.

Proposal 4: RAN2 agree on the PDCP text proposal in Annex.

In other words, without further optimization in Rel-18, the current procedure in TS 38.321 is that either MN or SN receives the PSI-based SDU discard MAC CE, the related MAC entity should notify the PDCP entity to perform the uplink PSI based discard.

Even if congestion in MN and SN is locally detected, the MN can still identify that there is congestion happening at the SN which activates this PSI-based SDU discard mechanism for a split bearer by implementation (e.g. due to a number of unexpected packet loss at the receiving PDCP entity), since the anchor PDCP of a split bearer is located at one of the MN and SN anyway. Then the gNB at the corresponding MN/SN node can de-configure the split bearer to avoid this issue. This is logically similar to the packet loss due to the data split case for a split bearer, which is also left to gNB implementation during Rel-15 discussion.

Considering the above situation, for Rel-19 XR, there is no strong motivation to further handle this issue from RAN2 point of view. That is to say, the issue raised can be solved according to gNB implementation without further clarification on UE aspect. Hence, we propose that:

Proposal 3: RAN2 reaches common understanding that PSI-based SDU discard mechanism is activated/deactivated for a split bearer, when the UE receives the PSI-Based SDU Discard Activation/Deactivation MAC CE sent from either MN or SN. This is the intended UE behaviour, and no enhancement is needed in Rel-19.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Reply to SA2 with the RAN2 understanding that there is no need for indicating periodicity via in-band signalling (i.e. in GTP-U) for the dynamic change of the periodicity. RAN2 understands that reuse of TSCAI remains feasible.

Proposal 2: From RAN2 point of view, the time to next burst is useful for RAN resource scheduling for both UL and DL only if SA2 can provide it accurately and early enough.

Proposal 3: RAN2 reaches common understanding that PSI-based SDU discard mechanism is activated/deactivated for a split bearer, when the UE receives the PSI-Based SDU Discard Activation/Deactivation MAC CE sent from either MN or SN. This is the intended UE behaviour, and no enhancement is needed in Rel-19.

Reference

- R2-2406241 LS on FS_XRM Ph2 (S2-2407351; contact: vivo)

- R2-2406221 LS on UL PSI based PDU discarding in NR-DC (R3-243957; contact: Qualcomm)

- R2-2405779 Reply LS to SA2 on FS_XRM PH2