3GPP TSG-RAN WG2 Meeting #127                                                        R2-2406559

Maastricht, NL, 19th – 23rd Aug, 2024

Source:	CATT

Title:	Discussion on Multi-Modality

Agenda Item:	8.7.2

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting, regarding to Multi-Modality, it was agreed that:

- Support Multi-Modality awareness in RAN in Rel-19 for UL and DL. - Ask SA2 whether/what information could be provided to RAN - Clarify in the LS how RAN2 considers to use this information, e.g. coordinated handling of multi-modal flows. - Can consider while drafting the LS to indicate the potential benefits

After discussion, LS [1] was sent to SA2. But unfortunately, SA2 did not reply to RAN2 in TSGS2#165 meeting. But considering RAN2 already agreed to support Multi-Modality awareness in RAN in Rel-19 for UL and DL, we will further discuss this topic in this contribution. In this contribution, the following issues will be discussed:

- What multi-modal information is needed in RAN?

- How the multi-modal information is provided to NG-RAN and UE?

- How to ensure the synchronized/coordinated transmission in RAN?

Discussion

What multi-modal information is needed in RAN?

In TS23.501 [2] , Multi-modal Service ID was introduced to indicate the multi-modal information. The corresponding description is as below:

The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, for each data flow that belongs to the multi-modal service, a Multi-modal Service ID, the service requirements and the QoS monitoring requirements: - The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and to apply appropriate QoS policies for the data flows that are part of a specific multi-modal application. - The AF may provide QoS monitoring requirements for data flows associated to a multi-modal service to the PCF . The PCF generates the authorized QoS Monitoring policy for each data flow.

If synchronization/coordination transmission is needed for different QoS flows belonging to one multi-modal service, RAN should also aware this multi-modal association amongst these QoS flows. MMSID or a new ID similar to MMSID can be used for upper layer to indicate the multi-modal association to RAN.

Observation 1: MMSID can be used to indicate the multi-modal association of different QoS flows belonging to one multi-modal service.

Furthermore, in TS22.261 [3], it also captured the typical synchronization thresholds for immersive multi-modality VR. The corresponding table is as below:

Table 6.43.1-1: Typical synchronization thresholds for immersive multi-modality VR applications Media components; synchronization threshold (note 1). audio-tactile; audio delay: 50 ms; tactile delay: 25 ms. visual-tactile; visual delay: 15 ms; tactile delay: 50 ms. NOTE 1: for each media component, “delay” refers to the case where that media component is delayed compared to the other..

Based on the above table, it is obvious that different QoS flows of one multi-modal service may have synchronization threshold requirement. Hence, in order to provide better synchronization/coordination transmission in RAN, upper layer should also provide the synchronization threshold to RAN.

Observation 2: The synchronization threshold amongst different QoS flows is also needed based on the requirement listed in TS22.261.

Since SA2 has not replied to RAN2, if RAN2 want to make further progress on Multi-Modality, some working assumption can be made, e.g., RAN2 assume upper layer will provide MMSI and synchronized threshold to AS. But on the working assumption, RAN2 can further discuss the RAN impacts.

Proposal 1: Before SA2 reply, RAN2 can assume upper layer will provide MMSID and synchronized threshold with the granularity of per QoS flow to AS.

How the multi-modal information is provided to RAN?

For DL, in our understanding, the multi-modal information is provided based on QoS flow. Hence, it is obvious that the multi-modal information can be provided together with QoS profile.

For UL, there are different understandings based on contributions submitted in the last RAN2 meeting. There are mainly two alternatives:

- Alt 1: UE can aware the multi-modal information from its upper layer, and it can report it to RAN;

- Alt 2: Same as DL, RAN get the multi-modal information from CN.

In our understanding, RAN can also aware the multi-modal information of UL QoS flow from CN. Hence, it is feasible for CN to provide the multi-modal information to RAN for UL.

Proposal 2: RAN2 assumes that at least CN provides the multi-modal information to RAN together with the QoS profile for both UL and DL. Whether UE reporting is necessary can be further discussed.

How to ensure the synchronized/coordinated transmission?

For DL, how to ensure the synchronized/coordinated transmission can be left to gNB implementation. Hence, the enhancements for synchronized/coordinated transmission mainly focus on UL.

Proposal 3: The enhancements for synchronized/coordinated transmission mainly focus on UL.

For UL, in order to ensure the synchronized/coordinated transmission, the following enhancements can be considered:

- DRB association acquisition from RAN

- Coordinated scheduling/LCP

- Coordinated admission control during mobility

- Multiple DRX configurations

For bullet 1), based on Proposal 2, assuming CN provides the multi-modal information to RAN for UL, when gNB provide the radio bearer configuration to UE via RRCReconfiguration, it should also provide the association indication amongst DRBs to help the UE perform UL synchronized/coordinated transmission.

Proposal 4: RAN should indicate the DRB associations to UE through RRCReconfiguration.

For bullet 2), to support synchronized/coordinated transmission of the different DRBs corresponding to one multi-modal service, scheduling or LCP procedure can be enhanced, e.g., when the difference of transmission delay between the corresponding DRBs is larger than a configured threshold, the one with larger latency can be scheduled or transmitted with higher priority.

Proposal 5: Scheduling/LCP enhancements can be considered for multi-modality.

For bullet 3), to ensure the synchronized/coordinated transmission of the DRBs with multi-modal association, admission control enhancement can be considered, that is the associated objects should be admitted simultaneously or rejected simultaneously. In case of mobility, in order to achieve the same admission control results, target gNB should be aware of the association between different DRBs.

Proposal 6: Admission control enhancement can be considered for multi-modality.

For bullet 4), according to the current MAC specification, one serving cell can have only one DRX configuration. For multi-modality, it is possible to introduce multiple DRX configurations in one serving cell. It is benefit for reducing the transmission latency.

Proposal 7: Multiple DRX configurations for one serving cell can be considered for multi-modality.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Before SA2 reply, RAN2 can assume upper layer will provide MMSID and synchronized threshold with the granularity of per QoS flow to AS.

Proposal 2: RAN2 assumes that at least CN provides the multi-modal information to RAN together with the QoS profile for both UL and DL. Whether UE reporting is necessary can be further discussed.

Proposal 3: The enhancements for synchronized/coordinated transmission mainly focus on UL.

Proposal 4: RAN should indicate the DRB associations to UE through RRCReconfiguration.

Proposal 5: Scheduling/LCP enhancements can be considered for multi-modality.

Proposal 6: Admission control enhancement can be considered for multi-modality.

Proposal 7: Multiple DRX configurations for one serving cell can be considered for multi-modality.

Reference

- R2-2405782  LS on multi-modality awareness at RAN	TSG RAN WG2 LS out	Rel-19

- TS23.501  System architecture for the 5G System (5GS); Stage 2 (Release 18) V18.4.0

- TS22.261  Service requirements for the 5G system; Stage 1(Release 19)  V19.5.0