3GPP TSG-RAN WG2 Meeting #127                                                                           R2-2406560

Maastricht, NL, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Consideration on XR-specific scheduling enhancement

Agenda Item:	8.7.4

Document for:	Discussion and Decision

Introduction

In this contribution, we will further discuss the following aspects related to the scheduling enhancement:

- LCP enhancement for the delay-critical data;

- DSR related enhancement.

Discussion

LCP enhancement

According to RAN2 progress, the LCP enhancement with consideration of delay-critical data includes the following two alternatives:

    - Alternative 1: Enhance LCH prioritization.

    - Alternative 2: Enhance LCP restrictions/LCH selection.

To identify the detailed solutions and impacts, we give analysis of these two alternatives.

Enhance LCH prioritization

One aspect to prioritize the transmission of delay-critical data is to adjust the priority of the data with low remaining time, so that delay-critical data can be transmitted with higher priority.

For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to within the LCH or for the whole LCH LCP prioritization within a logical channel will not be considered in RAN2 discussions FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

Based on above agreements in RAN2#126 meeting, it has agreed that the priority of LCH will be adjusted based on configuration and applied in the LCP procedure. More details are discussed here:

How to configure the additional priority?

Generally, there are two options to configure the additional priority:

- Option 1: Pre-configuration;

- Option 2: Configured by RRC signalling;

For Option 1, the priority to LCH with delay critical data could be preconfigured as a fix value, such as the highest priority. This option is not flexible. Furthermore, in some cases, it is not reasonable. For example, if there is important MAC CE or other data which is more important than the delay critical data, it is not reasonable to always prioritize the delay critical data. Hence, it is preferable to let it configurable as in Option 2. The network can configure the additional priority for the LCH based on its implementation.

Proposal 1: The additional priority of the LCH with delay critical data could be configured by network via dedicated RRC signaling.

When to apply the additional priority for the LCH?

With the introduction of the additional priority, there will be two priority values for one LCH, it should further discuss when to apply the legacy priority and when to apply the additional priority.

Obviously, when there is delay critical UL data, the additional priority should be applied for the LCH; otherwise, the legacy priority should be applied.  Hence, the key issue becomes how to judge there is delay critical UL data for an LCH. In Rel-18 XR, UL delay critical PDCP/RLC data volumes were introduced in TS38.323 and TS38.321. It had better follow it in Rel-19 to define UL delay critical data. That is if the delay critical PDCP/RLC data volumes is not empty, the additional priority can be used for the LCH.

Proposal 2：If the amount of the delay critical PDCP/RLC data volumes is not zero, the LCH should apply the additional priority.

Whether the priority of LCH can be changed during one LCP procedure?

According to legacy procedure as below, the logical channels selected for the UL grant are allocated resources in a decreasing priority order with Bj >0 (step 1), and Bj will be updated (step 2), then logical channels selected are allocated remaining resources in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first (step 3).

5.4.3.1.3 Allocation of resources Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover. The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. NOTE 1: The value of Bj can be negative.

If two priorities are configured for one LCH, a new issue rasied. That is, if the resource allocated in step 1 can accommodate all the UL delay critical data, when performs resource allocation in step 3, whether the legacy priority or additional priority can be used since there is no delay cirtical data. RAN2 needs to discuss whether the LCH priority will fallback to the legacy value to avoid allocating remaining resource to logical channel with low priority in real.

Proposal 3: RAN2 to discuss whether the priority fallback caused by all the UL delay critical data included in the initial resource allocation step will be applied in LCP procedure.

Enhance LCP restrictions/LCH selection

In last meeting, the following agreements are achieved for LCP restriction and LCH selection:

We try to avoid RAN1 impacts. RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial The solutions should not disallow non-delay critical data from using an UL grant.

Based on above agreement, a semi-static way could be used for LCP restriction and selection, which needs the NW configure a semi-static resource, such as configured grant, to transmit the delay-critical data. To support this way, RRC also needs to additionally control the LCP procedure by configuring mapping restrictions for the XR logical channel, e.g., set the allowed configured grant(s) and delay threshold for transmission. Then, UE could transmit the delay critical data for the logical channel on the allowed CG resource if the delay budget in the logical channel is lower than the configured threshold. Furthermore, the procedure should also allow the other logical channel with non-delay critical data to use the remaining UL grant resource.

Observation 1: For LCP restrictions based approaches in a semi-static way, the network needs to configure the CG resource, the mapping restrictions and delay threshold for the XR logical channel to UE.

Considering that the above a semi-static way should also allow non-delay critical data using the UL grant, which means that the following LCP prioritization procedure, i.e., resource allocation procedure, will consider both the data from XR traffic and other LCH, so this semi-static way will have the same function as above enhance LCH prioritization procedure. Hence, it is not necessary to introduce this type of enhancement, the transmission of delay critical data could be guaranteed by the legacy CG configuration and above enhanced LCP prioritization procedure.

Proposal 4: It is unnecessary to introduce the LCP restrictions/LCH selection in Rel-19.

DSR related enhancement

Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. FFS whether DSR triggering is impacted FFS whether PDU set importance needs to be included

In last meeting, it was agreed that multiple remaining time and the corresponding data buffer volume will be included in the enhanced DSR MAC CE. For the DSR triggering issue, the legacy triggering threshold remainingTimeThreshold already indicates that a DSR should be reported to network for UL grant, which means that the legacy triggering threshold in time is sufficient for network scheduling resource for UE, so it is not necessary to introduce new threshold to fast the scheduling procedure.

Proposal 5: Reuse the legacy DSR triggers, and DSR trigger enhancement is not needed.

The legacy DSR MAC CE is as follows, it is not easy to extend legacy DSR to include buffer size information for different remaining time since the number of remaining time of each LCG may be various, it is suggested to introduce a new MAC CE to enhance DSR. For the remaining time, there are 16 candidates for legacy discardTimer in ASN.1 coding, so the maximum number of remaining time range is 16 for enhanced DSR. It is suggested to shorten the number of remaining time to 2 or 4, the finer granularity for remaining time may be not needed for scheduling.

TS 38.321 figure 6.1.3.72-1: DSR MAC CE

Proposal 6: Introduce a new MAC CE for enhanced DSR, the number of candidate remaining time for one LCG can be 2 or 4.

For enhanced DSR, if the PSI is reported to network, the network may configure the UL grant based on different PSI information from UEs, however, UE will not perform any different handling in MAC scheduling procedure for different PSI, so the benefit of this method is limited and it is not necessary to introduce DSR with PHI information.

Proposal 7: The PSI is not included in enhanced DSR.

Based on the remaining delay, NW can decide the size of the grant to be scheduled. However, it is FFS whether the non-delay critical data or other information should be included within the DSR MAC CE.

Currently, the trigger of the DSR MAC CE and the trigger of the BSR MAC CE is separate procedure. That is, even receive the DSR MAC CE, NW is still unclear the real buffer status of UE. NW can only schedule the grant based on the volume of the delay critical data. But in case there are available data which is not delay critical but is in high priority, it still cannot ensure the delay critical data can be transmitted in the grant.

Observation 2: NW cannot schedule grant with proper size since only the data volume of the delay critical data is reported within the DSR MAC CE.

To address the issue, one straightforward way is to also report the non-delay critical data’s volume in the DSR MAC CE. However, report all data volume is signaling overhead, and NW cannot decide the priority of the data, hence NW still cannot decide how much resource is scheduled in case of the congestion. Based on this, it is proposed that UE include the available data volume of data which is in higher priority in the DSR MAC CE.

Proposal 8: The non-delay critical data volume which with higher priority can be reported in the DSR MAC CE.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: The additional priority of the LCH with delay critical data could be configured by network via dedicated RRC signaling.

Proposal 2：If the amount of the delay critical PDCP/RLC data volumes is not zero, the LCH should apply the additional priority.

Proposal 3: RAN2 to discuss whether the priority fallback caused by all the UL delay critical data included in the initial resource allocation step will be applied in LCP procedure.

Observation 1: For LCP restrictions based approaches in a semi-static way, the network needs to configure the CG resource, the mapping restrictions and delay threshold for the XR logical channel to UE.

Proposal 4: It is unnecessary to introduce the LCP restrictions/LCH selection in Rel-19.

Proposal 5: Reuse the legacy DSR triggers, and DSR trigger enhancement is not needed.

Proposal 6: Introduce a new MAC CE for enhanced DSR, the number of candidate remaining time for one LCG can be 2 or 4.

Proposal 7: The PSI is not included in enhanced DSR.

Observation 2: NW cannot schedule grant with proper size since only the data volume of the delay critical data is reported within the DSR MAC CE.

Proposal 8: The non-delay critical data volume which with higher priority can be reported in the DSR MAC CE.

Reference

[1] RAN2#126 meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei.