3GPP TSG-RAN WG2 Meeting #127                                                        R2-2406561

Maastricht, NL, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Consideration on XR-specific RLC enhancement

Agenda Item:	8.7.5

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting, some progress has been made for XR-specific RLC enhancement. In this contribution, we will give our views on timely RLC retransmissions and avoiding unnecessary retransmission.

Discussion

Timely RLC retransmissions for XR

In RAN2#126 meeting, the agreements on timely retransmissions on RLC layer were as follows, three options were proposed for further study.

- To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism - Impact on capacity should be considered - RAN2 focuses on the enhancements for UL traffic

On autonomous retransmission

In this option, RLC retransmission is not performed based on RLC status report but depends on blind retransmission. During this procedure, the RLC will retransmit the RLC SDU which may be already successfully received at Rx side, which will result resource waste and impact on system capacity. Besides, in congested scenario, this behavior will also deteriorate the congestion.

Observation 1: RLC autonomous retransmission will reduce the radio resource usage, have impact to system capacity and deteriorate the congestion.

If still going to this way, the specific condition to trigger the blind retransmission, such as based on the remaining PSDB, should also be defined. Considering the channel quality is variable and the XR delay requirement is stringent, it is not easy to find a suitable value to determine when is the best opportunity to perform the autonomous retransmission and keep balance between the XR QoS and other benefits, such as the capacity and radio resource usage.

Observation 2: It is difficult to determine a suitable value of the conditions to trigger the blind retransmission.

Based on above analysis, it is proposed:

Proposal 1: Not to support the RLC autonomous retransmission for R19 XR.

Retransmission based on enhanced status report

Since RAN2 agreed to focus on the enhancements for UL traffic, so the status report here is transmitted from network to UE. For the needed cases, network could send the status report based on several new methods. For example, the network could evaluate the time elapsing since receiving new transmission, the channel quality status, the congestion status to determine when to send the status report to accelerate the RLC retransmission, all of them could be considered as network implementation, thus it is not necessary to specify the enhanced method to trigger status report at network side.

Proposal 2: Enhanced status report for UL can be left to gNB implementation, no spec impact.

Retransmission based on enhanced polling

5.3.3.2 Transmission of a AMD PDU Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below. Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); - include a poll in the AMD PDU as described below. NOTE: Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation. <--skip--> Upon submission of an AMD PDU including a poll to lower layer, the transmitting side of an AM RLC entity shall: - set POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer; - if t-PollRetransmit is not running: - start t-PollRetransmit. - else: - restart t-PollRetransmit. 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

Based on above procedure for polling, it can be seen that the number of PDU_WITHOUT_POLL, BYTE_WITHOUT_POLL or the expiry of t-PollRetransmit could be used to trigger a poll in an AMD PDU. For UL XR traffic, the value associated to above three parameters could be configure to be short one by network, which could trigger PDCP status report more quickly. The short value for pollPDU and pollByte absolutely have the same function as a timer, thus is not necessary to introduce a new timer to trigger the polling. Obviously, the bits used by polling and the following status report is acceptable, the polling by new small value will have very limited impact on capacity. Generally, UE could apply the new value once configured or based on some new condition, for example, the remaining PSDB is under a threshold or triggered by PDCP layer.

Proposal 3: For UL XR traffic, the network can configure a new set of shorter value for pollPDU, pollByte, t-PollRetransmit to trigger the polling for timely RLC retransmission. When to apply the shorter value can be further studied.

Avoid unnecessary retransmission

In RAN2#126 meeting, the agreement on avoiding unnecessary retransmission on RLC layer is as follows, Tx or Rx initiated approaches will be down-selected in next step.

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned

Tx initiated approach VS Rx initiated approach

For Tx initiated approach, Tx needs to discard the obsolete RLC SDU or segment based on PDCP indication or RLC enhancement. Then, the SN of discarded RLC SDU or segment should be transferred to Rx side. Both Tx side and Rx side need to update its state variables. The status report from Rx side could be the legacy one, for Tx UE, it will ignore the ACK/NACK information for the discarded RLC SDU or segment.

For Rx initiated approach, it is better for Rx UE to discard the RLC SDU or segment based on the time information associated to the PSDB, however, the remaining PSDB is dynamic and maintained at Tx side, this time information is difficult to be received at Rx side. Instead, a timer, such as a shorter t-Reassembly/t-Reordering or a new timer, could not compute the right occasions to discard the RLC SDU or segment. The Rx side also needs to notice the Tx side the discarded RLC SDU or segment to avoid the unnecessary transmission. For the notice on the discarded RLC SDU or segment, generally, the proposed approach, such as fake ACK to a RLC SDU in status report is not acceptable, the reason is, it gives the wrong information to Tx UE RLC and PDCP layer, the retransmission at high layer will be not executed, which is not desired. Hence an enhanced status report or a new report should be applied to indicate the discarded RLC SDU or segment. Besides, both Tx side and Rx side need to update its state variables for the discarded RLC SDU or segment.

Hence, based on above analysis, the impact from Tx initiated approach and Rx initiated approach is almost same. Both need to update the status variable at Rx/Tx side, a new report or enhanced status report to indicate the discarded RLC SDU will be introduced, which both have impact to peer UE. In our view, the important reason is, the Rx initiated approach could not find an accurate occasion to discard the RLC SDU or segment, so Tx initiated approach is preferable.

Proposal 4: It is preferable to apply Tx initiated approach for unnecessary retransmission.

Details of Tx initiated approach

For avoiding the unnecessary retransmission, the following cases are identified:

- Case 1: The PDCP layer indicate the RLC layer to discard the PDCP PDU, but the RLC SDU or part of the RLC SDU have already been delivered to lower layer;

In this case, since the RLC SDU is already considered as invalid, the retransmission is useless and wastes the resource. Hence, the retransmission of these RLC SDU or the RLC SDU segments can be avoided.

- Case 2: For RLC SDU which is not indicated by the PDCP layer to be discarded, the remaining time of the RLC SDU/RLC SDU segments is, not enough to perform the next retransmission;

For this case, it may happen in case the PDCP discard timer is still running, but the remaining time of the RLC SDU or the RLC SDU segments is not available for next retransmission.

Upon all above cases, it is beneficial for the RLC Tx side to discard the out of data RLC SDUs so as to avoiding the unnecessary retransmission.

Proposal 5: Support the Tx side to avoid the unnecessary retransmission for the following cases:

- Case 1: The PDCP layer indicate the RLC layer to discard the PDCP PDU, but the RLC SDU or part of the RLC SDU have already been delivered to lower layer;

- Case 2: For RLC SDU for which is not indicated by the PDCP layer it is discarded, but RLC layer determine the remaining time of the RLC SDU is not enough for the next retransmission.

For Tx side to avoid the unnecessary retransmission, there are different identified use cases as we highlight above. For case 1, this can be triggered by the PDCP layer discard indication. As for case 2, the determination of the discard is in RLC. However, currently, there is not any mechanism in the RLC layer to decide the remaining time of the RLC SDU is not available for retransmission. From our perspective, this can be solved by introducing timer in the RLC layer, or reusing the PDCP layer discard timer and comparing the remaining time of the PDCP layer discard timer and the estimated retransmission time. Consequently, the RLC layer should notice the PDCP layer to discard the PDCP PDU.

Proposal 6: PDCP indication is used to indicate RLC to discard the PDCP PDU, for which RLC SDU or part of the RLC SDU have already been delivered to lower layer.

Proposal 7：A new timer in RLC layer is introduced to determine when to discard the obsolete RLC SDU or segment.

Another issue is, after the Tx side discards the RLC SDU or segment, there is some SN gap and the Rx side cannot aware of the gap. RAN2 already agreed that the transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs. Generally, this information is in control plane scope, so RLC control PDU could be used to report the obsolete RLC SDU, the details could be discussed in stage.

Proposal 8: A RLC Control PDU could be used to inform the obsolete RLC SDU to Rx side.

Correspondingly, Tx side needs to update its state variables based on the discarded RLC SDU or segment. Generally, the updating will have impact to TX_Next_Ack and POLL_SN.

For Rx side, once it received the obsolete RLC SDU report from Tx side, it should also discard the corresponding RLC SDU or segment. The state variables in Rx side should also be updated. Generally, the updating will have impact to RX_Next, RX_Highest_Status and RX_Next_Highest.

Proposal 9: Tx/Rx need to update its state variables based on the discarded RLC SDU or segment, the details could be discussed in stage3.

Conclusion

According to the analysis in section 2, it is proposed:

Observation 1: RLC autonomous retransmission will reduce the radio resource usage, have impact to system capacity and deteriorate the congestion.

Observation 2: It is difficult to determine a suitable value of the conditions to trigger the blind retransmission.

Proposal 1: Not to support the RLC autonomous retransmission for R19 XR.

Proposal 2: Enhanced status report for UL can be left to gNB implementation, no spec impact.

Proposal 3: For UL XR traffic, the network can configure a new set of shorter value for pollPDU, pollByte, t-PollRetransmit to trigger the polling for timely RLC retransmission. When to apply the shorter value can be further studied.

Proposal 4: It is preferable to apply Tx initiated approach for unnecessary retransmission.

Proposal 5: Support the Tx side to avoid the unnecessary retransmission for the following cases:

- Case 1: The PDCP layer indicate the RLC layer to discard the PDCP PDU, but the RLC SDU or part of the RLC SDU have already been delivered to lower layer;

- Case 2: For RLC SDU for which is not indicated by the PDCP layer it is discarded, but RLC layer determine the remaining time of the RLC SDU is not enough for the next retransmission.

Proposal 6: PDCP indication is used to indicate RLC to discard the PDCP PDU, for which RLC SDU or part of the RLC SDU have already been delivered to lower layer.

Proposal 7：A new timer in RLC layer is introduced to determine when to discard the obsolete RLC SDU or segment.

Proposal 8: A RLC Control PDU could be used to inform the obsolete RLC SDU to Rx side.

Proposal 9: Tx/Rx need to update its state variables based on the discarded RLC SDU or segment, the details could be discussed in stage3.

Reference

[1] RAN2#126 meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei.