3GPP TSG-RAN WG2 Meeting #127                                                        R2-2406562

Maastricht, NL, 19th – 23rd Aug, 2024

Source:	CATT

Title:	Discussion on multi-hop discovery and (re)selection

Agenda Item:	8.13.2

Document for:	Discussion and Decision

Introduction

In RAN#104 meeting, the WID of Rel-19 NR sidelink multi-hop relay was approved[1]. The objectives are as below:

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

- Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE):

First Priority:

- Intra-gNB multi-hop indirect to direct path switching using existing framework

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

Second Priority in order of importance:

- Intra-gNB direct to multi-hop indirect path switching

- Intra-gNB single-hop indirect to multi-hop indirect path switching

The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In the current contribution. Three parts are discussed:

- The terminology for Rel-19 multi-hop U2N relay;

- Relay discovery related issues for Rel-19 multi-hop U2N relay;

- Relay (re)selection related issues for Rel-19 multi-hop U2N relay.

Discussion

Discussion on the terminology for multi-hop

In Rel-17 U2N relay, there are one U2N remote UE and one U2N relay UE to achieve the U2N relay function. For Rel-19 multi-hop U2N relay case, besides the U2N remote UE and U2N relay UE, one newly added relay UE was introduced as shown in the below figure. For the sake of clear reference on different Relay UEs, it is better to first align the terminologies that refer to different Relay UEs to avoid any misalignment in future RAN2 discussions.

Figure 1. The change from Rel-17 U2N relay to Rel-19 two hops U2N relay

The terminologies used in RAN WID and SA2 TR are as follows:

- In the WID of Rel-19 NR sidelink multi-hop relay, it records that the work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB).In this sense, the newly added relay UE is names as first relay UE, and the legacy U2N relay UE (which relay is closest to gNB) is named as last relay UE[1].

- In Figure 6.1.2.1.1-1 of SA2’s TR, the newly added UE is named as intermediate U2N relay UE, and the legacy U2N relay UE (which relay is closest to gNB) is name as U2N relay UE[2].

Based on the understanding of the approved WID, the newly added relay UE should be U2N relay UE, since the remote UE’s destination is gNB. Next, we further discuss the below question:

- For multi-hop U2N relay, whether the newly added relay UE’s name should be distinguish with the legacy U2N relay UE(which relay is closest to gNB)?

The answer to the above question is unambiguous, since the legacy U2N relay’s interface is different to the newly added relay UE’s interface, they should be distinguished. For one additional hop relay case, there is no big difference between RAN2’s first relay UE and SA2’s intermediate U2N relay UE. For two additional hops relay case, RAN2 uses the below terminology (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). But SA2 uses the below terminology (i.e., 5G ProSe Remote UE -> Intermediate U2N Relay -> Intermediate U2N Relay ->5G ProSe U2N Relay -> 5GS). The difference is whether to further distinguish the newly added relay UE. If we just only want to distinguish the newly added relay UE and the legacy U2N relay UE, the RAN2’s terminology seems not easy to extend. Hence, we prefer RAN2 uses the below terminology:

Proposal 1: RAN2 uses the below terminology (i.e., remote UE -> intermediate relay UE -> intermediate relay UE ->last relay UE -> gNB) in Rel-19 multi-hop L2 U2N relay.

Discussion on multi-hop U2N relay discovery

- Discovery model

About the multi-hop U2N relay discovery model, there are below conclusions in SA2’s TR [2]:

-	Standalone discovery & link setup/management is considered and both Model A and Model B are supported.

-	When Model A discovery is performed, the Remote UE may choose the Intermediate Relay based on the announcement message sent by the Intermediate Relay.

-	When Model B discovery is performed, the Remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay. To perform link management, the DCR message is unicasted between Relays according to the path information included in the message. The path information is an (ordered) list of User Info ID of Relays in the selected path. The Remote UE sends the selected path information to the Intermediate UE-to-Network Relay for communication setup.

Based on the above conclusions, it is obvious that SA2 already agreed that standalone discovery and link setup is recorded and remote UE is responsible for the intermediate relay selection. For further discussion, RAN2 should consider both Model A discovery and Model B discovery.

Proposal 2:  For Rel-19 multi-hop L2 U2N relay, RAN2 assumes that further discussions on AS impact on Relay discovery should take into account both Model A and Model B discovery models.

- AS conditions for multi-hop U2N relay discovery

Since in multi-hop U2N relay, the remote UE may need to discover a relay UE which has contacted (or is to contact) another UE, it seems reasonable to discuss the relay discovery for multi-hop U2N relay with an analogy to Rel-18 U2U relay design. In Rel-18 U2U relay, the below AS condition(s) for announcing/forwarding/responding discovery message are cited below for model A and model B:

RAN2#122:

For Model A discovery, the relay UE should only announce the neighbour UEs for which the SD-RSRP/SL-RSRP between the relay UE and the neighbour UE is above a configured threshold in a discovery announcement message. LS is sent to SA2.

For Model A discovery, upon discovery message reception, remote UE considers a relay UE as a candidate relay UE if the SD-RSRP towards the relay UE is above a configured threshold.

RAN2#123:

For Model B, the relay UE forwards the solicitation message only if the PC5 RSRP between the relay UE and the source remote UE is above a threshold.

RAN2#121bis:

For Model B discovery, after receiving a discovery message from a relay UE, a target remote UE transmits the discovery response message only if the PC5 RSRP between the target remote UE and the relay UE is above a configured threshold.  FFS if there is separate impact for this agreement from the relay selection functionality.

RAN2#123:

For Model B, no AS criterion is needed for the relay UE to forward the response message to the source Remote UE.

RAN2#122:

For Model B discovery, upon discovery response messages reception, the source remote UE considers a relay UE as a candidate relay UE if the SD-RSRP towards the relay UE is above a configured threshold.

For Rel-19 multi-hop U2N relay discovery model A, there is no need to consider the AS condition for announcing discovery message. Since in the LS from SA2 to SA3[4], it recorded that the intermediate U2N relay had already been authenticated and established a connection to the network before serving the remote UE (for discovery or connection). Hence, we propose:

Proposal 3: For multi-hop relay discovery Model A, there is no AS channel condition should be considered for intermediate relay UE when announcing the discovery message.

Proposal 4: For multi-hop relay discovery Model A, upon discovery message reception, remote UE considers an intermediate relay UE as a candidate relay UE if the SD-RSRP towards the intermediate relay UE is above a configured threshold (similar to Rel-18 U2U relay discovery).

For Rel-19 multi-hop U2N relay discovery model B, referring to Rel-18 U2U relay, the similar AS condition should be considered:

Proposal 5: For multi-hop relay discovery Model B, the intermediate relay UE forwards the solicitation message only if the PC5 RSRP between the intermediate relay UE and the remote UE is above a threshold (similar to Rel-18 U2U relay discovery).

Proposal 6: For multi-hop relay discovery Model B, no AS criterion is needed for the intermediate relay UE to forward the response message to the intermediate relay UE/remote UE (similar to Rel-18 U2U relay discovery).

Proposal 7: For multi-hop relay discovery Model B, upon discovery response messages reception, the remote UE considers an intermediate relay UE as a candidate relay UE if the SD-RSRP towards the intermediate relay UE is above a configured threshold (similar to Rel-18 U2U relay discovery).

- Resource pool

In Rel-17 U2N relay, dedicated discovery resource pool is supported besides shared resource pool configuration, whether it is configured is based on network implementation. And PHY layer parameters and design shall reuse the Rel-16 legacy resource pool design (including resource allocation design). For Rel-19 multi-hop U2N relay, the issue raised is whether to further distinguish the dedicated discovery resource pool between single-hop U2N relay and multi-hop U2N relay. From our point of view, there is no clear benefit to further distinguish it. Hence, we prefer that the single-hop discovery and multi-hop discovery can use the same dedicated resource pool.

Proposal 8: There is no need to further distinguish the dedicated discovery resource pool configured for single-hop U2N relay and for multi-hop U2N relay, i.e. no dedicated discovery pool specific to multi-hop U2N relay is introduced.

Discussion on multi-hop U2N relay (re)selection

- Triggers for multi-hop U2N Relay Selection

In Rel-17 U2N relay, the below triggers were agreed for single hop U2N relay selection[3]:

Trigger of relay selection: Triggered at remote UE when: a) direct Uu link quality is below a configured threshold for an in-coverage remote UE (in IDLE/INACTIVE and CONNECTED for L3 U2N relay; L2 case to be further discussed); or b) triggered by upper layer.

For L2 U2N relay, RRC_IDLE/RRC_INACTIVE remote UE triggers relay selection when direct Uu link quality is below a configured threshold, and relay selection for RRC_CONNECTED remote UE by gNB is handled in CP procedure and service continuity topic for L2 relay.

For Rel-19 multi-hop U2N relay case, it is easy to conclude that the trigger of Rel-17 single hop U2N relay selection can be reused totally. Hence, we propose:

Proposal 9: The trigger of Rel-17 U2N relay selection can be reused for Rel-19 multi-hop L2 U2N relay case:

- when triggered by upper layers.

- when direct Uu link quality is below a configured threshold.

Proposal 10:  The trigger of relay selection for Rel-19 multi-hop L2 U2N relay for a remote UE in RRC_CONNECTED is left to service continuity discussion.

Besides, there is one additional case for multi-hop U2N relay which is different to single-hop U2N relay. In the below figure, the start point is single-hop U2N relay, and the U2N remote UE is in RRC_IDLE/RRC_INACTIVE mode. When the U2N remote UE moves far away from U2N relay UE, the channel quality between the U2N remote UE and U2N relay UE will reduce. Then for the U2N remote UE, according to Rel-17 U2N relay’s agreement, the U2N remote UE will be triggered to reselect another U2N relay UE. In case there is one intermediate relay UE, and the U2N remote UE finally selects the intermediate relay UE and connects to gNB by multi-hop U2N relay case, the tricky part is whether we need to treat it as multi-hop U2N relay selection and whether one new trigger condition should be introduced.

Figure 3. the scenario to trigger multi-hop U2N relay selection

Proposal 11: For Rel-19 multi-hop L2 U2N relay, RAN2 discusses whether RRC_IDLE/RRC_INACTIVE remote UE triggers relay selection when PC5 link quality between the remote UE and last relay UE is below a configured threshold.

- Triggers for multi-hop U2N Relay Reselection

In Rel-17 U2N relay, the below triggers are agreed for single hop U2N relay reselection[3]:

Trigger of relay reselection: Triggered at remote UE when: a) PC5 measurement towards current relay UE is below a (pre)configured threshold; or b) Reception of an upper layer release message or similar indication from current relay UE; or c) Triggered by upper layer.

For Rel-19 multi-hop U2N relay case, considering the intermediate U2N relay UE is introduced, the Rel-17 conclusions should be polished to reflect the more detailed situation. Hence, we propose:

Proposal 12: Reuse Rel-18 U2N relay reselection triggers with necessary terminology adaptation in Rel-19 multi-hop L2 U2N relay: Triggered at remote UE for multi-hop relay reselection when: a) PC5 measurement towards intermediate relay UE is below a (pre)configured threshold; or b) Reception of an upper layer release message of similar indication from intermediate relay UE/last relay UE; or c) Triggered by upper layer.

- Criteria for multi-hop U2N Relay (Re)Selection

In LTE, no other AS-related criteria, except radio link quality were specified for relay UE (re)selection. In Rel-17 U2N relay, serving cell ID and PLMN ID are considered as AS criteria. It is up to SA2 to decide how to include L2/L3 relay support in discovery message. Whether L2/L3 relay support can be used as additional criteria for relay (re)selection can be left to SA2. Besides, relay load is not considered as a (re)selection criterion in Rel-17.

For Rel-19 multi-hop U2N relay, we need to discuss that whether any additional AS criteria for relay (re)selection is needed. The most important issue that we raise to discuss is that whether to introduce the hops number as additional AS criteria. In the latest SA2’s TR for Rel-19 [2], the Solution#1 for model A and Solution#2 and #7 for model B are used as basis for normative work. Both solutions had recorded that the hops number will be considered as criteria for multi-hop U2N relay (re)selection. Considering the hop related information had been adopt by SA2 into discovery message, hence we propose:

Proposal 13: It is up to SA2 to decide how to include hop related information in discovery message.

Proposal 14: For relay selection and reselection, when remote UE has multiple suitable intermediate relay UE candidates which meet all AS-layer & higher layer criteria and remote UE need to select one intermediate relay UE by itself, it is up to UE implementation to choose one intermediate relay UE. Whether hop related information can be used as additional criteria for relay (re)selection can be left to SA2.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: RAN2 uses the below terminology (i.e., remote UE -> intermediate relay UE -> intermediate relay UE ->last relay UE -> gNB) in Rel-19 multi-hop L2 U2N relay.

Proposal 2:  For Rel-19 multi-hop L2 U2N relay, RAN2 assumes that further discussions on AS impact on Relay discovery should take into account both Model A and Model B discovery models.

Proposal 3: For multi-hop relay discovery Model A, there is no AS channel condition should be considered for intermediate relay UE when announcing the discovery message.

Proposal 4: For multi-hop relay discovery Model A, upon discovery message reception, remote UE considers an intermediate relay UE as a candidate relay UE if the SD-RSRP towards the intermediate relay UE is above a configured threshold (similar to Rel-18 U2U relay discovery).

Proposal 5: For multi-hop relay discovery Model B, the intermediate relay UE forwards the solicitation message only if the PC5 RSRP between the intermediate relay UE and the remote UE is above a threshold (similar to Rel-18 U2U relay discovery).

Proposal 6: For multi-hop relay discovery Model B, no AS criterion is needed for the intermediate relay UE to forward the response message to the intermediate relay UE/remote UE (similar to Rel-18 U2U relay discovery).

Proposal 7: For multi-hop relay discovery Model B, upon discovery response messages reception, the remote UE considers an intermediate relay UE as a candidate relay UE if the SD-RSRP towards the intermediate relay UE is above a configured threshold (similar to Rel-18 U2U relay discovery).

Proposal 8: There is no need to further distinguish the dedicated discovery resource pool configured for single-hop U2N relay and for multi-hop U2N relay, i.e. no dedicated discovery pool specific to multi-hop U2N relay is introduced.

Proposal 9: The trigger of Rel-17 U2N relay selection can be reused for Rel-19 multi-hop L2 U2N relay case:

- when triggered by upper layers.

- when direct Uu link quality is below a configured threshold.

Proposal 10:  The trigger of relay selection for Rel-19 multi-hop L2 U2N relay for a remote UE in RRC_CONNECTED is left to service continuity discussion.

Proposal 11: For Rel-19 multi-hop L2 U2N relay, RAN2 discusses whether RRC_IDLE/RRC_INACTIVE remote UE triggers relay selection when PC5 link quality between the remote UE and last relay UE is below a configured threshold.

Proposal 12: Reuse Rel-18 U2N relay reselection triggers with necessary terminology adaptation in Rel-19 multi-hop L2 U2N relay: Triggered at remote UE for multi-hop relay reselection when: a) PC5 measurement towards intermediate relay UE is below a (pre)configured threshold; or b) Reception of an upper layer release message of similar indication from intermediate relay UE/last relay UE; or c) Triggered by upper layer.

Proposal 13: It is up to SA2 to decide how to include hop related information in discovery message.

Proposal 14: For relay selection and reselection, when remote UE has multiple suitable intermediate relay UE candidates which meet all AS-layer & higher layer criteria and remote UE need to select one intermediate relay UE by itself, it is up to UE implementation to choose one intermediate relay UE. Whether hop related information can be used as additional criteria for relay (re)selection can be left to SA2.

Reference

- RP-241609  New WID on NR sidelink multi-hop relay

- TR23700-03   Study on system enhancement for Proximity based Service (ProSe) in the 5G System (5GS) Phase 3

- R2-2106641 Report of 3GPP TSG RAN WG2 meeting #113bis-e, Online ETSI MCC

- S2-2407207 LS on security considerations for 5G ProSe multi-hop relays