3GPP TSG-RAN WG2 Meeting #127                                                        R2-2406563

Maastricht, NL, 19th – 23rd Aug, 2024

Source:	CATT

Title:	E2E Connection Setup and QoS Split for Multi-hop Relay

Agenda Item:	8.13.3

Document for:	Discussion and Decision

Introduction

In RAN#104 meeting, the WID of Rel-19 NR sidelink multi-hop relay [1] was agreed. In this contribution, the E2E connection setup and QoS split will be discussed.

Discussion

Whether integrated connection setup is supported?

In Rel-18 U2U relay, regarding to the relationship between discovery and connection setup, there are two alternatives:

- Alt 1:  Separate discovery and connection setup procedure;

- Alt 2:  Integrated discovery and connection setup procedure.

In TR23.700-03 [2], the following conclusions were reached:

- Standalone discovery & link setup/management is considered and both Model A and Model B are supported. - When Model A discovery is performed, the Remote UE may choose the Intermediate Relay based on the announcement message sent by the Intermediate Relay. - When Model B discovery is performed, the Remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay. To perform link management, the DCR message is unicasted between Relays according to the path information included in the message. The path information is an (ordered) list of User Info ID of Relays in the selected path. The Remote UE sends the selected path information to the Intermediate UE-to-Network Relay for communication setup.

Based on the above conclusions, it is obvious that SA2 already agreed that standalone discovery and link setup is considered and Remote UE is responsible for the intermediate relay selection. Considering there is no need to establish the E2E PC5-RRC connection between the remote UE and the U2N relay UE connected with network via Uu, hence, integrated relay discovery and connection setup procedure is not considered in Rel-19.

Proposal 1:  Separate discovery and connection setup procedure is used for Rel-19 multi-hop relay.

How to establish the E2E connection between remote UE and network?

In Rel-17 U2N relay, the E2E connection setup procedure is as below:

Figure-1 Procedure for L2 U2N Remote UE connection establishment (Rel-17)

In case of multi-hop, the similar procedure can be used, as shown in the following figure:

Figure-2 Procedure for L2 U2N Remote UE connection establishment (Rel-19)

For the above Figure-2, step 1/2 is similar as Rel-17 U2N relay. The key difference is on step 3.

For step 3, in Rel-17 U2N relay, specified PC5 RLC channel can be used to transmit SRB0 between Remote UE and U2N relay. The same principle can be reused in Rel-19.

Proposal 2:  Specified PC5 RLC channel can be used to transmit E2E SRB0 between remote UE and intermediate relay UE.

For step 3, when intermediate relay UE receives the SL-RLC0, its next behavior depends on its RRC state. The detailed analysis is as below:

Based on the solutions listed in TR23.700-03 [2], SA2 made the following conclusions [3]:

SA2 would like to inform SA3 that for Rel-19 FS_5G_ProSe_Ph3 study has concluded, and the solutions were documented in TR 23.700-03. For the support of multi-hop UE-to-Network Relay operations (KI#1 of TR 23.700-03), it was concluded that: - for Model A based discovery and corresponding link establishment/management, Solution#1 (as documented in clause 6.1 of TR 23.700-03) would be used as basis for normative work; - for Model B based discovery and corresponding link establishment/management, Solution#2 and #7 (as documented in clause 6.2 and 6.7 of TR 23.700-03) would be used as basis for normative work.

For relay discovery model A, solution #1 in TR23.700-03 [2] is baseline and for relay discovery model B, solution #2 and solution#7 is baseline.

For Relay discovery model A, the following was captured in solution #1 in TR23.700-03 [2]:

To act as an Intermediate UE-to-Network Relay, e.g. forwarding the Model A discovery message or replying to the Model B Discovery Response message, the relay UE needs to have a valid connection towards the network via the 5G ProSe UE-to-Network Relay indicated by the Root Relay Info IE.

Based on the above, it can be deduced that relay discovery model A, there should be PC5-RRC connections between intermediate relay UE and its parent relay. But considering there may be no data transmission requirement during discovery procedure, both intermediate relay UE and U2N relay can be in RRC_IDLE/INACTIVE. That is to say, there may be no E2E connection between intermediate relay and network.

Proposal 3: RAN2 to confirm that for relay discovery Model A, there is PC5-S/PC5-RRC connection between intermediate relay and its parent relay, but both the intermediate relay and U2N relay can be in IDLE/INACTIVE state.

For relay discovery model B, there is no requirement on PC5-S/PC5-RRC connection in TR23.700-03[2].

Proposal 4: RAN2 to confirm that for relay discovery Model B, it is possible that there is no PC5-S/PC5-RRC connection between intermediate relay and its parent relay.

Based on Proposal 3/4, one issue needs to be discussed is that whether intermediate relay UE needs to establish E2E connection with network upon receiving SL-RLC0 from remote UE. For multi-hop relay, in order to support the routing, at least SRAP configuration is needed for each intermediate relay UE. In legacy U2N relay, the SRAP configuration is sent by NW through dedicated RRC signaling. If there is no E2E RRC connection between intermediate relay UE and NW, intermediate relay UE cannot receive the SRAP configuration through dedicated RRC signaling. Hence, in order to facilitate to provide the SRAP configuration to intermediate relay UE, it had better to establish the E2E RRC connection between intermediate relay UE and NW.

Proposal 5: RAN2 to agree E2E RRC connection is necessary between intermediate relay UE and NW.

If Proposal 5 is agreed, if both intermediate relay and U2N relay have E2E connection with network, it should further discuss which relay UE send sidelinkUEInformationNR to network to request the local L2 ID of remote UE and request the Egress RLC channel configuration for routing SL-RLC0. There are two alternatives:

- Alternative 1: Intermediate relay UE sends sidelinkUEInformationNR to network;

- Alternative 2: U2N relay UE sends sidelinkUEInformationNR to network.

No matter which alternative is used, network should aware the list of all relay UEs on the multi-hop path in order to provide the proper SRAP configuration for each relay UE. If Alternative 1 is used, the network can aware the list of all relay UEs on the multi-hops since each intermediate relay UE establish the E2E connection with network through its parent relay. But if Alternative 2 is used, the network cannot aware the list of all relay UEs and enhancement is needed. For example, the U2N relay should get the list of all relay UEs and report it to network via sidelinkUEInformationNR. This will bring specification effort. Hence, Alternative 1 is slightly preferred.

Proposal 6: Upon receiving SL-RLC0 from remote UE, intermediate relay UE can send SidelinkUEInformationNR to network.

Based on sidelinkUEInformationNR sent by intermediate relay UE, network can allocate the local L2 ID for remote UE and at least configure the mapping between SRB0 and its Egress RLC channel for each intermediate relay and U2N relay.

Proposal 7: Upon receiving SidelinkUEInformationNR from intermediate UE, network can allocate Local L2 ID for remote UE and at least configure the mapping between SRB0 and its Egress RLC channel for each intermediate relay and U2N relay.

After remote UE receiving the RRCSetup, it should further discuss which PC5 RLC channel can be used for transmission the E2E SL-SRB1. In legacy Rel-17 U2N relay, default SL-RLC1 can be used. The same principle can be used for multi-hop U2N relay.

Proposal 8:  Default PC5 RLC channel can be used to transmit E2E SRB1 between remote UE and intermediate relay UE.

QoS split for Multi-hop relay

In Rel-17, for L2 U2N relay, the network is responsible for QoS split between the Uu and PC5 interface. And only one parameter needs to be split, that is PDB.

In Rel-19, with the introduction of multi-hop relay, in order to meet the E2E QoS requirement, it should also discuss which node is responsible for QoS split and which QoS parameter needs to be split.

Based on Proposal 5, if there is E2E RRC connection between each relay UE and network, it is feasible to reuse the QoS split method in Rel-17 L2 U2N relay. That is the network can decide how to split the QoS for each hop and the network should inform the split result to remote UE and each relay UE. The parameter needs to be split is PDB.

Proposal 9: The network is responsible for the QoS split for each hop.

Proposal 10: Same as Rel-17 L2 U2N relay, the parameter needs to be split is PDB.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1:  Separate discovery and connection setup procedure is used for Rel-19 multi-hop relay.

Proposal 2:  Specified PC5 RLC channel can be used to transmit E2E SRB0 between remote UE and intermediate relay UE.

Proposal 3: RAN2 to confirm that for relay discovery Model A, there is PC5-S/PC5-RRC connection between intermediate relay and its parent relay, but both the intermediate relay and U2N relay can be in IDLE/INACTIVE state.

Proposal 4: RAN2 to confirm that for relay discovery Model B, it is possible that there is no PC5-S/PC5-RRC connection between intermediate relay and its parent relay.

Proposal 5: RAN2 to agree E2E RRC connection is necessary between intermediate relay UE and NW.

Proposal 6: Upon receiving SL-RLC0 from remote UE, intermediate relay UE can send SidelinkUEInformationNR to network.

Proposal 7: Upon receiving SidelinkUEInformationNR from intermediate UE, network can allocate Local L2 ID for remote UE and at least configure the mapping between SRB0 and its Egress RLC channel for each intermediate relay and U2N relay.

Proposal 8:  Default PC5 RLC channel can be used to transmit E2E SRB1 between remote UE and intermediate relay UE.

Proposal 9: The network is responsible for the QoS split for each hop.

Proposal 10: Same as Rel-17 L2 U2N relay, the parameter needs to be split is PDB.

Reference

- RP-241609   New WID on NR sidelink multi-hop relay

- TR23700-03   Study on system enhancement for Proximity based Service (ProSe) in the 5G System (5GS) Phase 3

- S2-2407207   LS on security considerations for 5G ProSe multi-hop relays