3GPP TSG-RAN WG2 Meeting #127　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　R2-2406565

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.2.5 Topology 2 considerations

Source:	NEC

Title:	Discussion on topology 2 related aspects between gNB and reader

Document for:	Discussion, Decision

Introduction

Rel-19 Ambient IoT (A-IoT) SI was approved at RAN#102. Following two deployment scenarios are included in the SID, RP-234058.

•	Deployment scenario 1 with Topology 1

•	Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

This contribution provides our initial views on topology 2 related aspects between gNB and reader.

Discussion

Topology 2 with UE as intermediate node is illustrated in Fig.1.

- Fig.1 Topology 2 with UE as intermediate node

- In Fig.1, the A-IoT device communicates bidirectionally with the reader, a UE supporting A-IoT radio (as an intermediate node ) between the device and base station (BS). A-IoT data and/or signalling between BS and the A-IoT device is transferred by the UE supporting A-IoT radio. In other words,

- For the reader to device direction (R2D)

- A-IoT data and/or signalling from the CN is firstly transferred over Uu interface between the BS and the UE reader, then conveyed over A-IoT radio interface between the UE reader and the A-IoT device.

- For the device to reader direction (D2R)

- A-IoT data and/or signalling from the device is firstly conveyed over A-IoT radio interface between the A-IoT device and the UE reader, then transferred over Uu interface between the UE reader and the BS.

For Topology 2 with UE as intermediate node, RAN2 assume A-IoT data and/or signalling is transferred over Uu interface between the BS and the UE reader and conveyed over A-IoT radio interface between the UE reader and A-IoT devices.

- Compared to Topology 1 in which the BS is the reader, how to transfer A-IoT data and/or signalling over Uu interface needs to be discussed additionally. In other words, there is no difference on A-IoT radio interface between Topology 1 and 2. Therefore, no enhancements are needed for conveying A-IoT data and/or signalling over A-IoT radio interface between the UE reader and A-IoT devices.

For Topology 2 with UE as intermediate node, RAN2 assume no enhancements are needed for conveying A-IoT data and/or signalling over A-IoT radio interface between the UE reader and A-IoT devices.

- To transfer A-IoT data and/or signalling over Uu interface, the first issue is whether control plane or user plane can be used. Here the control place solution means to transfer A-IoT data and/or signalling via RRC message/MAC CE/DCI which generated at the BS and transferred from the BS to the UE reader. Whereas the user plane solution means to transfer A-IoT data and/or signalling via a dedicated DRB from the BS to the UE reader. At the time being, RAN2 consider both control plane and user plane solutions as candidates and further study their pros/cons.

For Topology 2 with UE as intermediate node, RAN2 consider both control plane (i.e., via RRC message/MAC CE/DCI enhancements) and user plane (i.e., via dedicated DRB enhancement) solutions for transferring A-IoT data and/or signalling over Uu interface from the BS to the UE reader.

- Hereafter, we focus on control plane solution to transfer A-IoT data and/or signalling over Uu interface in Topology 2 with the UE reader and take inventory service defined in [1] as an example. Inventory service defined in [1] is copied below:

-	Inventory: Refers to determining the identity of all or a subset of AIoT Devices in range of a reader.

Moreover, the A-IoT reader is assumed to support functionality of performing one-time or periodic inventory in Clause 6.3.1.5.1 [1].

For Topology 2 with the UE reader, upon receiving the service trigger (e.g., for one-time inventory or for periodic inventory) from the CN, firstly the BS needs to transfer this service trigger over Uu interface. From our understanding, MAC CE/DCI enhancements for supporting transfer of the service request of one-time inventory over Uu interface between the BS and the UE reader is appropriate especially when latency is defined as a KPI of the inventory service. On the other hand, RRC signaling enhancements for supporting transfer of the service request of periodic inventory over Uu interface between the BS and the UE reader is an alternative for periodic inventory service. So far A-IoT signalling is still under discussion, therefore, choosing among RRC signalling/MAC CE/DCI enhancements may depend on individual A-IoT signalling design and can be further discussed later.

For control plane solution in Topology 2 with UE as intermediate node, RAN2 assume to enhance RRC signaling/MAC CE to support transfer of A-IoT data and/or signalling over Uu interface between the BS and the UE reader.

As indicated in [1] and highlighted above in yellow, inventory service trigger may be intended for all or a subset of A-IoT devices. The CN may optionally include Filter Criteria or a list of reader IDs in the service trigger. Here,

-	Filter Criteria is to limit the inventory to AIoT Devices matching those criteria;

-	a list of reader IDs is to limit the inventory to specific Readers;

Therefore, for topology 2 with UE as intermediate node, it is necessary to support broadcast, multicast, and unicast transferring of A-IoT data and/or signalling over Uu interface between the BS and the UE reader.

For Topology 2 with UE as intermediate node, RAN2 assume to support broadcast, multicast, and unicast transferring of A-IoT data and/or signalling over Uu interface between the BS and the UE reader.

A further issue needs to be considered is there may be more than one UE capable of operating as intermediate node, i.e., operating as the reader. Therefore, to determine the UE readers to be involved in one A-IoT data and/or signalling transferring procedure, following two-way reader selection may be considered:

- the BS perform reader selection before or upon triggering of the A-IoT data and/or signalling transferring;

- the selected UE readers determine whether to be involved in one A-IoT procedure based on predefined criteria or criteria indicated by the BS.

For Topology 2 with UE as intermediate node, RAN2 consider following two-way reader selection to determine UE readers to be involved in one A-IoT data and/or signalling transferring procedure:

- the BS perform reader selection before or upon triggering of the A-IoT data and/or signalling transferring;

- the selected UE readers determine whether to be involved in a A-IoT data and/or signalling transferring procedure based on predefined criteria or criteria indicated by the BS.

Summary

This contribution provided our initial view on Topology 2 related aspects between gNB and reader.

Proposal 1	For Topology 2 with UE as intermediate node, RAN2 assume A-IoT data and/or signalling is transferred over Uu interface between the BS and the UE reader and conveyed over A-IoT radio interface between the UE reader and A-IoT devices.

Proposal 2	For Topology 2 with UE as intermediate node, RAN2 assume no enhancements are needed for conveying A-IoT data and/or signalling over A-IoT radio interface between the UE reader and A-IoT devices.

Proposal 3	For Topology 2 with UE as intermediate node, RAN2 consider both control plane (i.e., via RRC message/MAC CE/DCI enhancements) and user plane (i.e., via dedicated DRB enhancement) solutions for transferring A-IoT data and/or signalling over Uu interface from the BS to the UE reader.

Proposal 4	For control plane solution in Topology 2 with UE as intermediate node, RAN2 assume to enhance RRC signaling/MAC CE to support transfer of A-IoT data and/or signalling over Uu interface between the BS and the UE reader.

Proposal 5	For Topology 2 with UE as intermediate node, RAN2 assume to support broadcast, multicast, and unicast transferring of A-IoT data and/or signalling over Uu interface between the BS and the UE reader.

Proposal 6	For Topology 2 with UE as intermediate node, RAN2 consider following two-way reader selection to determine UE readers to be involved in one A-IoT data and/or signalling transferring procedure:

a.	the BS perform reader selection before or upon triggering of the A-IoT data and/or signalling transferring;

b.	the selected UE readers determine whether to be involved in a A-IoT data and/or signalling transferring procedure based on predefined criteria or criteria indicated by the BS.

- References

[1] TR 23.700-13 V0.3.0