3GPP TSG-RAN WG2 Meeting #127　　　　　　　　　　　　　　　　　　　　　　　　　　　　　  　R2-2406566

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.7.1.1 Discussion on incoming LSs

Source:	NEC

Title:	Discussion on SA2 LS on FS_XRM Ph2 and RAN3 LS on UL PSI based PDU discarding in NR-DC

Document for:	Discussion, Decision

Introduction

SA2 sent an LS on FS_XRM Ph2 (S2-2407351) and RAN3 sent an LS on UL PSI based PDU discarding in NR-DC (R3-243957).

In this contribution, we will provide our views on these two LSs.

Discussion

SA2 LS on FS_XRM Ph2 (S2-2407351)

The content of SA2 LS is copied below and the questions asked by SA2 are highlighted in yellow.

SA2 has started to evaluate solutions for key issues for the study of XR and Media Services Ph2 (TR 23.700-70). SA2 thanks the LS replies for XRM Ph2 from RAN2, RAN3 and SA4 and SA2 would like to further coordinate on the following aspects: - Question1 [for SA4, RAN2 and RAN3]:  SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.  To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension? - Question2 [for SA4 and RAN2]: There is some discussion about time to next burst.  To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6?  Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server? - Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

Question1: indicate dynamic changes of the periodicity via in-band signalling (i.e., in GTP-U)

In Rel-18, 5GC may provide XR traffic assistance information to gNB through NG AP TSU Assistance Information (TSCAI) in which UL and/or DL Periodicity is included. Firstly, we observe that the periodicity changes indicating via NG AP TSCAI is supported in Rel-18.

The periodicity changes indicating via NG AP TSCAI is supported in Rel-18.

Regarding SA2 Question 1 which is focusing on indicating periodicity via GTP-U, we don’t see additional value from RAN2 perspective since this assistance information is mainly used by the gNB to configure DRX to enable better UE power saving, as indicated in TS38.300.

Reply to SA2 that from RAN2 perspective, no additional value is foreseen by additionally indicating periodicity via in-band signaling (i.e. in GTP-U) since that has been supported via NG AP TSCAI in Rel-18.

Question 2: Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

Currently, end of a burst information could be sent to NG-RAN to end up the scheduling to a UE and/or end up the DRX active time. Similarly, if the time to next burst is provided to NG-RAN, it could be used to prepare the scheduling to a UE and/or adjust its next on-duration window.

Reply to SA2 that the time to next burst is useful for RAN resource scheduling, assuming possible jitter between the NG-RAN and the application server is negligible.

RAN3 LS on UL PSI based PDU discarding in NR-DC (R3-243957)

The content of RAN3 LS is copied below and the question asked by RAN3 is highlighted in yellow.

RAN3 is currently discussing XR DC signaling enhancements for various bearer types. In case of NR-DC split bearer, MN DU and SN DU congestion is local to each node. Based on congestion situation in MN DU and/or SN DU, each of the nodes independently send PSI-Based SDU Discard Activation/Deactivation MAC CE to UE for activation/deactivation of UL PSI based PDU set discarding of the split bearer. when UE receives MAC CE from both nodes or from a single node, it is not clear how UE will activate/deactivate UL PSI based PDU set discarding when split bearer is configured. Thus, RAN3 kindly asks RAN2 to clarify the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE.

PSI based SDU discard with UL split bearers was discussed at RAN2#125bis meeting and conclusion of this topic was [1]:

- No further optimizations will be supported in Rel-18 for UL split-bearer

No further optimizations for UL split-bearer are supported in Rel-18 XR.

After checking Activation/deactivation of PSI-based SDU discard specified in TS38.321 Clause 5.18.3 (copied below), we observe that there is no specific UE behavior definition for UL-split bearer.

5.18.34 Activation/deactivation of PSI-based SDU discard The network activates and deactivates PSI-based SDU discard by sending the PSI-Based SDU Discard Activation/Deactivation MAC CE described in clause 6.1.3.73. The PSI-based SDU discard is initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync. The MAC entity shall for each DRB configured with discardTimerForLowImportance: 1> if a PSI-Based SDU Discard Activation/Deactivation MAC CE is received activating the PSI-based SDU discard for the DRB: 2> indicate the activation of the PSI-based SDU discard for the DRB to upper-layers. 1> if a PSI-Based SDU Discard Activation/Deactivation MAC CE is received deactivating the PSI-based SDU discard for the DRB: 2> indicate the deactivation of the PSI-based SDU discard for the DRB to upper-layers.

Regarding Activation/deactivation of PSI-based SDU discard, no specific UE behaviours are specified for UL-split bearer in Rel-18 XR.

Based on above observations, we summarize our understanding on possible UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node send the PSI-Based SDU Discard Activation/Deactivation MAC CE in Table 1. We believe there is no ambiguity on UE behavior to follow PSI-Based SDU Discard Activation/Deactivation MAC CE when both MN and SN send activation or deactivation. On the other hand, when PSI-Based SDU Discard Activation/Deactivation MAC CE from MN and SN are different (i.e., one side sends activation/deactivation while the other side sends deactivation/activation), we think whether activate (or deactivate) the PSI based PDU discarding is up to UE implementation. UE may choose to adjust the parameter ul-DataSplitThreshold before changing the status of PSI based PDU discarding.

Table 1　UE behavior for handling uplink PSI based PDU discarding

| - PSI-Based SDU Discard Activation/Deactivation MAC CE |  | - UE behaviour |
| --- | --- | --- |
| - MN node | - SN node |  |
| - activated | - activated | - activate PSI-based SDU discard |
| - deactivated | - deactivated | - deactivate PSI-based SDU discard |
| - activated | - deactivated | - Alt.1 activate PSI-based SDU discard - Alt.2 deactivate PSI-based SDU discard (UE may choose to adjust the parameter ul-DataSplitThreshold) Note: Either Alt.1 or Alt.2 is up to UE implementation. |
| - deactivated | - activated | Alt.1 activate PSI-based SDU discard Alt.2 deactivate PSI-based SDU discard (UE may choose to adjust the parameter ul-DataSplitThreshold) Note: Either Alt.1 or Alt.2 is up to UE implementation. |

Reply to RAN3 that UE behavior for handling uplink PSI based PDU discarding are:
- when MN and SN node send the same PSI-Based SDU Discard Activation/Deactivation MAC CE (activated or deactivated), the UE follows the received MAC CE

- - when MN and SN node send different PSI-Based SDU Discard Activation/Deactivation MAC CE (i.e., one side sends activation/deactivation while the other side sends deactivation/activation), whether to activate PSI based PDU discarding is up to UE implementation.

Summary

This contribution provided our views on SA2 LS and RAN3 LS.

SA2 LS on FS_XRM Ph2 (S2-2407351)

Observation 1	The periodicity changes indicating via NG AP TSCAI is supported in Rel-18.

Proposal 1	Reply to SA2 that from RAN2 perspective, no additional value is foreseen by additionally indicating periodicity via in-band signaling (i.e. in GTP-U) since that has been supported via NG AP TSCAI in Rel-18.

Proposal 2	Reply to SA2 that the time to next burst is useful for RAN resource scheduling, assuming possible jitter between the NG-RAN and the application server is negligible.

RAN3 LS on UL PSI based PDU discarding in NR-DC (R3-243957)

Observation 1	No further optimizations for UL split-bearer are supported in Rel-18 XR.

Observation 2	Regarding Activation/deactivation of PSI-based SDU discard, no specific UE behaviors are specified for UL-split bearer in Rel-18 XR.

Proposal 3	Reply to RAN3 that UE behavior for handling uplink PSI based PDU discarding are:

- when MN and SN node send the same PSI-Based SDU Discard Activation/Deactivation MAC CE (activated or deactivated), the UE follows the received MAC CE

- when MN and SN node send different PSI-Based SDU Discard Activation/Deactivation MAC CE (i.e., one side sends activation/deactivation while the other side sends deactivation/activation), whether to activate PSI based PDU discarding is up to UE implementation.

- References

[1] R2-2404102	Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China