3GPP TSG-RAN WG2 Meeting #127　　　　　　　　　　　　　　　　　　　　　　　　　　　　　  　R2-2406567

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.7.2 Multi-modality support

Source:	NEC

Title:	Potential enhancements based on multi-modal information awareness

Document for:	Discussion, Decision

Introduction

During RAN2#126 meeting, following agreements on multi-modality support was achieved and an LS to SA2 (CC: RAN3, SA4) was approved in R2-2405782 [1].

Agreements: - Support Multi-Modality awareness in RAN in Rel-19 for UL and DL. - Ask SA2 whether/what information could be provided to RAN

This contribution further discusses potential enhancements based on multi-modal information awareness, e.g., MMSID and/or synchronization thresholds in order to allow e.g., coordinated handling of the flows at RAN.

Discussion

With awareness of multi-modal synchronization requirement, following potential enhancements can be supported by RAN:

- transmit packets of multi-modal flows within synchronization thresholds

- handle multi-modal flows together

- DRX enhancements for multi-modal flows

- Transmit packets of multi-modal flows within synchronization thresholds

RAN2#125bis agreed to map multi-modal traffic having different QoS requirements to different QoS flow, therefore, on how to map multi-modal QoS flows to DRB, we think there are two options:

Option 1: Mapping different multi-modal QoS flows to a single DRB.

Option 2: Mapping different multi-modal QoS flows to different DRBs, eventually different LCPs for the UL.

Both options are shown in Fig.1.

Here the synchronization threshold requirements from the upper layer are assumed to be applied for packets belonging to different multi-modal flows (e.g., synchronized delivery time threshold of packets with the same sequence number belonging to different multi-modal flows). With Option 1, packets of different multi-modal QoS flows are mapped to a single DRB, therefore, to meet the synchronization threshold requirements, transmitting side of the SDAP entity needs to submit packets of multi-modal flows in the expected sequence to lower layers. So far, such a SDAP transmitting behavior is not supported by specification and it can be left to SDAP implementation. On the other hand, with Option 2, packets of different multi-modal flows are mapped to different DRBs. For the DL transmission, prioritization between different DRBs can be left to gNB implementation, however, for the UL transmission, to meet the synchronization threshold requirements, the LCP procedure enhancements considering synchronization threshold of LCHs is needed.

For UL transmission, to meet the synchronization threshold requirements, multi-modal aware LCP enhancements considering multi-modal synchronization threshold requirements is needed.

To meet synchronization threshold requirements, multi-modal aware LCP enhancements considering multi-modal synchronization threshold requirements is supported in Rel-19 XR.

LCP enhancements (i.e., LCH prioritization and LCH restrictions) was discussed as potential solutions for resolving the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority. Following agreements were achieved at RAN2#126 meeting.

Agreements (a part) on LCH prioritization: - Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. - The solution should consider impact on UE complexity (as already indicated in SI objective description) - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data Agreements on LCH restriction: - We try to avoid RAN1 impacts. - RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way.

Like the delay-aware LCP enhancement, for multi-modal aware LCP enhancements, the network can configure an additional priority (assuming it is higher than the legacy LCH priority) to LCH pairs corresponding to multi-modal DRBs/QoS flows. When a multi-modal LCH is selected by the MAC entity to allocate resource to, then the paired multi-modal LCH priority is overridden/adjusted to the additional priority.

For multi-modal aware LCP enhancement, RAN2 agree to use additional priority configured to multi-modal LCHs to override/adjust the priority of a multi-modal LCH in case of the paired LCH is selected to allocate resource to.

Furthermore, to determine the period of overriding/adjusting multi-modal LCH priority, a timer configured by the network (e.g., deriving from multi-modal synchronization threshold) is useful.

- Upon overriding/adjusting the priority of a multi-modal LCH, the timer is started

- if the timer expires, fall back to the legacy LCH priority

For multi-modal aware LCP enhancement, RAN2 agree to support a timer configured by the network to determine the period of overriding/adjusting multi-modal LCH priority.
- Upon overriding/adjusting the priority of a multi-modal LCH, the timer is started
- if the timer expires, fall back to the legacy LCH priority

Handle multi-modal flows together

Based on multi-modal information awareness, following potential enhancements for handling multi-modal flows together are possible:

- setup/release/modify multi-modality flows together

- discard multi-modality flows together

According to TS38.300 ANNEX A (informative) QoS handing in RAN, QoS flow setup is basically triggered by PDU session Establishment Request from UE to AMF and associated with PDU SESSION RESOURCE SETUP REQUEST from AMF to gNB.

After PDU session and DRBs establishment, TS38.300 ANNEX A also shows some example message flow for following cases:

- When the gNB receives a request to release a QoS flow from CN that involves explicit NAS signalling (ANNEC A.5)

- when the UE AS receives an UL packet for a new QoS flow for which a WFI to DRB mapping rule does not exist (ANNEX A.6)

TS38.413 specifies that for each PDU Session Resource Modify Request List IE:

- For each QoS flow included in the QoS Flow Add or Modify Request List IE, based on the QoS Flow Level QoS Parameters ID, the NG-RAN node may establish, modify or release the DRB configuration and may change allocation of resources on NG or Uu accordingly.

Furthermore, QoS Flow Accepted List, QoS Flow Failed to Setup List, QoS Flow Failed to Add or Modify List, QoS Flow Failed to Modify List and QoS Flow Failed to Resume List and associated behaviors are also specified in TS38.413. Based on the above analyses, we observe that setup/release/modify multi-modality flows together mainly falls in RAN3 scope.

Setup/release/modify multi-modality flows together mainly falls in RAN3 scope.

Hereafter, we focus on discard multi-modality flows together, which is in the RAN2 scope in our understanding.

PDU set based discard operation introduced in Rel-18 could be a reference of discarding multi-modality flows together. In PDU set based discard operation, all packets in a PDU set will be discarded when one PDU belonging to this PDU set is discarded due to discard timer expiry. Like that, in discarding multi-modality flow together, all packets belonging to a QoS flow/DRB associated with a multi-modality service can be discarded when discarding of one PDU belonging to the paired QoS flow/DRB happens.

RAN2 further discuss on discarding multi-modality flows together. Current PDCP SDU discard mechanism (e.g., PDU set based discard operation) can be taken as a starting point.

DRX enhancements for multi-modal flows

It is agreed that multi-modality support is for both DL and UL. Unlike UL, DL scheduling handling is mostly up to NW implementation, but the scheduling opportunity is subject to DRX. We believe DRX feature is important for XR, because both power saving and capacity have been considered equally important since Rel18 XR WI.

With multi-modal service, e.g., video + audio/data, different flows may have different periodicity (e.g., 16.6 and 10ms), different PDB (e.g., 10ms vs 30ms) and different packet size. With CA, all serving cells of a MAC entity may be grouped into two groups and configured with separate DRX. This two DRX cell groups could be configured with different drx-onDurationTimer, drx-InactivityTimer. However other key parameters, like periodicity, offset is still common. This does not fit for the multi-model flows with different periodicity and arrival time.

Non-integer DRX might be configured by considering multi-modality, however it is difficult to find a periodicity which can accommodate both flows with PDB satisfaction, and it will inevitably cause extra delay due to the misalignment between DRX cycle and either stream traffic cycle, or it will cause extra power consumption due to the long on duration.

To better support the multi-modality flows, multiple independent DRX could be configured and activated for a UE, and each DRX configuration will be aligned with each flows’ periodicity, arrival time and PDB requirements. Different from multiple DRX proposed for NES, these DRX should be run at the same time rather than only one DRX is activated at a time.

To support multi-modality, introduce multiple active independent DRX configurations.

- Other: Multi-modal awareness for Dual Connectivity

Dual connectivity (i.e., DC) is within the Rel-19 WI scope by default. For DC with split bearer, if the total amount of data volume pending for transmission is equal to or larger than the configured threshold (i.e., ul-DataSplitThreshold), the data shall be submitted to the primary entity or split secondary RLC entity based on UE’s implementation. Therefore, UE may submit the synchronous data of multi-modal flows to different RLC entities when split transmission happens. Since the data of RLC entities (or LCHs) belonging to different cell groups shall be scheduled and handled independently, the synchronized transmission may not be guaranteed for split bearer.

To solve the issue, some mapping constraints can be introduced to make sure the transmitting PDCP entity shall submit synchronous data of multi-modal flows to the same RLC entity when split transmission happens.

Synchronized transmission of multi-modal flows may not be guaranteed when split bearer is configured.

To support synchronized transmission of multi-modal flows, applying the constraint to the transmitting PDCP entity to submit synchronous data to the same RLC entity when split transmission happens.

Summary

This contribution discussed potential enhancements based on multi-modal information awareness, e.g., MMSID and/or synchronization thresholds in order to allow e.g., coordinated handling of the flows at RAN.

Observation 1	For UL transmission, to meet the synchronization threshold requirements, multi-modal aware LCP enhancements considering multi-modal synchronization threshold requirements is needed.

Observation 2	Setup/release/modify multi-modality flows together mainly falls in RAN3 scope.

Observation 3	Synchronized transmission of multi-modal flows may not be guaranteed when split bearer is configured.

Proposal 1	To meet synchronization threshold requirements, multi-modal aware LCP enhancements considering multi-modal synchronization threshold requirements is supported in Rel-19 XR.

Proposal 2	For multi-modal aware LCP enhancement, RAN2 agree to use additional priority configured to multi-modal LCHs to override/adjust the priority of a multi-modal LCH in case of the paired LCH is selected to allocate resource to.

Proposal 3	For multi-modal aware LCP enhancement, RAN2 agree to support a timer configured by the network to determine the period of overriding/adjusting multi-modal LCH priority.

- Upon overriding/adjusting the priority of a multi-modal LCH, the timer is started

- if the timer expires, fall back to the legacy LCH priority

Proposal 4	RAN2 further discuss on discarding multi-modality flows together. Current PDCP SDU discard mechanism (e.g., PDU set based discard operation) can be taken as a starting point.

Proposal 5	To support multi-modality, introduce multiple active independent DRX configurations.

Proposal 6	To support synchronized transmission of multi-modal flows, applying the constraint to the transmitting PDCP entity to submit synchronous data to the same RLC entity when split transmission happens.

- References

[1] R2-2405706	Report from session on R18 MBS, R18 QoE and R19 XR	Session chair (Huawei)