3GPP TSG-RAN WG2 Meeting#127                                	R2-2406569

Maastricht, Netherlands, August 19th – 23th, 2024

Agenda item:	8.5.3

Source: 	Google

Title: 	The procedure for the on-demand SIB1 transmission

Document for:	Discussion and Decision

# Introduction

The second phase (i.e., Release-19) of the NES WI was kicked off in February 2024, along with the latest Work Item Description (WID) RP-234065 [1]. The WID indicates that 3GPP will study the procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode.

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

The first Rel-19 meeting (i.e., RAN2#125bis) [2] had some initial discussions regarding the scenarios of the on-demand SIB1 feature and decided to focus on the scenario 1a (Cell A SIB assisted intra-cell WUS; WUS and SIB1 is sent to/from NES cell). Further agreements were made in the RAN2#126 meeting regarding the general procedure including the triggering condition for the UL WUS transmission and the content of the UL WUS [3]. In view of the current progress in RAN2 and RAN1, this paper discuss further the details on the procedure and the possible enhancements from the signaling perspective.

# Discussion

## The supported scenarios

It was agreed in RAN2#125bis [2] that at least RAN2 considers scenario 1a (i.e., Cell A SIB-assisted intra-cell WUS) as the starting point for the on-demand SIB1 discussion. Scenario 1a (Case 2 defined in RAN1) implies the scenario where the UL WUS configuration of the NES cell is not provided in the NES cell itself, but is provided in another cell (i.e., Cell A) that is the neighbor cell of the NES cell or the macro cell covering the NES cell. It is therefore obvious that the prerequisite of Case 2 is that UE has to camp on Cell A already before it can request the on-demand SIB1 transmission from the NES cell. That is, Case 2 only works for the UEs being in the cell reselection phase, but does not work for the UEs being in the cell selection phase.

Case 2 defined in RAN1 only works for the UEs in the cell reselection phase. The UEs in the cell selection phase cannot request the on-demand SIB1 transmission in Case 2.

If we only implement the on-demand SIB1 solution that only works in the cell selection scenario, all the NES cells providing on-demand SIB1 will not available to the UE and hence UEs will have much less choice in the cell selectin phase compared to in the cell reselection phase. The UE may still measure/detect these NES cells in the cell selectin phase, but eventually will not consider these cells as suitable cells due to the absence of SIB1. If any of these NES cell is closer to the UE (compared to cell UE camps initially) and hence is better in terms of cell quality, the UE may end up switching to that NES cell immediately. In general, a UE will consume more power if the UE selects one cell in the beginning and then switch to another cell immediately. Therefore, we think it is better to also consider Case 1 defined by RAN1 (i.e., the standalone WUS scenario) as the baseline from RAN2 perspective, as it supports the UEs in either the cell selection or the cell reselection phase.

RAN2 also consider the standalone WUS scenario (i.e., Case 1 defined by RAN1) as the baseline scenario.

One key concern against the standalone WUS scenario is how much impact the solution will have on the PBCH design as well as on the legacy UEs. If the standalone WUS scenario has any impact on the PBCH design then we should wait for RAN1’s progress on it. On the other hand, from RAN2 perspective, we may start working on the solution that has zero or at least minimal impact to PBCH, which can be worked in parallel with RAN1. For instance, one of such solutions is to provide the uplink WUS configuration in a new SIB of the NES cell, where the new SIB shares the same pdcch-ConfigSIB1 configuration as SIB1, and can be transmitted much less frequently (e.g., once per 200 ms) compared to SIB1. The new SIB is only present when SIB1 is not transmitted.

For the standalone WUS scenario (i.e., Case 1 defined by RAN1), the UL WUS configuration can be provided in a new SIB sharing the same pdcch-ConfigSIB1 as SIB1 but having longer periodicity than SIB1.

## Monitoring window for SIB1 reception

RAN1 has agreed in the RAN1#117 meeting [4] that the UE shall monitor the type 0 PDCCH monitoring occasions for the on-demand SIB1 transmission after transmitting the UL WUS, and the PDCCH monitoring occasions will last for a time window.

Agreement At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, after UE transmits the UL WUS in idle/inactive mode, RAN1 assumes following as a starting point: - Option 1: One or more type 0 PDCCH monitoring occasions for on demand SIB1 within a time window - FFS: How the search space zero configuration is provided (e.g. from searchSpaceZero in MIB or from a new search space that is indicated by UL-WUS configuration) - FFS: Details of the time window, including at least the starting time and duration - FFS: Whether/how to support transmission of on-demand SIB1 with the association with SSB(s) based on a received UL-WUS

Although RAN1 has not concluded on when to start the time window, we think it should be straightforward to start the SIB1 monitoring window only after the UE has received the RAR message acknowledging the UL WUS that the UE has transmitted earlier. However, the exact timing is still pending per RAN1 decision as it might be beneficial (in terms of UE power saving) for UE to delay the start of the SIB1 monitoring window for a few slots/symbols after the last slot/symbol of the RAR message.

UE starts a SIB1 monitoring window after receiving the RAR message acknowledging the UL WUS transmission. The exact timing on when to start the SIB1 monitoring window is up to RAN1.

From RAN2 perspective, what is more important is the UE behavior after the expiry of the SIB1 monitoring window. In our view, if UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, the UE shall consider the cell as being barred and pick the next candidate cell to evaluate the cell reselection criteria again. This is to avoid UE stranding too long in a cell that does not intend to response to the UL WUS immediately, otherwise the long waiting time will prolong the cell reselection procedure, prevent the UE from reselecting to another cell, and eventually compromise the idle/inactive mode mobility performance. This behavior also aligns with the following RAN2 agreement made in RAN2#126.

Barring for R19 NES UE - If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE considers the cell as being barred.

Moreover, we think the mobility state of a UE may have impact on how swiftly the UE shall make the cell reselection decision. Usually, if a UE has higher mobility, we would want the UE to make the cell reselection decision faster, otherwise the UE may soon leave the coverage of the original serving cell and lose it connectivity to the network. Therefore, we think the SIB1 monitoring window should cater to different UEs with different mobility states (as defined in TS 38.304), and the simplest way to make this happen is to let the UE scale the SIB1 monitoring window by a factor based on its mobility state

UE can scale the SIB1 monitoring window based on the mobility state of the UE.

## The content of the RAR

RAN2 has agreed in the RAN2#126 meeting that the UE is expected to receive the RAR responding to the UL WUS. RAN1 also made an agreement in RAN1#117 listing ‘RAR Reception’ as a required parameter/configuration inside the UL WUS configuration, which implicitly indicates the UE is expected to receive the RAR message.

RAR monitoring for MSG1 based OD-SIB1 REQ - RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

As the RAR message is merely served as an acknowledgement to the UL WUS that the UE has transmitted earlier, the contents (i.e., the temporary C-RNTI, the uplink grant, and the TAC value) within the RAR message are not really useful and hence can be ignored by the UE.

The UE ignores the TAC value, temporary C-RNTI, and uplink grant in the RAR message acknowledging a UL WUS.

Alternatively, we can adopt a similar mechanism used in confirming the completeness of the MSG1-based other SI request procedure, where the UE considers the MSG1-based other SI request procedure as being completed successfully, when the UE receives an RAPID MAC sub-header matching the PRACH preamble transmitted by the UE earlier.

UE considers the RACH procedure of a UL WUS transmission is completed successfully, upon receiving an RAPID MAC sub-header matching the PRACH preamble of the UL WUS.

## Area scope of the UL WUS configuration

RAN1 has agreed in the RAN1#117 meeting that a UL-WUS configuration can apply to either a single NES cell or multiple NES cells, and a unified configuration format is needed for supporting these two cases.

Agreement From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it. At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

Apart from the RAN1 agreement, we think it is beneficial from the network perspective to use a single UL WUS configuration across multiple cells, as it saves the configuration burdens for the network/operator and has a potential merit of reducing the signaling overheads. It is even more needed in the case when the UE is configured with the UL WUS via the RRC Release message, as the UE may move to another cell after entering the idle/inactive state.

The network can apply the same uplink WUS configuration to more than one cells.

RAN2 agreed that the PCI and frequency of a NES Cell can be used to associate a UL WUS configuration with the NES cell. That means for each UL WUS configuration there can be at least one PCI (and one frequency) being included in the UL WUS configuration.

Information of WUS configuration - Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

To extend to the cases where the same UL WUS configuration is applied to multiple cells, the simplest way is to include a PCI list containing more than one PCIs in the UL WUS configuration. However, if these cells happen to be within the same SI area, the same tracking area, or the same PLMN, there are more efficient ways to signal the UL WUS configuration. For instance, the UL WUS configuration may include an SI area ID, a TAI, or a PLMN ID within it instead of including a cell list. Alternatively, we may also index each UL WUS configuration with a configuration ID, and then associate this configuration ID to the cells listed in SIB3 or SIB4.

RAN2 to consider an efficient and unified method to signal the UL WUS configuration that can be applied to multiple cells.

## Impact on the cell reselection

It has been agreed in RAN2 that the UE should first perform the cell reselection procedure, and then request the on-demand SIB1 transmission from the cell UE picks, if the picked cell happens to be an NES cell not transmitting SIB1.

Triggering condition of WUS transmission - If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission. - After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

In the beginning of the cell reselection procedure, the UE perform the serving and neighboring cell measurement  by measuring the SSBs transmitted by the serving and neighboring cells. As the MIB payload within a SSB is able to indicate (through the ssb-SubcarrierOffset IE, pdcch-ConfigSIB1 IE, or the reserved bit) whether a cell is transmitting SIB1 or not, and RAN1 assumes the always-on SSB is transmitted on the NES cell with on-demand SIB1, the UE should be able to know whether a neighboring cell is transmitting SIB1 or not upon performing the measurement on that cell.

Agreement For further study of on-demand SIB1 in idle/inactive mode, it is assumed that always-on SSB is transmitted on the NES cell with on-demand SIB1.

UE is able to know whether a neighboring cell is transmitting SIB1 or not upon performing the measurement on the neighboring cell.

Given that the UE is able to know whether a neighboring cell is transmitting SIB1 or not while measuring the neighboring cell, reselecting to the neighboring cell having the highest ranking but not transmitting SIB1 may not be a best practice as it may introduce overhead and consume more power at the UE compared to reselecting to another cell with already-activated SIB1 transmissions. To achieve good balance among the mobility performance, the signaling overheads, and the power consumption, we may consider to introduce an NES version of the rangeToBestCell parameter, which allows UE to reselect to a non-NES cell among the non-NES cells whose R value is within rangeToBestCell of the R value of the highest ranked cell that happens to be an NES cell.

The network can configure whether the UE is allowed to reselect to a non-NES cell whose ranking is smaller than but within a ranking margin of the ranking of the best-ranked NES cell.

## Access control for legacy and Rel-19 UEs

It was also agreed in the RAN2#126 meeting that the network needs to bar the legacy UE from accessing the NES cell not transmitting SIB1, using the legacy barring mechanism. However, as the legacy UEs include the UEs capable of certain advanced features before Rel-19 (e.g., Rel-17 NTN, Rel-17 RedCap, etc.) and the barring mechanism for these UEs relies on additional barring indications resided in SIB1, using the cellBarred indication in MIB has limited effects in preventing the legacy UEs from accessing the on-demand SIB1 cell.

Legacy UE impact - NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism) - How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Using the legacy barring mechanism (i.e., cellBarred indication in MIB) has limited effects in preventing the legacy UEs from accessing the on-demand SIB1 cell.

Moreover, since now the cellBarred indication in MIB has to be set as ‘barred’ in the NES cell not transmitting SIB1, an exception is needed for the Rel-19 UE capable of on-demand SIB1 to access the cell even if the cellBarred is set as ‘barred’ in MIB. One possible way is to make these Rel-19 UEs ignore the ‘cellBarred = barred’ in MIB and check a new barring indication in SIB1 or other SIBs. However, as there is already a cellBarredNES indication in SIB1 targeting at the Rel-18 UEs supporting the NES feature, we prefer not to over-complicate the access control by introducing a new barring indication. Instead, we think a Rel-19 UE capable of the on-demand SIB1 should be allowed to request the SIB1 transmission from an NES cell with cellBarred being set as barred, as long as the UE has a valid UL WUS configuration for the cell.

Rel-19 UE is allowed to transmit an UL WUS to an NES cell as long as the UE has a valid UL WUS configuration for the NES cell, even if the cellBarred is set as barred in the NES cell.

## The provision of the neighbouring cell information

Since the idle/inactive UE may need to check the PLMN ID and certain feature-specific barring indication (e.g., cellbarredNTN-r17, cellBarredRedCap1Rx-r17, etc.) in SIB1 to determine whether the UE can camp on the cell or not, the entire cell reselection process may take much longer time if the cell the UE attempts to reselect to is an NES cell not transmitting SIB1. The worst case is that the UE may find an NES cell not suitable for cell reselection after requesting and acquiring the on-demand SIB1 from the cell.

The UE may find an NES cell not suitable for cell reselection after requesting and acquiring the on-demand SIB1 from the cell.

Therefore, we think it is beneficial if the PLMN ID and the feature-specific barring indications of an NES cell can be provided to UEs in other mean (e.g., provided through the SIB4/5 in the neighbor cells). In addition, if the UL WUS configuration can be applicable across multiple cells within a specific region (e.g., TAI, SI Area), it is better if the tracking area code (TAC) as well as the SI area identity of a NES cell can be provided to the UEs in the same way as the PLMN ID, so that a UE can know whether the UL-WUS configuration is applicable in that NES cell.

If SIB1 is not transmitted in an NES cell, the cell access related information (e.g., PLMN IDs and feature-specific barring indications) of the NES cell can be provided in the neighboring cells.

# Conclusion

In this paper, we discuss the several important issues for enabling the on-demand SIB1 mechanism based on the current progress in RAN1 and RAN2, and have the following observations.

Observation 1	Case 2 defined in RAN1 only works for the UEs in the cell reselection phase. The UEs in the cell selection phase cannot request the on-demand SIB1 transmission in Case 2.

Observation 2	The network can apply the same uplink WUS configuration to more than one cells.

Observation 3	UE is able to know whether a neighboring cell is transmitting SIB1 or not upon performing the measurement on the neighboring cell.

Observation 4	Using the legacy barring mechanism (i.e., cellBarred indication in MIB) has limited effects in preventing the legacy UEs from accessing the on-demand SIB1 cell.

Observation 5	The UE may find an NES cell not suitable for cell reselection after requesting and acquiring the on-demand SIB1 from the cell.

Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Proposal 1	RAN2 also consider the standalone WUS scenario (i.e., Case 1 defined by RAN1) as the baseline scenario.

Proposal 2	For the standalone WUS scenario (i.e., Case 1 defined by RAN1), the UL WUS configuration can be provided in a new SIB sharing the same pdcch-ConfigSIB1 as SIB1 but having longer periodicity than SIB1.

Proposal 3	UE starts a SIB1 monitoring window after receiving the RAR message acknowledging the UL WUS transmission. The exact timing on when to start the SIB1 monitoring window is up to RAN1.

Proposal 4	If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE considers the cell as being barred.

Proposal 5	UE can scale the SIB1 monitoring window based on the mobility state of the UE.

Proposal 6	The UE ignores the TAC value, temporary C-RNTI, and uplink grant in the RAR message acknowledging a UL WUS.

Proposal 7	UE considers the RACH procedure of a UL WUS transmission is completed successfully, upon receiving an RAPID MAC sub-header matching the PRACH preamble of the UL WUS.

Proposal 8	RAN2 to consider an efficient and unified method to signal the UL WUS configuration that can be applied to multiple cells.

Proposal 9	The network can configure whether the UE is allowed to reselect to a non-NES cell whose ranking is smaller than but within a ranking margin of the ranking of the best-ranked NES cell.

Proposal 10	Rel-19 UE is allowed to transmit an UL WUS to an NES cell as long as the UE has a valid UL WUS configuration for the NES cell, even if the cellBarred is set as barred in the NES cell.

Proposal 11	If SIB1 is not transmitted in an NES cell, the cell access related information (e.g., PLMN IDs and feature-specific barring indications) of the NES cell can be provided in the neighboring cells.

# Reference

RP-234065, New WID: Enhancements of network energy savings for NR, Ericsson.

R2-2404102, Report of 3GPP TSG RAN WG2 meeting #125bis, ETSI MCC.

Draft report of 3GPP TSG RAN WG2 meeting #126, ETSI MCC.

Draft Report of 3GPP TSG RAN WG1 meeting #117, ETSI MCC.