3GPP TSG-RAN WG2 Meeting#127                                	R2-2406570

Maastricht, Netherlands, August 19th – 23th, 2024

Agenda item:	8.9.2

Source: 	Google

Title: 	Discussion on the S&F indication

Document for:	Discussion and Decision

# Introduction

In Rel-19, the benefits of enabling the Store & Forward operation for IoT-NTN are identified and the following scopes are included in the WID of Rel-19 IoT-NTN [1].

Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]. - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

The Rel-19 IoT-NTN discussion was kicked off in the RAN2#125bits meeting, and some progress was made in RAN2#126 meeting, with regard to the S&F indication provided by the network [2].

RAN2 Agreements in RAN2#126: - S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed - RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN.

In this paper, we discuss several possible designs regarding the S&F indication based on the current progress, and also the interaction between the RRC and NAS layer for supporting the S&F operation. Based on the discussion, several way forwards are proposed in this paper to further push the progress on this important feature.

# Discussion

## Access control for the S&F operation

It was agreed in the RAN2#126 meeting that the IoT-NTN network shall be able to indicate UEs whether S&F satellite operation is applied, and such an indication is only provided through the AS layer. It was also agreed that the network can prevent the legacy UEs from accessing the cell operating in the S&F operation mode, by using the legacy cellBarred and cellBarredNTN bits in MIB and SIB1, respectively. This implies that the legacy cellBarred and cellBarredNTN bits can be used to indicate the S&F status of a cell, and the Rel-19 UEs capable of the S&F operation can still access the cell even if both the cellBarred and cellBarredNTN bits are set as barred in the cell. However, to support the scenario where the network may want to avoid all types of NTN access due to the cell overload, an additional indication needs to be signalled when both cellBarred and cellBarredNTN bits are set as barred. The purpose of that indication is to at least indicate whether the Rel-19 UEs capable of the S&F operation are allowed to access the cell or not. Here the UE capable of the S&F operation means the UE that is able to deal with the long break occurring in a transaction due to the temporary unavailability of the feeder link.

The S&F indication is present only when both legacy cellBarred and cellBarredNTN are set to barred. As the baseline, it indicates whether the Rel-19 UEs capable of the S&F operation are allowed to access the cell or not.

To facilitate the access control for the Rel-19 UE, it is beneficial if the S&F indication can also indicate the feeder-link status, so that the Rel-19 UE not capable of the S&F operation may still be allowed to access the cell, at least when the feeder link is available. The feeder-link status may further indicates the remaining time of the feeder-link when the feeder-link is currently available, so that the Rel-19 UE (not capable of the S&F operation) can determine whether to access the cell or not, based on how long the feeder-link will remain available.

The S&F indication can be used to indicate the feeder-link status, such as the availability or the remaining time of the feeder-link.

If Proposal 1 and Proposal 2 above can be agreed, the access control for the UE capable of the S&F operation can depend on whether the S&F operation is present or not. More specifically, the UE capable of the S&F operation is not allowed to access a cell if the S&F indication is not present and both legacy cellBarred and cellBarredNTN are set to barred in that cell; the UE is allowed to access the cell otherwise.

If the S&F indication is not present and both legacy cellBarred and cellBarredNTN are set to barred, the cell is considered as barred for NTN access.

If the S&F indication is present, the UE capable of the S&F operation is allowed to access the cell.

For the Rel-19 UE not capable of the S&F operation, if the S&F indication is present (and hence both the cellBarred and cellBarredNTN are set to barred), the access control would further depend on the feeder-link availability or even the remaining time of the feeder-link. Note that even if the feeder-link status indicates the feeder-link is not available, we think the Rel-19 UE not capable of the S&F operation can still access the cell in certain circumstances (e.g., EDT transmission or being paged for DL data reception).

If the S&F indication is present, the Rel-19 UE not capable of the S&F operation may be allowed to access the cell, depending on the feeder-link status.

We may also consider other options such as the option to simplify the S&F indication design and leave the access control to the network. That is, the S&F indication simply controls whether the Rel-19 UE (regardless of the S&F capability) can access the cell or not, and then the network determines whether to grant the access for each UE individually based on their S&F capabilities reported during the access attempt. For instance, the network can reject the access attempt from the UE not capable of the S&F operation if the feeder-link will not last for a long time. To facilitate such network decisions, we can make Rel-19 UEs inform the eNB of their S&F capabilities at the earliest timing during the initial access (i.e., via MSG3).

Rel-19 UE can inform eNB of its S&F capability via MSG3.

## The suspension of UE’s RRC and NAS

While connecting to a cell operating in the satellite S&F mode with no available feeder link, the UE may first establish its RRC connection with the cell and then initiate a NAS procedure by transmitting a NAS message. For the baseline scenario where only the full eNB is onboard, it would take quite some time for the UE to receive the response to the NAS message, most likely after the UE has become out of coverage of the connected satellite and returned to the coverage again of the satellite carrying the NAS response. Therefore, during the lifetime of a NAS transaction, the UE may have moved between different RRC states multiple times. As these transitions between different RRC states may clear/reset the NAS progress especially if the UE has moved to RRC_IDLE once, we should avoid UE moving to the idle state (RRC_IDLE as well as ECM_IDLE) whenever the feeder link or the service link becomes unavailable if there is an on-going NAS procedure. For this purpose, when the feeder is or becomes not available, eNB can suspend UE’s RRC connection by sending an RRC Connection Release message to the UE supporting the S&F satellite operation, where the RRC Connection Release message may include an rrc-InactiveConfig IE or set the releaseCause as ‘rrc-Suspend’. Upon receiving the RRC Connection Release message, the UE RRC indicates the suspension of the RRC to the NAS layer, which may result in the suspension to the NAS layer as well.

Since the existing RRC_INACTIVE state and RRC_IDLE state with RRC suspension is only applicable to the UE that has already attached/registered to a network, it can be further discussed if a new RRC suspension state/mode is needed (i.e., a new release cause is needed) to support the initial ATTACH scenario.

When the feeder-link is not available, eNB can suspend UE’s RRC and NAS by transmitting an RRC Connection Release message to the UE capable of the S&F satellite operation. FFS whether a new release cause is needed.

# Conclusion

In this paper, we discuss the impact of the S&F satellite operation on the UE RRC and NAS, and respectfully ask RAN2 to discuss and consider the following proposals.

Proposal 1	The S&F indication is present only when both legacy cellBarred and cellBarredNTN are set to barred. As the baseline, it indicates whether the Rel-19 UEs capable of the S&F operation are allowed to access the cell or not.

Proposal 2	The S&F indication can be used to indicate the feeder-link status, such as the availability or the remaining time of the feeder-link.

Proposal 3	If the S&F indication is not present and both legacy cellBarred and cellBarredNTN are set to barred, the cell is considered as barred for NTN access.

Proposal 4	If the S&F indication is present, the UE capable of the S&F operation is allowed to access the cell.

Proposal 5	If the S&F indication is present, the Rel-19 UE not capable of the S&F operation may be allowed to access the cell, depending on the feeder-link status.

Proposal 6	Rel-19 UE can inform eNB of its S&F capability via MSG3.

Proposal 7	When the feeder-link is not available, eNB can suspend UE’s RRC and NAS by transmitting an RRC Connection Release message to the UE capable of the S&F satellite operation. FFS whether a new release cause is needed.

# Reference

RP-234007, New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

Draft report of 3GPP TSG RAN WG2 meeting #126, ETSI MCC.