3GPP TSG-RAN WG2 Meeting#127                                	R2-2406571

Maastricht, Netherlands, August 19th – 23th, 2024

Agenda item:	8.8.2

Source: 	Google

Title: 	Discussion on the DL coverage enhancement

Document for:	Discussion and Decision

# Introduction

One key research area (i.e., the area for improvement) in the Rel-19 NR-NTN (i.e., NR_NTN_Ph3) is the downlink coverage enhancement, where the enhancement can be considered from both the link level and the system level perspectives, as described in the following Rel-19 NR-NTN WID [1].

DL coverage enhancements can be considered at both - Link level to improve the link margin of selected physical channels in order to accommodate the EIRP reduction in FR1-NTN. A link margin improvement for physical channels (e.g. PDSCH and PDCCH) may be considered without impact on SSB design. - System level to support an efficient dynamic and flexible power sharing between beams or different beam pattern/size (i.e., wide or narrow) across the satellite foot print for FR1-NTN and FR2-NTN.

The Rel-19 NR-NTN discussion was kicked off in the RAN2#125bits meeting, and some progress was made in RAN2#126 meeting [2] with regard to the DL coverage enhancement.

RAN2 Agreements in RAN2#126: - Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements. - RAN2 assumes that both EFC and EMC are supported

In this paper, we discuss several key issues of the cell/beam DTX configuration from the signaling perspective, and the impact of the signaling to the UEs in different RRC states. Based on the discussion, several way forwards are proposed in this paper to facilitate the discussion in RAN2 while waiting for more inputs from RAN1.

# Discussion

RAN1 is currently investigating the effect of increasing the SSB periodicity (as a consequence of introducing the beam hopping mechanism) on improving the coverage ratio, and has the following observations.

Observation Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km: - For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied - For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied. - Note: coverage ratio is N2+N3/ total beam footprints - Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs.

As the longest SSB periodicity the currently specification can support is up to 160ms, increasing the SSB periodicity to 320ms definitely will have impact to the legacy UE, since the legacy UE may not detect the SSB at the time when the UE expects to receive it. Even if the SSB periodicity is not extended beyond the legacy value 160ms, the introduction of the N1 beam footprints (i.e., the number of the beam footprints not providing service) will also have impact to the legacy UE in the idle/inactive state, as the camped cell may not transmit anything to the legacy UE at the time when the UE wakes up at its paging occasions. One may argue that the network can configure the legacy UE with a paging cycle and a paging frame offset that allow the UE to wake up only at the time when the cell/beam the UE is camping on is not in the ‘OFF’ state. However, as the paging frames are evenly distributed within a paging cycle and the UE will be monitoring a paging frame based on the UE ID, we are not sure if it is always possible to make UE only wake up at its paging occasions.

Extending the SSB periodicity and/or introducing the N1 beam footprints (i.e., the number of the beam footprints not providing service) may have impact to the legacy UE, as when the legacy UE wakes up at its paging occasions, it may not detect anything including the SSB.

The WID pointed out that the Rel-18 Network Energy Saving (NES) techniques should be considered as the baseline for signaling the beam-hopping pattern. However, the cell DTX/DRX configuration in Rel-18 NES cannot be directly employed here to signal the beam-hopping pattern, as the cell DTX/DRX configuration in Rel-18 NES is only applicable to the UE in the connected state, and the cell will still transmit common control signals even in the non-active period of a DTX cycle. As the beam-hopping mechanism may impact the legacy UE as well as the Rel-19 UE in the idle/inactive state, the cell DTX/DRX configuration shall be also applicable to the UEs in the idle/inactive state, and shall cover the state where the satellite beam does not transmit anything. Therefore, we think the beam-hopping pattern (i.e., the cell or beam DTX configurations) can be provided in the system information, so that at least the Rel-19 UE is able to determine when to wake up and monitor the paging message based on the beam-hopping pattern and the paging configuration.

The beam/cell DTX configuration can be provided in the system information for idle/inactive UEs, and covers the Tx state where the beam/cell is not transmitting anything.

Regarding whether the DTX configuration is provided to the UE on a per-beam or a per-cell basis, we think this is relevant to how the network/operator maps the satellite beams into an NR beam or an NR cell. According to R1-2400303 [3], there are five possible options for mapping the satellite beams into an NR beam or NR cell.

- Single NR cell per satellite beam and single NR beam cell

- Multi-Satellite beam NR cell and single NR beam cell: Cell splitting

- Multi-Satellite beam cell and multi NR beam cell

- Multi NR cells per satellite beam

- Multi NR cells split across satellite beams

If the DTX configuration is provided on a per-cell basis, it works for Option 1 and Option 4 above, but does not work for the other options. On the other hand, if the DTX configuration is provided on a per-beam basis, it works for all the options except for Option 2, and may result in some duplicated signaling in Option 4 (i.e., not optimized for Option 4). In fact, the cell DTX configuration can be treated as a special case of the beam DTX configuration, in which all the NR beams within the same cell share the same beam DTX configurations.

Providing beam DTX configuration can support most of the mapping options between the satellite beam and the NR beam/cell, and the cell DTX configuration can be considered as a special case of the beam DTX configure.

Therefore, we think the provision of the DTX configuration on a per-beam basis can be considered as the baseline, where each DTX configuration can be associated to an SSB index. It can be further discussed whether the DTX configuration can be associated to a CSI-RS resource ID.

The DTX configuration can be a beam-specific configuration, and can be associated to an SSB index.

To address the issue of having duplicated DTX configurations while providing the beam-specific DTX configuration in Option 4, one possible handling is to provide a DTX configuration but not associate it with any beam index. The UE would consider such a DTX configuration (not associated with any beam index) begin applicable to all the beams within the same cell.

If a DTX configuration is not associated with an SSB index, it is applicable for the entire cell.

When the UE is in the connected state and the serving beam or the serving cell is under the ‘off’ state (i.e., the state when the beam or cell is not transmitting anything), the UE is not required to monitor the PDCCH for saving energy. In order to further save the UE power and to prevent the UE from declaring the RLF or the beam failure while the beam/cell is not transmitting anything, we think the connected UE should refrain itself from performing the RLM and BFD, and may also refrain from performing the serving cell measurement during the beam/cell off periods.

The connected UE is not required to monitor the PDCCH, perform the RLM/BFD, and perform the serving cell measurement during the ‘off’ periods configured by the cell/beam DTX configuration.

As adopting P4 above may have some impact to the specification, an alternative approach to introduce minimum specification impact is to rely on the c-DRX configuration. Since the UE being configured with c-DRX would comply with the timing requirement (for measurement, RLM, and BFD) that is based on the c-DRX cycle length, the UE is able to avoid monitoring/measuring the RS during the beam/cell ‘off’ period if every c-DRX on-duration would fall within or overlap with the DTX active duration. To ensure the c-DRX on-duration would align with the DTX active duration, a stage-2 description similar to the one for the Rel-18 NES might be needed. That is, the UE in the connected state is expected to receive a c-DRX configuration whose the cycle length is equal to one or multiple times of the cell/beam DTX cycle length.

The connected UE is expected to receive a c-DRX configuration whose the cycle length is equal to one or multiple times of the cell/beam DTX cycle length.

# Conclusion

In this paper, we discussed several key issues regarding the signaling of the cell/beam DTX configuration, and have the following observations.

Observation 1	Extending the SSB periodicity and/or introducing the N1 beam footprints (i.e., the number of the beam footprints not providing service) may have impact to the legacy UE, as when the legacy UE wakes up at its paging occasions, it may not detect anything including the SSB.

Observation 2	Providing beam DTX configuration can support most of the mapping options between the satellite beam and the NR beam/cell, and the cell DTX configuration can be considered as a special case of the beam DTX configure.

Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Proposal 1	The beam/cell DTX configuration can be provided in the system information for idle/inactive UEs, and covers the Tx state where the beam/cell is not transmitting anything.

Proposal 2	The DTX configuration can be a beam-specific configuration, and can be associated to an SSB index.

Proposal 3	If a DTX configuration is not associated with an SSB index, it is applicable for the entire cell.

Proposal 4	The connected UE is not required to monitor the PDCCH, perform the RLM/BFD, and perform the serving cell measurement during the ‘off’ periods configured by the cell/beam DTX configuration.

Proposal 5	The connected UE is expected to receive a c-DRX configuration whose the cycle length is equal to one or multiple times of the cell/beam DTX cycle length.

# Reference

RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

Draft report of 3GPP TSG RAN WG2 meeting #126, ETSI MCC.

R1-2400303, Discussion on NR NTN Downlink coverage enhancements, Thales.