3GPP TSG-RAN WG2 Meeting #127                              R2-2406573

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.1.3

Source:	Mediatek Inc.

Title:	Enhanced MDT for Data collection for Network Side Model Training

Document for:	Discussion, Decision

# Introduction

In RAN2#125bis meeting, following agreements were made regarding the data collection of network-side model training.

Agreements

1	For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches

2	We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

3	RAN2 supports enhancements to MDT for data collection framework for training.  FSS Whether to enhance logged or immediate MDT

Subsequently, in the RAN2#126 meeting, further agreements were reached specifically for network-side training data collection pertaining to the use case of AI BM.

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting

This paper aims to extend the discussion by exploring the MDT mechanisms for data collection tailored to network-side model training, in alignment with the established principles.

# Discussion

## 2.1 Immediate MDT Enhancement

### 2.1.1 Rational

Current immediate MDT involves the real-time collection and transmission of measurement data from the UE to the network, i.e.  the UE collects and immediately reports measurement data to the network. The real-time data provided by Immediate MDT is used by network operators to quickly identify and resolve issues, optimize network performance, and enhance user experience. It is particularly useful for addressing problems in scenarios where rapid response is crucial, such as in the case of sudden network degradation or to address specific quality of service issues.

Since immediate MDT involves real-time reporting, it can have a greater impact on UE battery life and network resources. Therefore, its use is typically more targeted and limited to specific needs or troubleshooting efforts.

Observation 1: Immediate MDT is characterized by its real-time collection and prompt reporting of UE measurement data to the network for quick issue resolution and network optimization. However, Nonetheless, its persistent operation could affect the battery life of the UE and the utilization of network resources.

### 2.1.2 Decouple data collecting/logging and reporting in time

For all forms of offline model training, including UE-centric, network-centric, and two-sided model training, there is no specific latency requirement for the data collection process. This process typically involves distinct phases such as measurement, data generation, and reporting, with the purpose of transmitting the collected data to a different entity, such as from the UE to RAN/OAM or LMF. This indicates that latency is not a critical factor for any stage of the data collection process. Utilizing immediate MDT for data collection in offline training scenarios is overkilled in terms of latency needs and can lead to unnecessary power consumption by the UE.

Observation 2: Utilizing immediate MDT for data collection for purpose of offline training is overkilled in terms of latency needs and can lead to unnecessary power consumption by the UE.

However, it was agreed in last RAN2 meeting that immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model. Furthermore, reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. How to enhance immediate MDT to support data logging considering the limitation in UE memory, processing power, energy consumption and signalling overhead needs to be considered.

From both UE and network point of view, it is beneficial to have the flexibility to decouple data collecting and reporting in time intervals. For example, UE can send the collected data when the network is less congested. Considering that data collecting can be an intrusive mode that may have very significant impact on UE power consumption, it is desired that UE performs data collecting/logging in high power mode. In our view decouple of data collecting/logging and reporting can be important at least for the following reasons:

- Resource Efficiency: Decoupling data collecting/logging and reporting allows the UE to collect data/log measurements without using uplink resources immediately. This is particularly useful in scenarios where network coverage is poor, or the UE is in a power-saving mode.

- Network Load Management: If all UEs were to report measurements immediately, it could lead to congestion in the network. Decoupling data collecting/logging and reporting allows the network to control when the UEs send the collected data/logged information, thus managing the network load more effectively.

- Signaling process and Power consumption: Decoupling data collection/logging from reporting minimizes the need for frequent and immediate signaling interactions with the network, thereby reducing the UE's time in high power-consuming states. The UE can choose to report data when it has a high battery level, enhancing battery efficiency and conserving power by avoiding continuous reporting.

- Data Volume: data collection can generate a large volume of data. Decoupling data collecting/logging and reporting allows the UE to store the data and transmit it in an efficient manner, possibly using compression or when the UE has access to better network conditions.

Observation 3: Decoupling data collecting/logging and reporting has following benefits:

- to enhance resource efficiency by allowing UEs to collect and log data without immediately consuming uplink resources.

- to manage network load more effectively by controlling when UEs send collected data.

- to minimize the need for immediate reporting and conserve power consumption by enabling UEs to log data and report it later, avoiding continuous reporting that can drain the battery faster.

- to handle large volumes of data more efficiently by allowing UEs to store and transmit data in a more efficient manner.

Proposal 1: Enhance immediate MDT by decoupling the time intervals for data collection/logging and reporting, in order to limit the impact on the UE signaling processing, battery life and network load.

Figure 1. Enhance immediate MDT by decoupling data collection and reporting

### 2.1.3 UE measurement

In current immediate MDT mechanism, the UE follows the RRM measurement behavior, i.e., the UE starts to collect measurement data upon reception of the measurement configuration. There is no specific mechanism to stop collecting data. UE stops data collecting due to measurement reconfiguration or change of the RRC state from RRC CONNECTED to RRC INACTIVE/IDLE.

Observation 4: In current immediate MDT mechanism, the UE starts data collection upon reception of the measurement configuration. The UE stops data collection due to measurement reconfiguration or change of RRC state.

According to RAN2 agreement, the UE can report multiple instances of logged L1 measurement result via a RRC message from UE to gNB. In order to support decoupling of L1 measurement logging and reporting in time intervals, the UE needs to store the logged L1 measurement results for future reporting. Given the substantial volume of data to be collected and the need to accommodate various use cases, it is required to specify the UE memory allocation for data collection and logging. It remains to be FFS whether the UE memory allocation will be defined as a total size encompassing all use cases or allocated individually per use case.

Proposal 2: The UE memory size needs to be specified for data collecting/logging. FFS on the exact value and whether the UE memory size is defined as a total size or defined individually per use case.

As a baseline procedure, the UE initiates the collection and logging of L1 measurement results once it receives the measurement configuration for AI/ML-enabled features/FGs. However, additional criteria for starting and stopping data collection and logging needs to be considered, taking into account the UE's memory capacity and power constraints. For instance, the UE may stop data collection if its memory reaches capacity or if it enters a low power state, given the significant memory and power demands of the data collection process.

Furthermore, to ensure the collected data is of high quality and accuracy for model training, another criterion could involve the UE performing data collection when link quality is high and stopping when it deteriorates. Link quality assessment may rely on L1/3 RSRP measurements or the status of the T310 timer. These criteria are important for optimizing the balance between data quality and efficient resource utilization within the UE.

Proposal 3: As the baseline approach, the UE initiates the collection and logging of L1 measurement results once it receives the measurement configuration for AI/ML-enabled features/FGs.

Proposal 4: Other criteria to control UE data collection and logging for model training are considered, including the UE memory, power state and link quality.

### 2.1.4 UE reporting

In current immediate MDT mechanism, UE measurement reporting to the gNB is either event-triggered or periodic. However, as suggested in observation 3 and proposal 1, the network can also initiate reporting through on-demand requests. This flexibility for the network to determine the timing for data retrieval from the UE can improve network efficiency and user experience by effectively utilizing resources, balancing network traffic, preserving UE battery, and managing data volumes with strategically timed transmissions. Therefore, in the on-demand request mechanism, the network can send request to the UE and retrieve the stored data/logs.

As a starting point, data retrieval occurs within the same RRC connection as the UE's data logging, meaning the UE discards stored data/logs once the RRC connection is released. However, a pertinent question arises regarding the flexibility of data retrieval across different RRC connections. For instance, the network might request the UE to transmit stored logs/data during off-peak hours when network traffic is low. In such scenarios, the UE would need to preserve the logs/data even after the RRC connection is released and transmit the reports upon later retrieval by the network.

Figure 2. Enhance immediate MDT by decoupling data collection and reporting (cross different RRC connections)

Observation 5: Reporting triggered by on-demand requests provides the network the flexibility to determine the timing for data retrieval from the UE, thereby improving network efficiency, conserving UE battery and improving user experience.

Proposal 5: Data reporting can be triggered by an on-demand request from the network, i.e., the UE is asked by the network to send the collected L1 measurement logs via RRC signaling.

Proposal 6: As the baseline approach, data retrieval occurs within the same RRC connection as the UE's data logging, and the UE discards stored data/logs once the RRC connection is released. FFS whether data retrieval cross different RRC connection then the one used for data logging is supported or not.

## 2.2 Principles of data collection for network-side model training

Following the completion of the Rel-18 SI on AI/ML for the NR Air Interface, a set of principles for data collection for network-side model training has been established and documented in the TR [1].

A set of general data collection principles is expected to be considered for network-side model training. These include: - UE to support data logging, - UE to report the collected data periodically, event-based, and on-demand, - The UE memory, processing power, energy consumption, signalling overhead should be considered. Note: The above principles can be revised depending on RAN1 requirements.

The principles outlined for network-side model training are equally applicable to UE-side model training. Data collection for UE-side model training also needs to consider the support of data logging, flexible data reporting, resource efficiency, UE energy consumption, UE processing power, UE memory, etc. The most important benefit to apply the same set of principle is to have unified data collection solution over the air interface. Applying the same data collection principles to both UE-side and network-side model training creates a standardized approach， which facilitates easier design, implementation and maintenance of data collection procedures, leading to better compatibility and less complexity in both UE and NW side.

If we have common solution over the air interface for data collection for network-side model training and UE-side model training, the only problem needs to be addressed for data collection for UE-side model training is how to transfer the data collected by the network to the UE-side server.

Proposal 7: The general data collection principles for network side model training is applicable to UE-side model training. Common solution over the air interface for data collection for both UE-side model training and network-side model training is preferred.

## 2.2 Data Size

The baseline method for data collection and logs reporting is currently CP-based, relying on RRC messages for the transmission of collected data/logs. This approach is deemed suitable when the data size per sample remains within the maximum capacity that RRC messages can accommodate without necessitating segmentation. However, for larger data samples that exceed this threshold, RRC message segmentation is required, which can lead to inefficiencies due to the complexity and resource demands of handling multiple segments. For example, in AI-based Beam Management (AI-BM), the data size may vary between roughly 100 to 500 bits based on the number of beams being reported. Similarly, in AI-based Positioning (AI-POS), the measurement size can escalate to 4096*N bits, with N denoting the count of PRS/SRS resources engaged for positioning. Therefore, CP-based data/logs reporting via RRC messages is considered the baseline, provided the data size is within the current RRC message handling capabilities. However, it’s important to understand data size requirement and evaluate whether RRC message is sufficient for data/logs reporting. The the issue is the common for both network-side data collection and UE-side data collection. RAN2 should seek input from RAN1 regarding data volume to evaluate the sufficiency of RRC signaling for the task. For scenarios where the data volume exceeds what RRC can handle, RAN2 should engage SA2/SA5 to investigate the feasibility of utilizing a UP tunnel to facilitate data transfer.

Proposal 8: RAN2 should consult RAN1 on the data volume for data collection for purpose of model training to evaluate whether it exceeds RRC signaling capacity.

# Conclusion

Observations:

Observation 1: Immediate MDT is characterized by its real-time collection and prompt reporting of UE measurement data to the network for quick issue resolution and network optimization. However, Nonetheless, its persistent operation could affect the battery life of the UE and the utilization of network resources.

Observation 2: Utilizing immediate MDT for data collection for purpose of offline training is overkilled in terms of latency needs and can lead to unnecessary power consumption by the UE.

Observation 3: Decoupling data collecting/logging and reporting has following benefits:

- to enhance resource efficiency by allowing UEs to collect and log data without immediately consuming uplink resources.

- to manage network load more effectively by controlling when UEs send collected data.

- to minimize the need for immediate reporting and conserve power consumption by enabling UEs to log data and report it later, avoiding continuous reporting that can drain the battery faster.

- to handle large volumes of data more efficiently by allowing UEs to store and transmit data in a more efficient manner.

Observation 4: In current immediate MDT mechanism, the UE starts data collection upon reception of the measurement configuration. The UE stops data collection due to measurement reconfiguration or change of RRC state.

Observation 5: Reporting triggered by on-demand requests provides the network the flexibility to determine the timing for data retrieval from the UE, thereby improving network efficiency, conserving UE battery and improving user experience.

Proposals:

Decouple data collecting/logging and reporting in time

Proposal 1: Enhance immediate MDT by decoupling the time intervals for data collection/logging and reporting, in order to limit the impact on the UE signaling processing, battery life and network load.

UE measurement

Proposal 2: The UE memory size needs to be specified for data collecting/logging. FFS on the exact value and whether the UE memory size is defined as a total size or defined individually per use case.

Proposal 3: As the baseline approach, the UE initiates the collection and logging of L1 measurement results once it receives the measurement configuration for AI/ML-enabled features/FGs.

Proposal 4: Other criteria to control UE data collection and logging for model training are considered, including the UE memory, power state and link quality.

UE reporting

Proposal 5: Data reporting can be triggered by an on-demand request from the network, i.e., the UE is asked by the network to send the collected L1 measurement logs via RRC signaling.

Proposal 6: As the baseline approach, data retrieval occurs within the same RRC connection as the UE's data logging, and the UE discards stored data/logs once the RRC connection is released. FFS whether data retrieval cross different RRC connection then the one used for data logging is supported or not.

Common Principles

Proposal 7: The general data collection principles for network side model training is applicable to UE-side model training. Common solution over the air interface for data collection for both UE-side model training and network-side model training is preferred.

Data size

Proposal 8: RAN2 should consult RAN1 on the data volume for data collection for purpose of model training to evaluate whether it exceeds RRC signaling capacity.