3GPP TSG-RAN WG2 Meeting #127                                                      R2-2406574

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | draft | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on simultaneousU-TCI-UpdateListx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2024-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN2#124 meeting, RAN2 received the reply LS from RAN1 (R2-23113910), for the issue (Question 2b as below) about simultaneous unified TCI state update, RAN1 gived the answer as following. Question 2b: Is there any restrictions in configuring the serving cells of one list for sDCI mTRP, mDCI mTRP or sTRP operation? Answer 2b: It has been agreed in RAN1 that cells operated as sDCI mTRP, mDCI mTRP and sTRP should not be configured in the same list, i.e., they should be configured with separate lists for simultaneous unified TCI state(s) update. Other than this, there is no further/other restriction(s).. Based on the above answer, RAN2 modified the field description of simultaneousU-TCI-UpdateListx as follow. simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType. Network should not configure serving cells that are configured with a BWP with different number of coresetPoolIndexes in these lists.. However, the description highlighted in yellow is ambiguous, for example, it can have following several understandings: - a serving cell is configured with a BWP with different number of coresetPoolIndexes compared with other BWP in this cell should not be included in these lists; - serving cells with different number of coresetPoolIndexes compared with other serving cells should not be included in these lists, i.e., the serving cells in these lists have the same number of coresetPoolIndexes; - serving cells with different number of coresetPoolIndexes should not be included in the same list, the serving cells in different lists can have different number of coresetPoolIndexes; Therefore, modification is needed to clarify these understandings. Based on the answer 2b (highlighted in green above) in the reply LS, RAN1 has agreed that cells operated as sDCI mTRP, mDCI mTRP and sTRP should not be configured in the same list, i.e., they should be configured with separate lists for simultaneous unified TCI state(s) update, the understanding 3) seems more suitable. Furthermore, based on understanding 3), only the mDCI mTRP can be excluded from the case of sDCI mTRP and sTRP. But it couldn’t separet the sDCI mTRP and sTRP. |
| Summary of change: |  | Clarify in the field description of simultaneousU-TCI-UpdateListx that cells operated as sDCI mTRP, mDCI mTRP and sTRP should be configured in separate lists for simultaneous unified TCI state(s) update. Impact analysis Architecture options NR SA, NR-DC Impacted functionality: Unified TCI framework extension for multi-TRP Inter-operability: If only the network is implemented according to the CR and the UE is not, no interoperability problems are foreseen. If only the UE is implemented according to the CR and the network is not, the network may configure the simultaneousU-TCI-UpdateListx incorrectly, which can cause the error when UE updates the TCI state(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous exists in current spec for simultaneousU-TCI-UpdateListx configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First of change

6.3.2	Radio resource control information elements

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells). For an NCR-MT, the CellGroupConfig IE is also used to provide the configuration of side control information for the NCR-Fwd access link.

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                        SEQUENCE {

cellGroupId                                CellGroupId,

rlc-BearerToAddModList                     SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig                        OPTIONAL,   -- Need N

rlc-BearerToReleaseList                    SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity                  OPTIONAL,   -- Need N

mac-CellGroupConfig                        MAC-CellGroupConfig                                                     OPTIONAL,   -- Need M

physicalCellGroupConfig                    PhysicalCellGroupConfig                                                 OPTIONAL,   -- Need M

spCellConfig                               SpCellConfig                                                            OPTIONAL,   -- Need M

sCellToAddModList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig                       OPTIONAL,   -- Need N

sCellToReleaseList                         SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex                        OPTIONAL,   -- Need N

...,

[[

reportUplinkTxDirectCurrent                ENUMERATED {true}                                                   OPTIONAL    -- Cond BWP-Reconfig

]],

[[

bap-Address-r16                            BIT STRING (SIZE (10))                                                  OPTIONAL,   -- Need M

bh-RLC-ChannelToAddModList-r16             SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelConfig-r16 OPTIONAL,   -- Need N

bh-RLC-ChannelToReleaseList-r16            SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelID-r16     OPTIONAL,   -- Need N

f1c-TransferPath-r16                       ENUMERATED {lte, nr, both}                                              OPTIONAL,   -- Need M

simultaneousTCI-UpdateList1-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousTCI-UpdateList2-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList1-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList2-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

uplinkTxSwitchingOption-r16                ENUMERATED {switchedUL, dualUL}                                         OPTIONAL,   -- Need R

uplinkTxSwitchingPowerBoosting-r16         ENUMERATED {enabled}                                                    OPTIONAL    -- Need R

]],

[[

reportUplinkTxDirectCurrentTwoCarrier-r16  ENUMERATED {true}                                                       OPTIONAL    -- Need N

]],

[[

f1c-TransferPathNRDC-r17                   ENUMERATED {mcg, scg, both}                                             OPTIONAL,   -- Need M

uplinkTxSwitching-2T-Mode-r17              ENUMERATED {enabled}                                                    OPTIONAL,   -- Cond 2Tx

uplinkTxSwitching-DualUL-TxState-r17       ENUMERATED {oneT, twoT}                                                 OPTIONAL,   -- Cond 2Tx

uu-RelayRLC-ChannelToAddModList-r17        SEQUENCE (SIZE(1..maxUu-RelayRLC-ChannelID-r17)) OF Uu-RelayRLC-ChannelConfig-r17

OPTIONAL,   -- Need N

uu-RelayRLC-ChannelToReleaseList-r17       SEQUENCE (SIZE(1..maxUu-RelayRLC-ChannelID-r17)) OF Uu-RelayRLC-ChannelID-r17

OPTIONAL,   -- Need N

simultaneousU-TCI-UpdateList1-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList2-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList3-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousU-TCI-UpdateList4-r17          SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

rlc-BearerToReleaseListExt-r17             SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentityExt-r17           OPTIONAL,   -- Need N

iab-ResourceConfigToAddModList-r17  SEQUENCE (SIZE(1..maxNrofIABResourceConfig-r17)) OF IAB-ResourceConfig-r17   OPTIONAL, -- Need N

iab-ResourceConfigToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofIABResourceConfig-r17)) OF IAB-ResourceConfigID-r17 OPTIONAL  -- Need N

]],

[[

reportUplinkTxDirectCurrentMoreCarrier-r17 ReportUplinkTxDirectCurrentMoreCarrier-r17                            OPTIONAL  -- Need N

]],

[[

prioSCellPRACH-OverSP-PeriodicSRS-r17      ENUMERATED {enabled}                                                  OPTIONAL  -- Need R

]],

[[

ncr-FwdConfig-r18                          SetupRelease { NCR-FwdConfig-r18 }                                 OPTIONAL,  -- Cond NCR

autonomousDenialParameters-r18             SetupRelease {AutonomousDenialParameters-r18}                      OPTIONAL,   -- Need M

nonCollocatedTypeMRDC-r18                  ENUMERATED { true }                                                OPTIONAL,   -- Need R

nonCollocatedTypeNR-CA-r18                 ENUMERATED { true }                                                OPTIONAL,   -- Need R

uplinkTxSwitchingMoreBands-r18             SetupRelease { UplinkTxSwitchingMoreBands-r18 }                    OPTIONAL    -- Need M

]]

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=                        SEQUENCE {

servCellIndex                       ServCellIndex                                               OPTIONAL,   -- Cond SCG

reconfigurationWithSync             ReconfigurationWithSync                                     OPTIONAL,   -- Cond ReconfWithSync

rlf-TimersAndConstants              SetupRelease { RLF-TimersAndConstants }                     OPTIONAL,   -- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                                             OPTIONAL,   -- Need S

spCellConfigDedicated               ServingCellConfig                                           OPTIONAL,   -- Need M

...,

[[

lowMobilityEvaluationConnected-r17  SEQUENCE {

s-SearchDeltaP-Connected-r17        ENUMERATED {dB3, dB6, dB9, dB12, dB15, spare3, spare2, spare1},

t-SearchDeltaP-Connected-r17        ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                               OPTIONAL,   -- Need R

goodServingCellEvaluationRLM-r17    GoodServingCellEvaluation-r17                               OPTIONAL,   -- Need R

goodServingCellEvaluationBFD-r17    GoodServingCellEvaluation-r17                               OPTIONAL,   -- Need R

deactivatedSCG-Config-r17           SetupRelease { DeactivatedSCG-Config-r17 }                  OPTIONAL    -- Cond SCG-Opt

]]

}

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                     OPTIONAL,   -- Need M

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                                               OPTIONAL,   -- Need N

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

daps-UplinkPowerConfig-r16      DAPS-UplinkPowerConfig-r16                                      OPTIONAL    -- Need N

]],

[[

sl-PathSwitchConfig-r17         SL-PathSwitchConfig-r17                                         OPTIONAL    -- Cond DirectToIndirect-PathSwitch

]],

[[

rach-LessHO-r18                 RACH-LessHO-r18                                                 OPTIONAL,   -- Need N

sl-IndirectPathMaintain-r18     ENUMERATED{true}                                                OPTIONAL     -- Cond MP

]]

}

DAPS-UplinkPowerConfig-r16 ::=      SEQUENCE {

p-DAPS-Source-r16                   P-Max,

p-DAPS-Target-r16                   P-Max,

uplinkPowerSharingDAPS-Mode-r16     ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic }

}

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Need S

]],

[[

preConfGapStatus-r17             BIT STRING (SIZE (maxNrofGapId-r17))                           OPTIONAL,   -- Cond PreConfigMG

goodServingCellEvaluationBFD-r17 GoodServingCellEvaluation-r17                                  OPTIONAL,   -- Need R

sCellSIB20-r17                   SetupRelease { SCellSIB20-r17 }                                OPTIONAL    -- Need M

]],

[[

plmn-IdentityInfoList-r17       SetupRelease {PLMN-IdentityInfoList}                            OPTIONAL,   -- Cond SCellSIB20-Opt

npn-IdentityInfoList-r17        SetupRelease {NPN-IdentityInfoList-r16}                         OPTIONAL    -- Cond SCellSIB20-Opt

]]

}

SCellSIB20-r17 ::= OCTET STRING (CONTAINING SystemInformation)

DeactivatedSCG-Config-r17 ::=       SEQUENCE {

bfd-and-RLM-r17                     BOOLEAN,

...

}

GoodServingCellEvaluation-r17 ::=       SEQUENCE {

offset-r17                              ENUMERATED {db2, db4, db6, db8}                         OPTIONAL   -- Need S

}

SL-PathSwitchConfig-r17 ::=         SEQUENCE {

targetRelayUE-Identity-r17          SL-SourceIdentity-r17,

t420-r17                            ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

...

}

IAB-ResourceConfig-r17 ::=          SEQUENCE {

iab-ResourceConfigID-r17            IAB-ResourceConfigID-r17,

slotList-r17                        SEQUENCE (SIZE (1..5120)) OF INTEGER (0..5119)                           OPTIONAL,    -- Need M

periodicitySlotList-r17             ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160}     OPTIONAL,    -- Need M

slotListSubcarrierSpacing-r17       SubcarrierSpacing                                                        OPTIONAL,    -- Need M

...

}

IAB-ResourceConfigID-r17 ::=        INTEGER(0..maxNrofIABResourceConfig-1-r17)

ReportUplinkTxDirectCurrentMoreCarrier-r17 ::= SEQUENCE (SIZE(1.. maxSimultaneousBands)) OF IntraBandCC-CombinationReqList-r17

IntraBandCC-CombinationReqList-r17::=   SEQUENCE {

servCellIndexList-r17                   SEQUENCE (SIZE(1.. maxNrofServingCells)) OF ServCellIndex,

cc-CombinationList-r17                  SEQUENCE (SIZE(1.. maxNrofReqComDC-Location-r17)) OF IntraBandCC-Combination-r17

}

IntraBandCC-Combination-r17::=      SEQUENCE (SIZE(1.. maxNrofServingCells)) OF CC-State-r17

CC-State-r17::=                     SEQUENCE {

dlCarrier-r17                       CarrierState-r17                             OPTIONAL, -- Need N

ulCarrier-r17                       CarrierState-r17                             OPTIONAL  -- Need N

}

CarrierState-r17::=                 CHOICE {

deActivated-r17                     NULL,

activeBWP-r17                       INTEGER (0..maxNrofBWPs)

}

AutonomousDenialParameters-r18 ::=  SEQUENCE {

autonomousDenialSlots-r18           ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1},

autonomousDenialValidity-r18        ENUMERATED {n200, n500, n1000, n2000}

}

RACH-LessHO-r18 ::=                 SEQUENCE {

targetNTA-r18                       ENUMERATED {zero, source}                                   OPTIONAL,   -- Need N

beamIndication-r18                  CHOICE {

tci-StateID-r18                     TCI-StateId,

ssb-Index-r18                       SSB-Index

}                                                                                               OPTIONAL,   -- Need N

...

}

UplinkTxSwitchingMoreBands-r18::=              SEQUENCE {

uplinkTxSwitchingBandList-r18                  SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FreqBandIndicatorNR OPTIONAL,  -- Need M

uplinkTxSwitchingBandPairList-r18              UplinkTxSwitchingBandPairList-r18                OPTIONAL,   -- Need M

uplinkTxSwitchingAssociatedBandDualUL-List-r18 UplinkTxSwitchingAssociatedBandDualUL-List-r18   OPTIONAL,   -- Need M

...

}

UplinkTxSwitchingBandPairList-r18::=      SEQUENCE (SIZE (1.. maxULTxSwitchingBandPairs)) OF UplinkTxSwitchingBandPairConfig-r18

UplinkTxSwitchingBandPairConfig-r18::=    SEQUENCE {

bandInfoUL1-r18                           UplinkTxSwitchingBandIndex-r18,

bandInfoUL2-r18                           UplinkTxSwitchingBandIndex-r18,

switchingOptionConfigForBandPair-r18      ENUMERATED {switchedUL, dualUL},

switching2T-Mode-r18                      ENUMERATED {enabled}                                             OPTIONAL,   -- Need S

switchingPeriodConfigForBandPair-r18      ENUMERATED {n35us, n140us}                                       OPTIONAL,   -- Need S

...

}

UplinkTxSwitchingAssociatedBandDualUL-List-r18::= SEQUENCE (SIZE (0..maxSimultaneousBands)) OF UplinkTxSwitchingAssociatedBandDualUL-r18

UplinkTxSwitchingAssociatedBandDualUL-r18::=  SEQUENCE {

transmitBand-r18                              UplinkTxSwitchingBandIndex-r18,

associatedBand-r18                            UplinkTxSwitchingBandIndex-r18

}

UplinkTxSwitchingBandIndex-r18::=  INTEGER (1..maxSimultaneousBands)

-- TAG-CELLGROUPCONFIG-STOP

-- ASN1STOP

AutonomousDenialParamters field descriptions

autonomousDenialSlots Indicates the maximum number of the UL slots for which the UE is allowed to deny any UL transmission. Value n2 corresponds to 2 slots, value n5 to 5 slots and so on.

autonomousDenialValidity Indicates the validity period over which the UL autonomous denial slots shall be counted. Value n200 corresponds to 200 slots, value n500 corresponds to 500 slots and so on.

CC-State field descriptions

dlCarrier Indicates DL carrier activation state for this carrier and the related active BWP Index, if activated.

ulCarrier Indicates UL carrier activation state for this carrier and the related active BWP Index, if activated.

CellGroupConfig field descriptions

bap-Address BAP address of the parent node in cell group.

bh-RLC-ChannelToAddModList Configuration of the backhaul RLC entities and the corresponding MAC Logical Channels to be added and modified.

bh-RLC-ChannelToReleaseList List of the backhaul RLC entities and the corresponding MAC Logical Channels to be released.

f1c-TransferPath The F1-C transfer path that an EN-DC IAB-MT should use for transferring F1-C packets to the IAB-donor-CU. If IAB-MT is configured with lte, IAB-MT can only use LTE leg for F1-C transfer. If IAB-MT is configured with nr, IAB-MT can only use NR leg for F1-C transfer. If IAB-MT is configured with both, it is up to IAB-MT to select an LTE leg or a NR leg for F1-C transfer. If the field is not configured, the IAB node uses the NR leg as the default one.

f1c-TransferPathNRDC The F1-C transfer path that an NR-DC IAB-MT should use for transferring F1-C packets to the IAB-donor-CU. If IAB-MT is configured with mcg, IAB-MT can only use the MCG for F1-C transfer. If IAB-MT is configured with scg, IAB-MT can only use the SCG for F1-C transfer. If IAB-MT is configured with both, it is up to IAB-MT to select the MCG or the SCG for F1-C transfer.

mac-CellGroupConfig MAC parameters applicable for the entire cell group.

ncr-FwdConfig Configuration of side control information for the NCR-Fwd access link.

nonCollocatedTypeMRDC This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements when indicating support of interBandMRDC-WithOverlapDL-Bands-r16.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

npn-IdentityInfoList This field is used to transfer npn-IdentityInfoList in SIB1 of the SCell. The UE uses this field to translate the plmn-Index in MCCH of SCell to SNPN Identity. If this field and plmn-IdentityInfoList are both absent, the UE uses the npn-IdentityInfoList in SIB1 of the PCell.

plmn-IdentityInfoList This field is used to transfer plmn-IdentityInfoList in SIB1 of the SCell. The UE uses this field to translate the plmn-Index in MCCH of SCell to PLMN Identity. If this field and npn-IdentityInfoList are both absent, the UE uses the plmn-IdentityInfoList in SIB1 of the PCell.

prioSCellPRACH-OverSP-PeriodicSRS When configured, the UE applies UL power control prioritization by prioritizing PRACH transmission on SCell over semi-persistent and/or periodic SRS transmission as defined in clause 7.5 of TS 38.213 [13].

rlc-BearerToAddModList Configuration of the MAC Logical Channel, the corresponding RLC entities and association with radio bearers.

reportUplinkTxDirectCurrent Enables reporting of uplink and supplementary uplink Direct Current location information upon BWP configuration and reconfiguration. This field is only present when the BWP configuration is modified or any serving cell is added or removed. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. If UE is configured with SUL carrier, UE reports both UL and SUL Direct Current locations.

reportUplinkTxDirectCurrentMoreCarrier Enables reporting of uplink Direct Current location information when the UE is configured with intra-band CA. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. The UE only reports the uplink Direct Current location information that are related to the indicated cc-CombinationList. The network does not include carriers which locate in DL only spectrum described in TS 38.101-2 [39], clause 5.3A.4 and defined by Fsd according to Table 5.3A.4-3 in FR2 in the IntraBandCC-CombinationReqList. I.e. DL-only carrier in FR2 frequency spectrum is not used to calculate the default DC location.

reportUplinkTxDirectCurrentTwoCarrier Enables reporting of uplink Direct Current location information when the UE is configured with uplink intra-band CA with two carriers. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message.

rlc-BearerToReleaseListExt List of the RLC entities and the corresponding MAC Logical Channels to be released for multicast MRBs.

rlmInSyncOutOfSyncThreshold BLER threshold pair index for IS/OOS indication generation, see TS 38.133 [14], table 8.1.1-1. n1 corresponds to the value 1. When the field is absent, the UE applies the value 0. Whenever this is reconfigured, UE resets N310 and N311, and stops T310, if running. Network does not include this field.

sCellSIB20 This field is used to transfer SIB20 of the SCell in order to allow the UE for MBS broadcast reception on SCell. The network configures this field only for a single SCell at a time.

sCellToAddModList List of secondary serving cells (SCells) to be added or modified.

sCellToReleaseList List of secondary serving cells (SCells) to be released.

simultaneousSpatial-UpdatedList1, simultaneousSpatial-UpdatedList2 List of serving cells which can be updated simultaneously for spatial relation with a MAC CE. The simultaneousSpatial-UpdatedList1 and simultaneousSpatial-UpdatedList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

simultaneousTCI-UpdateList1, simultaneousTCI-UpdateList2 List of serving cells which can be updated simultaneously for TCI relation with a MAC CE. The simultaneousTCI-UpdateList1 and simultaneousTCI-UpdateList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType. Network should not configure serving cells that are configured with a BWP with different number of coresetPoolIndexes in the same list. Network should not configure serving cells that are configured with a BWP with different number of SRS resource sets with usage for codebook or noncodebook in the same list.

spCellConfig Parameters for the SpCell of this cell group (PCell of MCG or PSCell of SCG).

uplinkTxSwitchingOption Indicates which option is configured for dynamic UL Tx switching for inter-band UL CA or (NG)EN-DC. The field is set to switchedUL if network configures option 1 as specified in TS 38.214 [19], or dualUL if network configures option 2 as specified in TS 38.214 [19]. Network always configures UE with a value for this field in inter-band UL CA case and (NG)EN-DC case where UE supports dynamic UL Tx switching.

uplinkTxSwitchingPowerBoosting Indicates whether the UE is allowed to enable 3dB boosting on the maximum output power for transmission on carrier2 under the operation state in which 2-port transmission can be supported on carrier2 for inter-band UL CA case with dynamic UL Tx switching as defined in TS 38.101-1 [15]. Network can only configure this field for dynamic UL Tx switching in inter-band UL CA case with power Class 3 as defined in TS 38.101-1 [15].

uplinkTxSwitching-2T-Mode Indicates 2Tx-2Tx switching mode is configured for inter-band UL CA or SUL, in which the switching gap duration for a triggered uplink switching (as specified in TS 38.214 [19]) is equal to the switching time capability value reported for the switching mode. If this field is absent and uplinkTxSwitching is configured, it is interpreted that 1Tx-2Tx UL Tx switching is configured as specified in TS 38.214 [19]. In this case, there is one uplink (or one uplink band in case of intra-band) configured with uplinkTxSwitching, on which the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources should be 1 and non-codebook based UL MIMO is not configured.

uplinkTxSwitching-DualUL-TxState Indicates the state of Tx chains if the state of Tx chains after the UL Tx switching is not unique (as specified in TS 38.214 [19]) in case of 2Tx-2Tx switching is configured and uplinkTxSwitchingOption is set to dualUL. Value oneT indicates 1Tx is assumed to be supported on the carriers on each band, value twoT indicates 2Tx is assumed to be supported on that carrier. This field applies for all band pairs if uplinkTxSwitchingMoreBands is configured.

uplinkTxSwitchingMoreBands Indicates UL band list, band pair list and other configurations for ULTx switching.

uu-RelayRLC-ChannelToAddModList List of the Uu RLC entities and the corresponding MAC Logical Channels to be added or modified.

uu-RelayRLC-ChannelToReleaseList List of the Uu RLC entities and the corresponding MAC Logical Channels to be released.

<omit part>

End of change