3GPP TSG-RAN WG2 Meeting #127		R2-2406575

Maastricht, Netherlands, August 19th – 23rd, 2024

Source:	CATT

Title:	LP-WUS in RRC_IDLE/INACTIVE

Agenda Item:	8.4.2

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting, some agreements were achieved on LP-WUS in RRC_IDLE/INACTIVE, including entry/exit condition for LP-WUS monitoring, subgrouping and SI reception. In this contribution, we give our further considerations on LP-WUS in RRC_IDLE/INACTIVE.

Discussion

Entry/exit condition

In RAN2#126 meeting, we agreed:

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it always configured) Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

When the serving cell quality from MR is above threshold(s), it is still questionable whether the coverage from LR is good enough for LP-WUS monitoring considering the signal strength may be different for MR and LR. Therefore, the serving cell quality from LR apart from the serving cell quality from MR needs to be considered for entry condition of LP-WUS monitoring.

Proposal 1: For entry condition, the serving cell quality from LR needs to be considered, i.e. if the serving cell quality from MR is above threshold(s) (if configured) and the serving cell quality from LR is above threshold(s) (if configured), UE may start to monitor LP-WUS.

For the serving cell measurement metrics, similarly as other thresholds, e.g. the criteria for UE not at cell edge, (LP-)RSRP is always considered while (LP-)RSRQ can be considered with optionally configured. If both (LP-)RSRP and (LP-)RSRQ are configured, entry/exit condition(s) of LP-WUS are fulfilled when the criteria with both (LP-)RSRP and (LP-)RSRQ are fulfilled. And these measurement metrics are applied to thresholds for both MR and LR for entry/exit condition(s) of LP-WUS.

Proposal 2a: If thresholds for MR/LR are configured, (LP-)RSRP is mandatorily configured while (LP-)RSRQ can be optionally configured.

Proposal 2b: If both (LP-)RSRP and (LP-)RSRQ are configured, entry/exit condition(s) of LP-WUS monitoring are fulfilled only when the criteria for both (LP-)RSRP and (LP-)RSRQ are fulfilled.

RAN1 assumes the entry/exit thresholds can be configured separately for different types of LR considering LP-WUS coverage for different types of LR may be different. For example, OFDM based LP-WUS has the same coverage of MR, while the coverage of OOK based LP-WUS is different with MR. If LP-WUS has the same coverage of MR, the threshold(s) for entry/exit conditions for LP-WUS monitoring is not needed, which is similar to PEI behavior. It is up to UE implementation whether/when to start/stop LP-WUS monitoring when threshold(s) for entry/exit conditions for LP-WUS is not configured.

Proposal 3: Thresholds for entry/exit conditions of LP-WUS monitoring are optionally configured by the gNB. If threshold(s) is not configured, it is up to UE implementation whether/when to start/stop LP-WUS monitoring.

Subgrouping

In RAN2#126 meeting, we agreed:

RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

RAN1 has agreed it is up to UE implementation whether to monitor PEI or not after the UE receives LP-WUS indicating wake-up. In order to reduce paging false alarm rate when LP-WUS monitoring is used together with PEI monitoring, UEs belonging to the same LP-WUS subgrouping are expected to be assigned to different PEI subgroupings as much as possible.

Observation 1: In order to reduce paging false alarm rate when LP-WUS monitoring is used together with PEI monitoring, UEs belonging to the same LP-WUS subgrouping are expected to be assigned to different PEI subgroupings as much as possible.

For CN-based subgrouping, it is up to CN to assign the UE ID. RAN2 could send LS to e.g., SA2 to inform the preference on subgrouping, i.e. UEs belonging to the same LP-WUS subgrouping are expected to be assigned to different PEI subgroupings as much as possible. Let other WGs do the final decision.

For UE_ID based PEI subgrouping, the gNB and UE can determine the subgroup ID based on the UE ID and the total number of subgroups for UE ID based subgrouping in the cell. The following is the formula for UE_ID based PEI subgrouping in TS 38.304[1]:

SubgroupID = (floor(UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID), where: N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1 Ns: number of paging occasions for a PF UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192 subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information

It is straightforward to reuse the similar formula as PEI as possible for UE-ID based subgrouping. However, the case is a little more complex for UE-ID-based subgrouping. Take DRX case as one example. The part of UE ID which determines the PO is the up to 10 LSBs (least significant bits), and part of UE ID used for PEI is the up to 3 LSBs next to the ones used to determine the PO. That is to say, up to 13 LSBs is used if a UE is configured with PEI only. For CN-originated paging the RAN is provided with the entire UE ID, and for RAN-originated paging RAN node is provided with an extended IE of 16 LSBs [2].

#### 9.3.3.52 Extended UE Identity Index Value This IE is used by the NG-RAN node to calculate the Paging Frame and Paging Occasion as specified in TS 36.304 [29], the Paging Frame and Paging Occasion for eDRX and the UE_ID based subgroup ID as specified in TS 38.304 [12]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Extended UE Identity Index Value; M; BIT STRING (SIZE(16)).

Hence, only 3 bits are left for LP-WUS subgrouping. For LP-WUS can be used with eDRX, the issue is more severe since only 1 bit is left for LP-WUS subgrouping. Considering the LP-WUS subgrouping number are still under discussion in RAN1. If the LP-WUS subgrouping number exceeds 8 for DRX case (which requires 4bits) or LP-WUS can be configured together with eDRX (for which only 1 bit left), how to design the UE-ID-based subgrouping can be further discussed by RAN2 and perhaps brings impact to other WGs.

Observation 2: The remaining bits of UE ID may be not enough for LP-WUS subgrouping if LP-WUS subgrouping number exceeds 8 for DRX case or LP-WUS can be configured together with eDRX.

Therefore, for UE-ID based solution, the baseline is that RAN2 reuses the formula of PEI as much as possible. But details can be further discussed until some progresses are made in RAN1, e.g. the maximum LP-WUS subgroup number.

Hence, we propose:

Proposal 4: For UE_ID based subgrouping for LP-WUS, similar formula for PEI subgrouping is reused, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns)) mod subgroupsNumForUEID_LP) + (subgroupsNumPerPO_LP – subgroupsNumForUEID_LP), where

UE_ID is related to 5G-S-TMSI, detail depending on RAN1 progress, e.g. LP-WUS subgrouping number,

N is the number of total paging frames in DRX cycle,

Ns is the number of the PO for a PF,

subgroupsNumForUEID_LP and subgroupsNumPerPO_LP are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Configuration

In RAN2#126 meeting, it was agreed:

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

According to the agreements for entry/exit condition for LP-WUS monitoring, serving cell quality from LR needs to be defined. Multi-beam operation is supported for LP-WUR. Therefore, RAN2 needs to discuss how to derive serving cell measurement quantity from LR. The derivation of cell measurement quantity from MR with multi-beam operation can be reused, e.g. derivation of cell measurement quality from LR as the linear average of the power values of up to N highest beam measurement quantity values above a threshold. Similar to derivation of cell measurement quantity from MR, parameters for derivation of serving cell from LR, e.g. the threshold for beams used for derivation of cell measurement quantity and the number of beams (e.g. N ) used for derivation of cell measurement quantity, need to be included in SIB for LP-WUS monitoring in RRC_IDLE/INACTIVE.

Proposal 5: Parameters for derivation of serving cell from LR, e.g. the threshold for beams used for derivation of cell measurement quantity and the number of beams used for derivation of cell measurement quantity, need to be included in SIB for LP-WUS monitoring in RRC_IDLE/INACTIVE.

Conclusion

According to the analysis in section 2, we propose:

Entry/exit condition:

Proposal 1: For entry condition, the serving cell quality from LR needs to be considered, i.e. if the serving cell quality from MR is above threshold(s) (if configured) and the serving cell quality from LR is above threshold(s) (if configured), UE may start to monitor LP-WUS.

Proposal 2a: If thresholds for MR/LR are configured, (LP-)RSRP is mandatorily configured while (LP-)RSRQ can be considered with optionally configured.

Proposal 2b: If both (LP-)RSRP and (LP-)RSRQ are configured, entry/exit condition(s) of LP-WUS monitoring are fulfilled only when the criteria  for both (LP-)RSRP and (LP-)RSRQ are fulfilled.

Proposal 3: Thresholds for entry/exit conditions of LP-WUS monitoring are optionally configured by the gNB. If threshold(s) is not configured, it is up to UE implementation whether/when to start/stop LP-WUS monitoring.

Subgrouping:

Observation 1: In order to reduce paging false alarm rate when LP-WUS monitoring is used together with PEI monitoring, UEs belonging to the same LP-WUS subgrouping are expected to be assigned to different PEI subgroupings as much as possible.

Observation 2: The remaining bits of UE ID may be not enough for LP-WUS subgrouping if LP-WUS subgrouping number exceeds 8 for DRX case or LP-WUS can be configured together with eDRX.

Proposal 4: For UE_ID based subgrouping for LP-WUS, similar formula for PEI subgrouping is reused, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns)) mod subgroupsNumForUEID_LP) + (subgroupsNumPerPO_LP – subgroupsNumForUEID_LP), where

UE_ID is related to 5G-S-TMSI, detail depending on RAN1 progress, e.g. LP-WUS subgrouping number,

N is the number of total paging frames in DRX cycle,

Ns is the number of the PO for a PF,

subgroupsNumForUEID_LP and subgroupsNumPerPO_LP are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Configuration:

Proposal 5: Parameters for derivation of serving cell from LR, e.g. the threshold for beams used for derivation of cell measurement quantity and the number of beams used for derivation of cell measurement quantity, need to be included in SIB for LP-WUS monitoring in RRC_IDLE/INACTIVE.

Reference

- 3GPP TS 38.304 V18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state;

- 3GPP TS 38.413 V18.2.0, NG-RAN, NG Application Protocol (NGAP).