3GPP TSG-RAN WG2 Meeting #127	R2-2406576

Maastricht, Netherlands, August 19th – 23rd, 2024

Source:	CATT

Title:	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting, the following agreements were achieved on RRM relaxation and offloading in RRC_IDLE/INACTIVE.

For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

In this contribution, we give our further considerations on RRM relaxation and offloading in RRC_IDLE/INACTIVE for LP-WUS.

Discussion

Background

A UE in RRC_IDLE/INACTIVE mode needs to perform RRM measurements for cell selection/reselection to ensure it camps on the best cell. And the serving cell shall be measured and evaluated according to the requirement defined in TS 38.133. If the serving cell is good enough (e.g. in cell center), the UE may not perform neighbor cell measurements in the cases including:

-	If the serving cell fulfills Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may not perform intra-frequency measurements.

-	If the serving cell fulfills Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

-	The UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies.

In R16, in order to reduce UE power consumption, that UE relaxes RRM measurements for neighbor cells when it meets the criteria of low mobility and/or not at cell edge was introduced. For example:

- If the UE is configured with low mobility criterion and UE has fulfilled, or the UE is configured with both low mobility criterion and not at cell edge criterion and both criteria are not required to combined and UE has fulfilled only low mobility criterion, the UE may choose to perform relaxed measurements for intra-frequency cells, NR inter-frequency cells or inter-RAT frequency cells according to relaxation methods in TS 38.133.

In R17, In RRC_IDLE and RRC_INACTIVE an (e)RedCap UE is allowed to relax neighbor cell RRM measurement when the stationary criterion is met or when both stationary and not-at-cell-edge criteria are met.

In order to achieve more power saving, it is preferred that MR neighbor cell measurement is stopped or relaxed, when serving cell measurement is offloading to LR or MR serving cell measurement is relaxed. Hence, in the following sections, we would like to discuss MR serving cell measurement together with MR neighbor cell measurements.

Fully offloading MR serving cell measurement

As mentioned above, when the serving cell is good enough, the UE may not perform intra-frequency measurements, NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority. When the serving cell is good enough, the UE doesn’t need to perform measurements for mobility purpose. In this case, the UE can offload MR serving cell measurement to LR. And at least the condition that serving cell quality from LR is above threshold(s) can be considered as the offloading criterion.

Proposal 1: At least RAN2 consider serving cell quality from LR above threshold(s) can be as a criterion for fully offloading MR serving cell measurement to LR.

In RAN4#111 meeting, RAN4 also agreed the MR serving cell measurement fully offloading case.

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON.

In this case, it not required to perform MR neighboring cell measurement either. In our understanding, MR neighbor cell measurements on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority are not required when MR serving cell measurement is offloading to LR. But whether MR neighbor cell measurements on NR inter-frequency cells of higher priority, or inter-RAT frequency cells of higher priority can be stopped depends on RAN4’s further discussion.

If MR neighbor cell measurements on high priority are not stopped in this case, cell reselection may be performed to a neighbor cell with high priority. With the existing cell reselection criteria for MR inter-frequency cells or inter-RAT frequency cells of higher priority, only the neighbor cell quality needs to be considered, as shown below. Hence, the existing cell reselection criteria for higher priority can be reused here.

5.2.4.5 NR Inter-frequency and inter-RAT Cell Reselection criteria If threshServingLowQ is broadcast in system information and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a higher priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if: - A cell of a higher priority NR or EUTRAN RAT/frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT Otherwise, cell reselection to a cell on a higher priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if: - A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and - More than 1 second has elapsed since the UE camped on the current serving cell.

Proposal 2：When the criterion for fully offloading MR serving cell measurement to LR is fulfilled, it is not required to perform measurement for MR neighbor cell on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

Proposal 3：When the criterion for fully offloading MR serving cell measurement to LR is fulfilled, it is left for RAN4 to discuss whether the MR neighbor cell measurements on NR inter-frequency cells of higher priority, or inter-RAT frequency cells of higher priority can be stopped.

Proposal 4: The existing cell reselection criteria for higher priority can be reused if MR serving cell measurement is fully offloaded to LR while MR neighbor cell measurements on higher priority are not stopped.

Relaxed RRM measurement

In RAN4#111 meeting, case #2, case #3 and case #4 were discussed without conclusion. In this section, we would like to discuss the possible RAN2 impact in these cases.

Case #2: MR serving cell measurement is relaxed while MR neighbor cell measurements are off

Similar to case #1, MR neighbor cell measurements in case #2 doesn’t include MR neighbor cell measurements on NR inter-frequency cells of higher priority, or inter-RAT frequency cells of higher priority. If the case is agreed, RAN2 need to specify corresponding criteria.

In addition, when the UE performs relaxed MR serving cell measurement during LP-WUS operation, there are possible options for cell reselection.

- Option 1: LR Serving cell measurement is used for cell reselection.

- Option 2: MR Serving cell measurement is used for cell reselection.

- Option 3: Both LR and MR Serving cell measurement are used for cell reselection.

Regarding option 1, cell-ranking criterion R for cell reselection needs to be updated. When the UE performs RRM measurement relaxation on serving cell via MR during LP-WUS operation, RRM measurement relaxation on serving cell via MR is still available and valid. For option 2, the existing criteria for cell reselection are kept while Option 3 is complex considering the measurement quality for MR and LR may be different and more spec work may be needed. Based on the analysis above, Option 2 is preferred as it is simple and the existing cell reselection criteria can be used.

Observation 1: For case #2 (MR serving cell measurement is relaxed while MR neighbor cell measurements are off), RAN2 need to specify corresponding criteria for cell reselection may be kept or be impacted which is depended on the relaxed mechanism of MR serving cell measurement.

Case #3: Both MR serving cell measurement and MR neighbor cell measurement are relaxed.

If the case is agreed, RAN2 need to specify corresponding criteria. Furthermore, similar to case #2, there are 3 possible options for cell reselection too. And cell reselection may be kept or be impacted which depends on the relaxed mechanism of MR serving cell measurement.

Observation 2: For case #3 (both MR serving cell measurement and MR neighbor cell measurement are relaxed), RAN2 need to specify corresponding criteria for cell reselection may be kept or be impacted which is dependent on the relaxed mechanism of MR serving cell measurement.

Case #4: MR serving cell measurement is off while MR neighbor cell measurements are on

For case #4, it is unclear the difference between case#1 and case#4. There are two possible understanding for MR neighbor cell measurements in case#4:

- Understanding 1: MR neighbor cell measurements only include MR neighbor cell measurements on NR inter-frequency cells of higher priority, or inter-RAT frequency cells of higher priority.

- Understanding 2: MR neighbor cell measurements include all possible neighbor cell measurements with higher, equal and lower priority.

With understanding 1, case#4 is a special case of case#1 and the analyses for case#1 can be reused. With understanding 2, RAN2 needs to update cell reselection criterion as LR serving cell measurement needs to be considered in R criterion for serving cell.

Observation 3: It is unclear the difference between case #1 (fully offloading case) and case #4 (MR serving cell measurement is off while MR neighbor cell measurements are on).

Considering RAN4 would further discuss and make conclusions on case #2 to case #4, RAN2 can wait for RAN4’s further conclusions to analyze possible impacts on RAN2.

Proposal 5: RAN2 waits for RAN4 further conclusions on cases of relaxed MR RRM measurements.

Conclusion

For fully offloading case, we propose:

Proposal 1: At least RAN2 consider serving cell quality from LR above threshold(s) can be as a criterion for fully offloading MR serving cell measurement to LR.

Proposal 2：When the criterion for fully offloading MR serving cell measurement to LR is fulfilled, it is not required to perform measurement for MR neighbor cell on intra-frequency, NR inter-frequency cells of equal or lower priority, and inter-RAT frequency cells of lower priority.

Proposal 3：When the criterion for fully offloading MR serving cell measurement to LR is fulfilled, it is left for RAN4 to discuss whether the MR neighbor cell measurements on NR inter-frequency cells of higher priority, or inter-RAT frequency cells of higher priority can be stopped.

Proposal 4: The existing cell reselection criteria for higher priority can be reused if MR serving cell measurement is fully offloaded to LR while MR neighbor cell measurements on higher priority are not stopped.

For other cases in RAN4, we observe and propose:

Observation 1: For case #2 (MR serving cell measurement is relaxed while MR neighbor cell measurements are off), RAN2 need to specify corresponding criteria for cell reselection may be kept or be impacted which is depended on the relaxed mechanism of MR serving cell measurement.

Observation 2: For case #3 (both MR serving cell measurement and MR neighbor cell measurement are relaxed), RAN2 need to specify corresponding criteria for cell reselection may be kept or be impacted which is dependent on the relaxed mechanism of MR serving cell measurement.

Observation 3: It is unclear the difference between case #1 (fully offloading case) and case #4 (MR serving cell measurement is off while MR neighbor cell measurements are on).

Proposal 5: RAN2 waits for RAN4 further conclusions on cases of relaxed MR RRM measurements.

Reference

- 3GPP TS 38.304 V18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state;