3GPP TSG-RAN WG2 Meeting #127                             R2-2406578

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.1.2.2

Source:	Mediatek Inc.

Title:	Availability and Applicability Reporting for UE-side Functionality

Document for:	Discussion, Decision

# Introduction

In RAN2#125bis meeting, following agreements were made regarding the reporting of UE-sided applicable functionality.

Agreements 1. Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress. “supported” means that the UE is capable of supporting the functionality and doesn’t mean neccesarily that the UE has the model available. FFS what functionality refers to. 2. Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability. Agreements for positioning and beam management - Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message. - Support reactive reporting of UE-sided applicable functionality. The NW configures AI/ML functionalities via RRC/LPP message. FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality 3 FFS how the two approaches will be specified and whether we can combine them into one procedure. FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

Subsequently, in the RAN2#126 meeting, further agreements were reached specifically for UE-side functionality availability/applicability pertaining to the use case of AI BM.

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

This paper tries to further clarify the availability and applicability reporting for UE-side functionality based on the outcome of email discussion [POST126][032][AI/ML PHY] LCM (Intel/Samsung).

# Discussion

## 2.1 Network-Side Condition and Associated ID

At the RAN1#116bis meeting, RAN1 introduced the concept of an associated ID just as highlighted in yellow. The use of associated IDs, as discussed from the RAN1 perspective, facilitates an approach for developing UE-side AI/ML models. Initially, the network signals data collection configurations along with their associated IDs, tailored for specific sub-use cases and additional network-side conditions. Subsequently, the UE collects data corresponding to these associated IDs, which then serves as the foundation for the development (training and updating) of AI/ML models at the UE side. Finally, the UE reports back to the network about its AI/ML models, linking them to the original associated IDs. This process ensures the consistency in model training and inference processes for UE-side model while safeguarding the network side's specific implementation and configuration details from disclosure. By associating network-side conditions and specific configurations with an ID, signaling overhead can be minimized.

From the RAN2 perspective, the associated ID is considered as a type of network-side condition that is signaled from network to the UE along with the data collection configuration. It remains to be FFS whether additional network-side conditions are necessary or will be explicitly signaled, which is up to RAN1 discussion.

Proposal 1: RAN2 assumes that the associated ID is a type of network-side condition. It’s up to RAN1 whether additional network-side conditions are necessary or will be explicitly signaled.

Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

## 2.2 Applicability Determination

In last RAN2 meeting, it was agreed that the UE determines whether a functionality is applicable as a baseline. In the post meeting email discussion, this issue was raised, and three options were listed, i.e., Joint decision by UE and NW, UE decision only; and network decision only.

The decision of the AI/ML functionality applicability is based on a combination of factors including the UE’s additional/internal conditions, the NW-side additional conditions, and the availability of the AI/ML models for the functionality.

Given that the UE's additional/internal conditions are closely linked to its proprietary implementation, disclosing these for standardization would be impractical due to concerns over revealing proprietary information, maintaining competitive differentiation, and adding complexity to the standardization process. Introducing a similar ID for the UE-side condition, akin to the associated ID for the network side, would unnecessarily complicate interactions between the UE and the network.

Proposal 2: UE-side additional condition is kept by UE itself and is not signaled to network.

Figure 1

However, since the network-side condition is already associated with an ID as per RAN1's agreement, there are no issues in signaling this ID from the network to the UE. This allows the UE to make an informed and precise decision on the applicability of AI/ML functionalities, considering its internal conditions, the availability of AI/ML models, and the associated ID for network-side conditions. As the model's availability is already factored into the UE's decision on AI/ML functionality applicability, there is no need for this information to be signaled back to the network.

Proposal 3: RAN2 assumes that the associated ID related to the network condition is signaled from network to the UE for the UE to determine the AI/ML functionality applicability.

In last RAN2 meeting, it was agreed that the UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable. Based on the UE-side additional/internal condition, the availability of the AI/ML model, as well as the associated ID related to the network-side condition, UE can decide the AI/ML functionality applicability in a more informed and precise decision-making process. Since the availability of the AI/ML model has already been considered when deciding the AI/ML functionality applicability by the UE, the UE doesn’t need to be signaled to the network.

Proposal 4: The applicability of the UE-side functionality is determined by the UE according to the UE-side additional/internal condition, the network-side condition, and availability of the AI/ML models for the functionality.

After receiving the applicability reporting of the AI/ML functionality from the UE, the network decides on the appropriate inference configuration, e.g., the specific parameters and settings that will optimize the performance of the AI/ML functionalities on the UE, considering the current network conditions and the capabilities of the UE as reported. Subsequently, the network transmits the inference configuration to the UE and may optionally activate the AI/ML functionality. It then continues to monitor the performance of these functionalities to ensure optimal operation.

Proposal 5:  Following the UE's report on AI/ML functionality applicability, the network decides and provides the inference configuration to the UE.

## 2.3 Availability Reporting

During the RAN1#116 and RAN1#116bis meetings, RAN1 concluded that while model transfer/delivery cases z5, z2, and z3 are deprioritized, cases z1 and z4 remain under consideration. According to proposal 4, indicating the availability of AI/ML models for UE-side functionality is not necessary for applicability reporting. However, in the context of model transfer discussions, supporting cases z1 and z4—which involve transferring models from the network to the UE—may necessitate AI/ML model availability reporting for UE-side functionality.

Conclusion: From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19. Conclusion From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons: - Risk of proprietary design disclosure - Burden of offline cross-vendor collaboration Conclusion From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y): - No much benefit compared to Case y - Risk of proprietary design disclosure - Large burden of offline cross-vendor collaboration - Additional burden on model storage within in 3GPP network

As depicted in Figure 2, the network-controlled model transfer procedure is the baseline approach. Here, the UE must signal its AI/ML model availability for UE-side functionality. The network then uses this information, along with the UE capability report, to decide whether to transfer the AI/ML model to the UE. If the network considers the applicability of the AI/ML model during the transfer, such as in case z4 where only the model's parameters are updated, it may not be necessary for the UE to send a separate applicability report to the network. Essentially, the network could transfer only those AI/ML models that are applicable for inference to the UE.

Figure 2

While AI/ML functionality availability reporting and applicability reporting serve different Life Cycle Management (LCM) purposes—model inference and model transfer, respectively—the baseline method, UAI, can be utilized for AI/ML functionality availability reporting.

Proposal 6: AI/ML functionality availability reporting and applicability reporting serve distinct LCM purposes, with the former facilitating model inference and the latter supporting model transfer processes.

Proposal 7: The UAI framework can be used for AI/ML functionality availability reporting.

# Conclusion

Proposal 1: RAN2 assumes that the associated ID is a type of network-side condition. It’s up to RAN1 whether additional network-side conditions are necessary or will be explicitly signaled.

Proposal 2: UE-side additional condition is kept by UE itself and is not signaled to network.

Proposal 3: RAN2 assumes that the associated ID related to the network condition is signaled from network to the UE for the UE to determine the AI/ML functionality applicability.

Proposal 4: The applicability of the UE-side functionality is determined by the UE according to the UE-side additional/internal condition, the network-side condition, and availability of the AI/ML models for the functionality.

Proposal 5:  Following the UE's report on AI/ML functionality applicability, the network decides and provides the inference configuration to the UE.

Proposal 6: AI/ML functionality availability reporting and applicability reporting serve distinct LCM purposes, with the former facilitating model inference and the latter supporting model transfer processes.

Proposal 7: The UAI framework can be used for AI/ML functionality availability reporting.