3GPP TSG-RAN WG2 Meeting #127                                         R2-2406580

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.3.2.2

Source:	Mediatek Inc.

Title:	Cluster-based Approach for RRM Prediction and Other Aspects

Document for:	Discussion, Decision

# Introduction

For AI/ML RRM prediction, in the #126bis meeting, RAN2 focus on the discussion of AI approach in which the model input contains the measurement result of “single cell”. Also, the study of performance metrics of each use case focuses on the intermediate KPI, e.g., prediction accuracy. However, from our point of view, to fully exploit the AI capability and examine the real impact of mobility performance, other advanced options should be considered in the RAN2 SID.

For the AI approaches, we notice that instead of considering the single-cell AI approach where the input only contains single-cell measurement, “cluster-based approaches” which consider multiple-cell input can further increase the AI performance and can reduce the complexity of AI LCM, e.g., model update and/or switch.

In this paper, we introduce cluster-based approach. Furthermore, to examine AI performance gain in real communication systems, we think system-level KPI, e.g., RLF, HOF, ping pong, ToS, etc, are still necessary.

To simplify the overhead of simulation with system-level KPI and let the comparison with different companies more meaningful, we propose that RAN2 discuss aligned non-AI baseline for each sub-case such that companies can provide their AI performance gain by comparing AI and non-AI results under the same simulation environment. With an aligned non-AI baseline, we can also reduce the calibration overhead and mitigate the impact of implementation differences between companies.

# Discussion

## Cluster-based Approaches

Cluster-based approaches, which refer to “use measurement results of multiple cells to predict the measurement of one or more cells”, have great potential in the AI mobility topic. There are two key differences between cluster and cell-specific approaches (a) the cluster-based approach can incorporate measurements from multiple cells as inputs to the model, and (b) the AI model is applicable across a region rather than being limited to a single, specific cell. In the following content, we explain the detailed reasons and motivations for considering cluster-based approaches and provide the simulation comparison in all sub-use cases of AI RRM prediction agreed upon RAN2 discussion.

Cluster-based approaches enable AI models to consider measurements from multiple cells, making these models applicable to a larger region beyond a single cell, offering several potential advantages:

- Improved Prediction Accuracy: Utilizing measurements from surrounding cells enhances the predictive accuracy of the model.

- Efficient Measurement Process: In intra-frequency cases, simultaneous measurement of SSBs from both the serving and neighboring cells does not require additional effort from the UE.

- Enhanced Generalization and Robustness: This method generalizes across different cells, capturing a broader range of scenarios and network condition variations, reducing the risk of model overfitting and increasing robustness to environmental changes

- Simplified Life Cycle Management (LCM): Managing a single model for a group of cells simplifies LCM by reducing operational overhead and optimizing model monitoring, model management, and model update processes.

- Minimized Ping-Pong Effects: By learning handover characteristics across multiple cells, the model optimizes decisions holistically, effectively detecting and proactively mitigating potential ping-pong effects based on a broader dataset.

For the first reason, even though we all agree the measurement of the cell targeted for prediction provides the most directly correlated information, however, the measurement of other cells can also contribute valuable information, aiding the AI model in learning the characteristics of the cell targeted for prediction. The correlation may come from the spatial (geographical) relation between different cells, for example, if two cells are in the same direction respective to the UE, they may be blocked by the same building. If the AI model is cell-specific, it’s possible that the AI model between the serving cell and neighboring cells are independent, leading to suboptimal decisions that cause ping-pong handovers. Also, cluster-based approaches may reduce the complexity of AI model management. The model update may be based on the location of UE (within a certain area). UE does not need to change its model each time when HO occurs.

Proposal 1: RAN2 assumes that cluster-based approaches can provide benefits including improved prediction accuracy, efficient measurement process, enhanced generalization and robustness, simplified LCM, and minimized ping-pong effect during HO.

Proposal 2: Cluster-based approaches refer to the AI method that uses measurement results of multiple cells, and the AI model is applicable cross multiple cells.

### Simulation Comparison

In this subsection, we provide the simulation results for different RRM prediction use cases. We compare two different approaches, i.e., cluster-based and cell-specific approaches, as shown in Figure 1. The AI input/output of the cluster-based approach contains the measurement of all 21 cells and only one model is trained in this approach. For the cell-specific approach, we will train 21 models, where each model is trained by the measurement and label data of the corresponding cell, e.g., cell0~cell20. The model input/output are only one cell. When performing inference, the prediction of each cell will be obtained from the corresponding model.

Figure 1: Illustration of the cluster and cell-specific approaches

Table 1: Comparison of cluster-based and cell-specific approaches in different use cases

| Use cases | Approaches | AI Input/ output | Abs L3 Cell RSRP diff (dB) | Flops | Model size (KByte) |
| --- | --- | --- | --- | --- | --- |
| Temporal domain RRM prediction (Goal 1, 50% meas. reduction, FR1, 30km) | Non-AI | NA | 0.88 | NA | NA |
|  | Cluster-based (1 model) | L3 to L3 | 0.21 | 64,663,200 | 187 |
|  |  | L1 to L3 | 0.63 |  |  |
|  |  | L1 to L1 | 0.78 |  |  |
|  | Cell-specific (21 models) | L3 to L3 | 0.25 | 4,155,200 * 21 =87,259,200 | 118*21 =2478 |
|  |  | L1 to L3 | 0.67 |  |  |
|  |  | L1 to L1 | 0.85 |  |  |
| Frequency domain RRM prediction (FR1, 30km) | Non-AI | NA | (a) 6.17 (b) 2.34 | NA | NA |
|  | Cluster-based (1 model) | L3 to L3 | 1.64 | 3,029,460 | 201 |
|  |  | L1 to L3 | 1.93 |  |  |
|  |  | L1 to L1 | 1.97 |  |  |
|  | Cell-specific (21 models) | L3 to L3 | 2.29 | 142,260 * 21 =2,987,460 | 27.8*21 = 585 |
|  |  | L1 to L3 | 2.31 |  |  |
|  |  | L1 to L1 | 2.42 |  |  |
| Spatial domain RRM prediction (50% meas. reduction, FR2, 30km) | Cluster-based (1 model) | L1 to L3 | 2.08 | 566,415,360 | 302 |
|  |  | L1 to L1 | 0.29 | 354,009,600 | 3857 |
|  | Cell-specific (21 models) | L1 to L3 | 2.20 | 33,382,400*21= 701,030,400 | 90*21 =1820 |
|  |  | L1 to L1 | 0.43 | 132,915,200*21= 2,731,219,200 | 336*21 =7056 |

Table 1 shows the performance comparison between cluster and cell-specific approaches in temporal/frequency/spatial domain RRM prediction.

For temporal domain prediction, the non-AI approach refers to the “sample and hold” approach, i.e., use the nearest observation as the prediction measurement result. For frequency domain prediction, we consider two different non-AI baselines (a) prediction (4GHz) = observation (2GHz) and (b) prediction (4GHz) = observation (2GHz) – pathloss adjustment (~6dB).

Different AI input/output sub-use cases, e.g., L1 to L1, L1 to L3, and L3 to L3, are considered in each use case. As shown in the table, the cluster-based approach outperforms cell-specific approaches in all use cases under a similar AI model architecture, e.g. CNN model with a similar structure.

For the frequency domain prediction case, our results show that the cell-specific method achieves limited gain compared to those of a non-AI approach that incorporates pathloss compensation. This implies that cell-specific approaches are hard to learn additional information except for pathloss difference between 2GHz and 4GHz. However, with the help of neighbor cell measurement, cluster-based approaches can achieve lower RSRP differences.

Another significant benefit is that under a similar AI model structure the cluster approach can save more device memory since UE can predict all 21 cells with one trained model instead of 21 different AI models.

Observation 1: The cluster-based approach outperforms cell-specific approaches across all use cases including temporal-domain, frequency-domain, and spatial-domain prediction.

Observation 2: In frequency domain prediction cases, cell-specific approaches achieve limited gain compared to the non-AI approach without the help of neighbor cell measurement.

Observation 3: When employing a similar AI model architecture, such as CNN, the cluster-based approach is more storage-efficient for devices.

Proposal 3: RAN2 support to study the cluster-based approaches in each RRM prediction use case.

## AI Gain Verification with Aligned Non-AI Baseline

One of the important goals of this SID is to examine the AI performance gain of each use case such that one can determine the potential way to enhance mobility with AI. To obtain a concrete and consistent conclusion from different companies’ results, a fair comparison is necessary. However, it is challenging since the implementation of the simulation environment and the assumption of UE/gNB behavior in HO procedure vary from different companies even with considering aligned simulation settings. A more practical and simple way is to consider an aligned non-AI baseline as a performance benchmarking which helps us to

- Show the AI performance gain by comparing with the non-AI baseline

- Offer a consistent baseline for performance benchmarking that remains comparable across different simulation implementations from different companies

No matter considers the intermediate KPI, e.g., RSRP difference, or the system level KPI, e.g., HOF. Those metrics needed a baseline to provide useful meaning. For example, considering temporal-domain prediction for Goal 1, observing a low RSRP difference value from an AI approach does not mean a benefit from AI, if a non-AI baseline, e.g., sample and hold, can achieve a lower (or similar) RSRP difference value. It just means that the channel does not change much between two measurement instances.

For the second purpose described above, even though RAN2 has agreed on many simulation settings before starting simulation work, due to differences in implementation, results are still expected to vary significantly from company to company. Similar observations can be found in the evaluation results in R18 AI-BM [3]. Even with considering the alignment of the evaluation setting and assumption, the results provided by each company still vary. Thus, it is necessary to consider an aligned non-AI baseline that provides a consistent baseline for performance benchmarking such that we can identify the bias due to different companies' simulation environments and implementation.

For example, if we observe the results of a company are always better than others, no matter the non-AI and AI approaches, this may imply the dataset, channel, and/or simulation environment provided by this company is easier for prediction, but it may not imply the AI can provide such good performance. Also, since both AI and non-AI approach results from the same company are generated through the same simulation environment, it is more reasonable to compare the AI performance gain, i.e., the difference between AI and aligned non-AI approaches, instead of comparing AI performance directly. This can reduce the impact of implementation differences.

Observation 4: In R18 AI-BM [3] the evaluation results vary in different companies due to the different simulation implementations, even with considering the aligned evaluation settings. A non-AI baseline can provide a consistent baseline for performance benchmarking that helps to identify the bias from different companies' simulations.

The selection of non-AI baseline is different from each sub use case. Since the non-AI baseline is only considered as a reference, we can start from the simplest approach and does not need to consider too advanced approaches. For Goal 1, the baseline of temporal domain prediction can reuse the approach in R18 AI-BM [3], i.e., sample and hold approach, where the measurement result in prediction instance is replaced by the result in the nearest observation instance directly. For the spatial domain prediction, we can modify the method in [3], the predicted cell level result can be derived from the observation set B only. For frequency domain prediction, if co-located deployment is considered, a simple way is to assume the non-AI prediction to be equal to observation with pathloss modification, e.g., prediction (4GHz cell) = observation (co-located 2GHz cell) – pathloss difference (~6dB). For Goal 2, the baseline of temporal domain prediction could be the normal measure pattern, e.g., measure all instances. Other non-AI approaches are FFS.

Proposal 4: RAN2 discuss the non-AI baseline for performance benchmarking for each use case. The baseline in Table 2 can be the starting point. Other options could be FFS.

|  | Goal 1 | Goal 2 |
| --- | --- | --- |
| Temporal | Sample and hold | Measure all instance |
| Spatial | Derive cell level quality from observation set B | NA |
| Frequency | Predict based on co-located cell measurement with pathloss modification | FFS |

Table 2: Non-AI baseline for different sub use case.

## System KPI

In RAN plenary # 102, the SID on AI mobility [1] agree to study the system performance KPIs. Although from the previous meeting, RAN2 agrees to study the prediction accuracy, e.g., intermediate KPI, at the first step, and the need of system performance will be FFS, we still think the system KPI plays an important role in the evaluation and is necessary for each use case.

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

From our perspectives, intermediate KPIs, e.g., RSRP difference, are designed to examine the predicted capability of AI model. To identify the real AI benefit, system KPIs, e.g., HOF, RLF rate, throughput, are necessary. For example, in the Goal 1, i.e., measurement overhead reduction, we cannot check the tradeoff between the measurement reduction and performance degradation through the intermediate KPI such as RSRP difference. For Goal 2, i.e., HO performance enhancement, it is also impossible to realize, through RSRP difference, the tradeoff between the benefit from early awareness of the future measurement by prediction and the degradation from prediction error. The intermediate KPIs, only help us to understand the potential of AI approaches in terms of their prediction capability, but the real benefit should be examined by the system KPIs. Due to the above reason, we propose to consider the system KPIs in AI mobility use case.

Observation 5: Intermediate KPIs are designed to examine the predicted capability of AI model. To examine the real benefit for AI approaches, system KPIs are necessary. For example, to examine (1) the tradeoff between measurement reduction and performance degradation (2) the tradeoff between the benefit from predicted future measurement and the degradation due to the prediction error.

One of the challenges for RAN2 to study and evaluate system-level performance comes from the overhead of aligning system-level simulation settings. Considering the time budget and workload, we propose RAN2 to study the system-level simulation with aligned non-AI baselines as a reference, so that detailed alignment and calibration of system-level simulation, e.g., the interference model, the HO procedure, the msg transmission model, the HOF, RLF mechanism, are not mandatory. RAN2 can directly compare the AI gain by examining the difference between AI and aligned non-AI results from the same company. Since both AI and non-AI results are obtained from the same simulation environment, comparing the AI/nonAI performance difference is fairer than directly comparing the AI performance, and thus, it is more likely to achieve concrete conclusions.

Proposal 5: RAN2 consider the system KPIs, HOF/RLF/ToS…etc, for each use case.

Proposal 6: To reduce the workload of system-level simulation alignment, RAN2 can compare the system-level performance through AI performance gain, i.e., the difference between AI and non-AI approach used as the performance benchmarking.

# Conclusion

Observations:

Observation 1: The cluster-based approach outperforms cell-specific approaches across all use cases including temporal-domain, frequency-domain, and spatial-domain prediction.

Observation 2: In frequency domain prediction cases, cell-specific approaches achieve limited gain compared to the non-AI approach without the help of neighbor cell measurement.

Observation 3: When employing a similar AI model architecture, such as CNN, the cluster-based approach is more storage-efficient for devices.

Observation 4: In R18 AI-BM [3] the evaluation results vary in different companies due to the different simulation implementations, even with considering the aligned evaluation settings. A non-AI baseline can provide a consistent baseline for performance benchmarking that helps to identify the bias from different companies' simulations.

Observation 5: Intermediate KPIs are designed to examine the predicted capability of AI model. To examine the real benefit for AI approaches, system KPIs are necessary. For example, to examine (1) the tradeoff between measurement reduction and performance degradation (2) the tradeoff between the benefit from predicted future measurement and the degradation due to the prediction error.

Proposals:

Cluster-basaed approach

Proposal 1: RAN2 assumes that cluster-based approaches can provide benefits including improved prediction accuracy, efficient measurement process, enhanced generalization and robustness, simplified LCM, and minimized ping-pong effect during HO.

Proposal 2: Cluster-based approaches refer to the AI method that uses measurement results of multiple cells, and the AI model is applicable cross multiple cells.

Proposal 3: RAN2 support to study the cluster-based approaches in each RRM prediction use case.

Non-AI benchmarking

Proposal 4: RAN2 discuss the non-AI baseline for performance benchmarking for each use case. The baseline in Table 2 can be the starting point. Other options could be FFS.

Necessecity of SLS

Proposal 5: RAN2 consider the system KPIs, HOF/RLF/ToS…etc, for each use case.

Proposal 6: To reduce the workload of system-level simulation alignment, RAN2 can compare the system-level performance through AI performance gain, i.e., the difference between AI and non-AI approach used as the performance benchmarking.

# Reference

[1]	RP-234055 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

[2]	TR 36.839 Mobility enhancements in heterogeneous networks

[3] 	TS 38.331 Radio Resource Control (RRC);Protocol specification

# Appendix: Simulation Setting

| parameters | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz,30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model (7 sites, 3 sectors/cells per site) | 2-tier model (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and LOSsoft; Inter-frequency correlation model is optional. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and LOSsoft; |
| System BW | 20MHz | 80MHz |
| UE speed | 30,90 km/h for study targeting measurement reduction 60,120 km/h for study targeting HO performance improvement | 30,,90 km/h for study targeting measurement reduction 60,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ | (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 1, 1, 1) |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 with coarse update | Procedure A in TR38.901 with coarse update |
| UE trajectory model | 3 options in 38.843 section 6.3.1 | 3 options in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | circle-bouncing model, | circle-bouncing model, |
| Sampling period | 40ms | 20ms |