3GPP TSG-RAN WG2 Meeting #127                                         R2-2406581

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.3.4.1

Source:	Mediatek Inc.

Title:	Simulation assumptions and evaluation methodology for RLF failure prediction

Document for:	Discussion, Decision

# Introduction

For AI/ML RRM prediction, in the #125bis meeting, RAN2 made the following agreement：

Agreements

1	Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR).

2	Study Direct: Directly RLF prediction by AI/ML models.

3	FR2 study will be prioritized for RLF prediction

4	The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

5	HOF prediction is downprioritized in our study.  NO simulations/evaluations should be done/submitted

6	RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

7	No evaluation/simulations are expected for August meeting for RLF

8	Simulation assumption specific to RLF will be discussed in August.  The assumption is that we will reuse RRM simulation assumptions (where possible).

In this paper, we focus on the discussion of AI approaches, including direct/indirect prediction, the RLF condition, and the simulation setting which is necessary for RLF prediction, especially for the assumption of interference and UE behavior.

# Discussion

## RLF Prediction

The goal of RLF prediction is to use AI to detect possible RLF events so that UE and gNB can early respond to the incoming RLF. According to [1], the declaration of RLF includes multiple conditions. From the previous meeting, RAN2 agree to consider one of those conditions, i.e., the T310 expiry, as the starting point. The trigger and stop of T310 timer will based on the monitoring of RLM-RS. UE estimates the DL downlink quality and compares it with the threshold Qin and Qout, where Qin and Qout are defined according to in-sync block error rate and out-of-sync block error rate on PDCCH transmission, respectively [2].  In [2], the default values of in-sync block error rate and out-of-sync block error rate are given by 2% and 10%.

One option to simulate the RLF mechanism is to follow the flow mentioned above, i.e., first derive Qin and Qout by simulating PDCCH block error rate, and then detect in-sync and out-of-sync states by comparing DL quality with Qin and Qout thresholds to trigger/stop T310 timer.

Another simpler way, RAN2 can consider a similar method in [3], where the in-sync/out-of-sync indication is triggered by comparing average SINR within a given period with predefined thresholds Qin/Qout respectively. Based on the comparison, UE starts T310 timer when receiving N310 consecutive out-of-sync indications, and UE stops the running T310 timer, when receiving N311 consecutive in-sync indications. Due to the complexity and workload, we propose to apply the method in [3].

Proposal 1: RAN2 consider RLF based on the expiry of T310, which is started/stopped according to the comparison of SINR with Qout and Qin, respectively. The parameters given in TR36.839 can be used as a starting point.

| Parameter | Value |
| --- | --- |
| T310 timer | 1s |
| Qin | -6dB |
| Oout | -8dB |
| N310 | 1 |
| N311 | 1 |

Table 1: RLF parameters

### Applications of RLF prediction

To enable a proactive response from the UE/network to prevent or mitigate the impacts of a potential RLF event, the UE must predict the RLF occurrence significantly ahead of the present moment. This lead time, called as the 'prediction distance,' refers to the interval between the current time slot and the predicted time slot for the RLF event.

The following question is how to utilize the outcome of RLF prediction, there are two possible applications of AI RLF prediction:

- For Early HO to Prevent RLF: The prediction distance should be sufficiently long to allow the UE and the gNB adequate time to react, such as performing an HO procedure, before the channel quality deteriorates significantly, for instance, before the T310 timer starts running. This ensures that the network can maintain connectivity by managing the handover in advance of the RLF condition worsening.

- For Early RRC Reestablishment: The prediction distance can be shorter, allowing the UE to initiate RRC reestablishment proactively based on the RLF prediction, even while the T310 timer is running. This application focuses on quickly reestablishing a stable connection to anothr cell once an RLF is anticipated, thereby minimizing disruption.

Clearly, the predicted distance will affect the difficulty of AI prediction. It may be directly configured by the network or can be defined as a requirement so that the UE needs to ensure that the RLF prediction can achieve a certain accuracy at least over that designated predicted distance.

Observation 1: There are two possible applications of RLF prediction a)For Early HO to Prevent RLF and b)For Early RRC Reestablishment.  For (a), the predicted distance needs to be sufficiently long to allow the UE/gNB to respond—such as executing a Handover (HO) procedure—before the channel quality deteriorates. For (b), the required prediction distance can be shorter, enabling the UE to initiate early RRC reestablishment based on RLF prediction, even while the T310 timer is running.

Proposal 2: RAN2 discuss the utilization of RLF prediction, such as a) for initiating Early HO to Prevent RLF, and b) for triggering Early RRC Reestablishment. Other possibilities are not excluded.

Proposal 3: RAN2 consider different predicted distances i.e., the interval between the current time slot and the predicted time slot for the RLF event, for different applications.

### Direct RLF prediction

In the previous meeting, RAN2 agree to study both direct and indirect RLF prediction. For direct prediction, we use AI to directly predict whether RLF will occur in the future. We propose to consider the RLF prediction within a given prediction region instead of at an exact time slot, as shown in Figure 1. A prediction region refers to a temporal window within which AI is used to predict the likelihood of an RLF event occurring, rather than pinpointing its occurrence to an exact time slot. The prediction region is considered due to the inherent challenges and limited benefits of attempting to predict RLF events at precise time slots for several reasons:

- Trigger Mechanisms: RLF events are typically initiated based on long-term performance monitoring, such as through the T310 mechanism, rather than in response to sudden, short-term changes in channel quality at a specific time slot. This necessitates that AI algorithms learn and analyze channel quality over extended periods, making the task of accurately predicting the exact timing of an RLF event exceedingly difficult.

- Practical Utility: The practical benefits of accurately predicting the exact time slot of an RLF event are relatively minor. For critical applications like RLF prevention or early RRC reestablishment, the primary concern is to determine the likelihood of RLF events occurring in the near future rather than specifying their exact timing.

Given these considerations, it is deemed more practical and beneficial to focus on detecting RLF events within a given prediction region, such as within 80ms, 160ms, etc. The specific length of the prediction region can be determined based on various factors, including the use case requirements and the settings of the T310 timer. This approach allows for a more feasible and useful application of AI in predicting RLF events, aligning with the operational realities and limitations of current network monitoring and management practices.

Figure 1: Illustration of direct RLF prediction with prediction region

Observation 2: It is inherent challenges and limited benefits to predict the RLF event at a specific time slot. Instead, a more reasonable way is to consider the prediction of whether an RLF event occurs within a given prediction region.

For AI model input, since the RLF detection not only depends on the signal strength but also the interference from neighbor cells, it is more reasonable to consider the SINR as the AI model input instead of RSRP. This allows AI model to learn both signal and inference information from past measurements. For AI model output, it could be the probability (or YES/NO flag) of any RLF event occurring in the given prediction region.

Proposal 4: For direct prediction, AI predicts whether RLF events will happen in a given prediction region, i.e. a temporal window within which AI is used to predict the likelihood of an RLF event occurring. The predicted distance and the length of the prediction region could be FFS according to the application and other configurations, e.g., T310 timer.

Proposal 5:  For direct prediction, the AI model input is SINR of all cells and AI model output is the probability (or YES/NO flag) of any RLF event occurring at the given prediction region.

### Indirect RLF prediction

For indirect prediction, AI model is used to predict future measurements, i.e.., SINR of the serving cell, instead of RLF event directly. If AI approach can predict the serving cell SINR for continuous time slots as shown in Figure 2, we can detect RLF by checking in-sync/out-of-sync conditions with the conventional rule based on those predicted serving cell SINR. Similar to the direct prediction, the predicted distance depends on the applications and T310 timer. Also, to let AI learn the information of both signal and interference, we consider the AI model input to be the SINR instead of RSRP.

Figure 2: Illustration of indirect RLF prediction

Proposal 6: For indirect prediction, AI predicts the serving cell SINR. RLF can be detected based on the continuous predicted SINR. The predicted distance is FFS according to the applications and other configurations, e.g., T310 timer. The AI model input is the SINR of all cells and AI model output is the predicted serving cell SINR.

### Performance metric

For both direct and indirect prediction, the goal is to predict possible RLF event in the future. As mentioned in Sec 2.1.2, we think to predict the RLF event in a given region is more reasonable. Thus, we could consider the performance metric as “whether AI can detect an RLF event in a given prediction region” instead of the accuracy of predicting RLF at the exact time slot. Assume AI approaches generate an RLF detection at time t, we can check if there is any real RLF event occurring from time (t + predicted distance) to time (t + predicted distance + length of prediction region). F-score or detected/false alarm probability could be considered as the performance metric. Note that this metric can also be applied to indirect prediction. For indirect prediction, we can generate RLF prediction by comparing the predicted SINR with Qin and Qout threshold and with T310 mechanism.

Proposal 7: For both direct and indirect prediction, consider performance metric as the accuracy of predicting RLF occurs in a given prediction region. Specifically, when AI generates RLF prediction at time t, check if any real RLF event will occur from time (t + predicted distance) to time (t + prediction distance + length of prediction region). F-score could be the performance metric, others can be FFS.

## Simulation setting

### Interference model

Compared to the RRM prediction use case, there is a key difference for RLF detection. For RRM prediction, we only care about the signal strength and the channel quality, e.g., RSRP. But to simulate RLF, we need to know not only the signal quality but also what is the interference from other cells. RAN2 needs to consider how to simulate the interference model, or the result of RLF prediction will vary from different companies, and thus hard to make a comparison. One of the simplest ways is to consider the pre-defined interference pattern through a given probability distribution according to the distance. However, this may be unrealistic, since the interference also depends on the number of UE serviced by the neighbor cell, the traffic load of neighbor cells, and the resource conflict situation. A more realistic way is to align the simulation setting. For example, consider the number of UE per cell, which is dropped randomly. We may assume those neighbor UE are static for simplicity. Consider the traffic load model, e.g., full buffer or 50% load, and the resource conflict ratio, e.g., 50% or 100%. Then companies can simulate interference based on those settings to achieve a relatively realistic interference model. However, this may result in a much higher workload for RAN2 study. But from our point of view, we should consider the interference model, since it can examine if AI approaches has the capability of learning not only signal and channel quality but also the interference pattern.

Proposal 8: RAN2 considers how to simulate the inference model, which is necessary for RLF prediction, since RLF is triggered based on not only signal and channel quality but also the inference from neighbor cells.

### Possible issue: Sparse dataset

As we know, a supervised learning approach needs a huge dataset to train the AI model. However, for RLF prediction problem, it is very hard to collect enough label data, since the RLF event occurs very rarely in a normal situation. We can observe that the typical value of T310 timer is 1s, and for normal situations, UE will perform HO before staying out-of-sync status for such a long time. Even if UE performs HO too late, it may probably result in HOF instead of RLF. This will result in the dataset only containing very few RLF events and it is very difficult for AI to learn from such sparse datasets. Of course, we can generate more dataset to collect enough RLF events, however, this dramatically increases the simulation effort, not only for dataset generation but also the training time. To reduce the simulation overhead, we propose RAN2 consider the UE simulation without HO behavior as a simplified method. This makes sure RLF occurs in almost all UE trajectories within a relatively short running time. Another relatively realistic approach is to consider UE with normal HO behavior and generate the channel model with more challenging scenarios, e.g., blocking, deep fading, etc., where the RLF event will occur more frequently. We think the RAN2 can consider the former as a starting point and the latter can be FFS due to the complexity.

Observation 3: With normal simulation, the dataset will contain very few RLF events since UE usually performs HO before staying in the out-of-sync state for such a long time. Even if UE performs HO too late, it will result in HOF instead of RLF.

Proposal 9: To collect the dataset with more RLF events, RAN2 could consider the case where UE does not perform HO behavior for simplicity. Other options, e.g., generating dataset with HO behavior as well as blocking, deep fading, can be FFS.

# Conclusion

Observations:

Observation 1: There are two possible applications of RLF prediction a)For Early HO to Prevent RLF and b)For Early RRC Reestablishment.  For (a), the predicted distance needs to be sufficiently long to allow the UE/gNB to respond—such as executing a Handover (HO) procedure—before the channel quality deteriorates. For (b), the required prediction distance can be shorter, enabling the UE to initiate early RRC reestablishment based on RLF prediction, even while the T310 timer is running.

Observation 2: It is inherent challenges and limited benefits to predict the RLF event at a specific time slot. Instead, a more reasonable way is to consider the prediction of whether an RLF event occurs within a given prediction region.

Observation 3: With normal simulation, the dataset will contain very few RLF events since UE usually performs HO before staying in the out-of-sync state for such a long time. Even if UE performs HO too late, it will result in HOF instead of RLF.

Proposals:

Proposal 1: RAN2 consider RLF based on the expiry of T310, which is started/stopped according to the comparison of SINR with Qout and Qin, respectively. The parameters given in TR36.839 can be used as a starting point.

Proposal 2: RAN2 discuss the utilization of RLF prediction, such as a) for initiating Early HO to Prevent RLF, and b) for triggering Early RRC Reestablishment. Other possibilities are not excluded.

Proposal 3: RAN2 consider different predicted distances i.e., the interval between the current time slot and the predicted time slot for the RLF event, for different applications.

Proposal 4: For direct prediction, AI predicts whether RLF events will happen in a given prediction region, i.e. a temporal window within which AI is used to predict the likelihood of an RLF event occurring. The predicted distance and the length of the prediction region could be FFS according to the application and other configurations, e.g., T310 timer.

Proposal 5: For direct prediction, the AI model input is SINR of all cells and AI model output is the probability (or YES/NO flag) of any RLF event occurring at the given prediction region.

Proposal 6: For indirect prediction, AI predicts the serving cell SINR. RLF can be detected based on the continuous predicted SINR. The predicted distance is FFS according to the applications and other configurations, e.g., T310 timer. The AI model input is the SINR of all cells and AI model output is the predicted serving cell SINR.

Proposal 7: For both direct and indirect prediction, consider performance metric as the accuracy of predicting RLF occurs in a given prediction region. Specifically, when AI generates RLF prediction at time t, check if any real RLF event will occur from time (t + predicted distance) to time (t + prediction distance + length of prediction region). F-score could be the performance metric, others can be FFS.

Proposal 8: RAN2 considers how to simulate the inference model, which is necessary for RLF prediction, since RLF is triggered based on not only signal and channel quality but also the inference from neighbor cells.

Proposal 9: To collect the dataset with more RLF events, RAN2 could consider the case where UE does not perform HO behavior for simplicity. Other options, e.g., generating dataset with HO behavior as well as blocking, deep fading, can be FFS.

# Reference

[1]	TS 38.300 NR and NG-RAN Overall description;

[2]  TS 38.133 Requirements for support of radio resource management

[3]	TR 36.839 Mobility enhancements in heterogeneous networks