3GPP TSG RAN WG2 #126	R2-2406583

Maastricht, Netherlands, August 19th – 23rd, 2024

Source:	Panasonic

Title:	MBService area indication & geoblocking

Agenda Item:	8.8.4 Support of Broadcast service (NTN for NR Ph3)

Document for:	Discussion, decision

# 1. Introduction

In conjunction with agenda item “8.8.4 Support of Broadcast service” we reached the following two conclusions (besides other agreements) during the most recent RAN2 meeting #126 in Fukuoka:

- Come back next meeting to check whether a UE may not (or should not) establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area)

- Come back next meeting to check whether a UE may (or should) release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area)

The question that is derived from the two come-back items above is, if the area indication realized with circles and/or polygons – as agreed in Changsha (meeting #125-bis) for further consideration – is sufficient for geoblocking/geofencing (from here on we refer to “geoblocking” only) purposes or not. We believe it is not sufficient and propose a more accurate approach. On that basis, the two come-back topics above would be resolved.

# 2. Discussion

# 2.1 Background

# Usually, licenses for broadcasting movies/films are connected to a geographical area, within which that reproduction is permitted for the license fee that has been agreed between the service/programme item provider (ZDF, BBC, NHK, …) and the film company (Warner Bros., Universal, Metro-Goldwyn-Mayer, …).

# Hence the network/service/content provider has to make sure that the content in question can’t be reproduced elsewhere. Furthermore, there might be payments connected to having access to the service or programme item. So the responsible provider needs to assure that the related payment has been executed and the corresponding descrambling means become available to the entitled users.

# Typical means in combination with traditional TV systems in use is the application of Conditional Access (CA), which covers both aspects – making sure that the user of the affected UE has paid for the content she/he wants to watch/hear and that the user sits inside the area the reproduction is permitted within. Whereby the latter aspect – usually being connected to the users’ home address – is a bit fuzzy and not really applicable to mobile devices in the same way as to traditional TV sets with their limited mobility.

# Observation 1:	Traditional (usually TV-connected) means of geoblocking – based on users’ home addresses – might not be sufficient for mobile devices.

# Observation 2:	For making sure that the user has paid for the content she/he wants to watch/listen to, CA systems are applicable.

# 2.2 Rationale

# Reproduction of a(n) (audio/video or radio) service (or single programme item) on a UE shall be restricted to
a) inhabitants of a certain area, e.g. a country,
b) users who paid for the content,
b) that aforementioned area, e.g. a country (in case users travel abroad, they shall not be able to access the service in question anymore in the other area).

# Observation 3:	A mechanism is required for verifying the fulfilment of multiple conditions entitling a user/UE for watching/reproducing certain content.

# Proposal 1:	Define a geoblocking solution fulfilling the requirements for a) determining UE’s position within or outside of a specified (home) area and b) for optionally checking the execution of a potentially required payment for the MBS content in question.

# 2.2 Geoblocking proposal

# 2.2.1 Area indication

Countries around the world and their administration geographies are structured in a similar way – on the highest layer there is the country itself, on the layer below there are federal states, provinces, prefectures, you name it. See figure 1 below for Europe and its neighbourhood – with Germany being the highlighted example here. Usually, these two highest layers are sufficient for the purpose of geoblocking/geofencing. Nonetheless, these layers continue down to a very fine granularity. The corresponding geographical information about the borders of the aforementioned structures is available from public databases. Hence the need for a related indication in the form of circles or polygons seems somewhat redundant. Furthermore, the outlined geo database approach saves signalling resources, i.e. the indication of multiple circles and polygons would not be required. Instead, a URL for the related database entry would be signalled, e.g. the entry describing the border between Germany and its neighbouring countries.

Figure 1: Administrative areas (two top layers) in Europe and neighbouring regions

Observation 4:	An indication of an intended MBS service area by means of circles and/or polygons might be enhanced with pointing to a geographical database describing the borders of administrative areas precisely.

Proposal 2:	Define the option of indicating the URL of the desired entry (e.g. country or federal state) in a geographical database. That option could enhance the already agreed circle/polygon-based approach.

# Entitlement verification sequence

The network/service/content provider needs to make sure that the user/UE fulfills all requirements being connected to the reproduction of a service or a single programme item. Variants of that verification sequence may arise from a shift of responsibilities for assigning entitlements to users/UEs and from a deferral of accountabilities for verifying/falsifying the fulfillment of the outlined requirements. These responsibilities might be assigned to providers of services and programme items or network providers – based on corresponding functions/APIs. In other words, either the network overtakes tasks the content/service provider assigns to the network or the network provides functions/APIs allowing the content/service provider to overtake control via these interfaces. Also mixed cases of the aforementioned structures are possible. The example below – including figure 2 – is agnostic regarding the entity that controls access to services/programme items. For abstraction purposes the entity that has control is called “control entity” here.

Example:	A certain programme item, e.g. a movie – call it “Lost in Space” – is available between 20h00 and 22h00 local time via service Orbital Television to users within area X who have paid 10 US-Dollars for watching it.

Step 1: The control entity asks the UE to provide information about its geographical position - on GNSS basis. The control entity checks – on geodata basis - if the UE is located within target area X. If that is not the case, the service/programme item will be indicated in the service list as not reproducible (reproduction – if underway – would be stopped). MRBs – if established beforehand – would be released. Otherwise continue with step 2.
Step 1 needs to be repeated in intervals due to the mobility of the UE.

Step 2: If deemed necessary, the control entity may check by means of network-verified UE location, if the geo-position indication received from the UE beforehand (-> step 1) is reliable. Should that be the case, the data making up service and programme item Orbital Television: Lost in Space would be forwarded to subsequent stages (for e.g. CA handling), otherwise indicated as non-reproducible. In the latter case, reproduction underway would be stopped and related MRBs released.

Figure 2: Release/establishment of MRBs (as far as RAN2 is concerned), light yellow boxes executed by control entity, others by UE

Observation 5:	All listed requirements for reproducing a service or a single programme item of a service only within a predefined geographical area can be fulfilled with a verification sequence (see above).

Observation 6:	The role of the circle/polygon-based area indication as agreed during RAN2#126 has been reviewed and an alternative/additional proposal has been described that fulfills the requirements of geoblocking/-fencing in a more accurate and resources-saving manner.

Observation 7:	To answer the two come-back issues listed in clause 1, the introduction to this paper, the circle/polygon approach for indicating an intended service area for an MBService is seen as not accurate enough for enabling or preventing users’ access to geoblocked broadcast content. Therefore that rough area indication shall not build the basis for establishing or releasing MRBs on UE side.

Proposal 3:	Geoblocking broadcast content should be based on the described database approach for indicating administrative areas more accurately. This control is not required to be carried out by the RAN layer.

# 3. Conclusion

In this document, we discussed two come-back issues from last meeting in conjunction with the establishment and release of MRBs in case of entering/leaving an intended MBService area. At the same time, we have proposed a more accurate mechanism for geoblocking than the one feasible on circle/polygon-based area indication.

The following concrete observations and proposals were made:

Observation 1:	Traditional (usually TV-connected) means of geoblocking – based on users’ home addresses – might not be sufficient for mobile devices.

Observation 2:	For making sure that the user has paid for the content she/he wants to watch/listen to, CA systems are applicable.

Observation 3:	A mechanism is required for verifying the fulfilment of multiple conditions entitling a user/UE for watching/reproducing certain content.

Observation 4:	An indication of an intended MBS service area by means of circles and/or polygons might be enhanced with pointing to a geographical database describing the borders of administrative areas precisely.

Observation 5:	All listed requirements for reproducing a service or a single programme item of a service only within a predefined geographical area can be fulfilled with a verification sequence.

Observation 6:	The role of the circle/polygon-based area indication as agreed during RAN2#126 has been reviewed and an alternative/additional proposal has been described that fulfills the requirements of geoblocking/-fencing in a more accurate and resources-saving manner.

Observation 7:	To answer the two come-back issues listed in clause 1, the introduction to this paper, the circle/polygon approach for indicating an intended service area for an MBService is seen as not accurate enough for enabling or preventing users’ access to geoblocked broadcast content. Therefore that rough area indication shall not build the basis for establishing or releasing MRBs on UE side.

Proposal 1:	Define a geoblocking solution fulfilling the requirements for a) determining UE’s position within or outside of a specified (home) area and b) for optionally checking the execution of a potentially required payment for the MBS content in question.

Proposal 2:	Define the option of indicating the URL of the desired entry (e.g. country or federal state) in a geographical database. That option could enhance the already agreed circle/polygon-based approach.

Proposal 3:	Geoblocking broadcast content should be based on the described database approach for indicating administrative areas more accurately. This control is not required to be carried out by the RAN layer.

- References

[1]	RP-240776	Revised WID on Non-Terrestrial Networks (NTN) for NR-NTN Phase 3

[2]	R2-126 NR-NTN-IoT-NTN (Sergio) EOM.docx