3GPP TSG-RAN WG2 Meeting #127	R2-2406590

MAASTRICHT 19.08.2024-23.08.2024

Source: 	Vodafone, Qualcomm, Nokia, Interdigital, Xiaomi, Samsung, Deutsche Telekom

Title: 		Energy Status report

Document for: Decision

Agenda Item: 8.2.2 - Functionality aspects

## Introduction

## During RAN WG2#126, 3gpp reached the following agreements:

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective.   FFS how we handle segmentation case (if needed)

As highlighted in R2-2405466, one of the reasons why the UE/tag might need to implement buffer status report like functionality is that energy stored in the device might not be sufficient to deliver all needed data.

“ 2) the energy stored at the device side is not enough for all data transmission at one time. In this case, the reader should be aware of the data volume for the UL data transmission. Thus, BSR-like function can be introduced so that the device can indicate the required resource for the UL data transmission.”

## Also R2-2405233 highlights the need for the system to receive the energy status for efficient scheduling

## Discussion

In addition to the reasons above, there might be other situations where the stored energy would not be sufficient to proceed with the communication:

Once the paging is received the device (in case it is in coverage) might only have energy for MSG1. As already assumed this message might have random number generated by the UE. If the reader would know that the device is not capable to process any additional messages, it could abort the access procedure and not send message 2. The tag itself once the MSG1 is sent could stop listening to the paging’s and try to re-charge.

Picture 1

Besides saving access signalling and reducing the number of paging repetitions, the information regarding energy status could also be provided to the core network and might be used to adapt deployments of CW/reader nodes, especially in case that many devices under a particular CW/Reader reports that several/many devices/tags do not have sufficient energy for communication.

Picture 2

The other situation is that the UE/tag can deliver MSG3 including the device ID, but not having any energy for any other communication. From the reader point of view, it would be important to know the status in order not to consider it during command delivery to such UEs. Also in this case, the information about the energy status could be provided to the core network. It is then up to the network implementation if it makes sense to start e.g. command procedure to the remaining UEs or wait before more devices are capable to receiver commands.

The third situation is when the device is close to being out of energy during data communication e.g. delivering the information requested by the commands. In this situation, such a status might be of importance to understand if the command procedure could be finalised from the user point of view.

Picture 3

Some other considerations:

The device may know how much energy is needed to send MSG1 as most likely it would have a pre-defined value. The same is valid for MSG3. In both of these cases, it might be sufficient for the device just to include a bit reporting to the network that there is no energy. In case of the command/data deliver, it might not be important to know how much energy is left, but just the fact if the UE has an energy for the same amount of data as delivered with the last message being able to be transmitted.

In all these cases, the energy needed by the device to provide energy status can be considered as minimum as the energy status can be expressed using just 1 or a few bits

Proposal 1: It is proposed to include energy status report to be delivered from the device to the reader in case the device does not have sufficient energy for subsequent operations.

Proposal 2: It is proposed to discuss if one bit of information is sufficient for any use case highlighted above or there is a need to represent the energy status in a different way (e.g. sufficient energy for sending of a particular number of bits or an expected time when the device will be ready for subsequent operations or any others)

## Conclusions

Proposal 1: It is proposed to include energy status report to be delivered from the UE to the reader in case there the device does not have anergy for the follow up messages

Proposal 2: It is proposed to discuss if one bit of information is sufficient for any use case highlighted above or there is a need to represent the energy status in a different way (e.g. sufficient energy for sending of a particular number of bits or an expected time when the device will be ready for subsequent operations or any others)

References:

R2-2405233: Nokia “On AIoT Functionality Aspects”