3GPP TSG-RAN WG2 Meeting #127                                    	R2-2406593

Maastricht, The Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.9.3

Source:	Deutsches Zentrum für Luft- und Raumfahrt (DLR) e.V.

Title:	Performance of Advanced Random Access Protocols

Document for:	Discussion and Decision

- Introduction

RAN WID [1] has following objective for the EDT enhancements.

Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signalling to complete an Early Data Transmission (EDT) transaction [RAN2]:

- Msg3 transmission without msg1/ Random Access Response (RAR)

In [2] it was proposed to study EDT enhancements based on Diversity Slotted Aloha and Contention Resolution Diversity Slotted Aloha [4,5]. In this document, we provide some additional results on the performance gains that the two methods can provide, which need to be discussed.

- Discussion

Performance Improvements via Diversity Slotted Aloha

As observed in [2,3], the use of packet replication can drastically improve the performance of Msg3-EDT transmission w/o Msg1 in NB-IoT. Packet replicas can be introduced with minor changes to the standard, leaving several possibilities at the receiver end on how to take advantage of the replication.

The simplest approach exploits packet replicas to achieve collision diversity, reducing the packet error rate of plain slotted Aloha at moderate channel loads. In this approach, the receiver does not require any modification with respect to a legacy slotted Aloha receiver. This approach is commonly referred to a diversity slotted Aloha (DSA) [4].

A minor modification of this approach, referred to a contention resolution diversity slotted Aloha (CRDSA) [3,4] applies interference cancellation techniques at the receiver side to drastically improve the gains achieved by DSA. The implementation of CRDSA receiver is nowadays well understood, as CRDSA has been part of the DVB-RCS standard since several years (second release of DVB-RCS [3]). 
Note that, to implement interference cancellation, packet copies should contain a field pointing to the position of the packet copy (copies). Alternatively, the position of the packet copies can be obtained by applying a hash function (to be specified) to the packet payload, where the hash function generates the indexes of pointing to the EDT resources where packet copies are transmitted. The use of a linear congruential generator to implement such a behavior was already adopted by the DVB-RCS2 standard [3].

Observation 1: To provide the indexes of the EDT resources where the packet copies are transmitted, one possibility is to employ a hash function that generates the indexes as a function of the packet payload.

Figure 1 reports the packet error rate achieved by slotted Aloha, DSA, and CRDSA with packet. For DSA and CRDSA, packets are transmitted twice (two replicas). The results are obtained via layer-2 Monte Carlo simulation. The physical layer has been abstracted by (a) assuming that collision results in the impossibility of correct decoding and (b) that transmission w/o collision incurs in a packet error probability of 1%. Furthermore, under CRDSA, ideal interference cancellation is assumed. The latter assumption typically yields realistic performance estimates [4,5], especially considering that the signal processing chain required to implement interference cancellation in CRDSA is particularly simple, with channel estimation that can be reliably obtained by exploiting the decoded data.

The parameters used for the simulation are summarized in Table 1. The channel load G is measured in packets/slot, and it is obtained as

G=Number_of_active_usersNumber of EDT resources per occasion*Number of EDT occasions

where Number_of_active_users denotes the number of attempted packet transmissions, and where the denominator, according to the parameters of Table 1, is 12*4 = 48. Note that a fixed value of G refers to a fixed Number_of_active_users. Assuming a target PER = 10%, DSA achieves a two-fold gain over slotted Aloha, allowing to support twice the traffic of slotted Aloha. CRDSA yields a five-fold gain over slotted Aloha. By setting the target PER to lower values, the relative gain of DSA and CRDSA over slotted Aloha further increases. Note, in fact, that a target PER = 1% can be achieved, by slotted Aloha, only when no collisions can occur (owing to the single user PER = 1%), i.e., at a load G = 1/48  0.02. DSA achieves the same target PER at G  0.08 (4-fold gain), and CRDSA at G  0.16 (8-fold gain).

Observation 2: The gains provided by DSA/CRDSA over slotted Aloha are large already at high PER regimes, and tend to become larger as the target PER is lowered.

Figure 2 reports a comparison of CRDSA with 2 and 3 replicas. The case with 3 replicas is providing gains over 2 replicas which range from about 20% at target PER = 10%, to about 200% at target PER = 1%. Hence, 3-replicas CRDSA achieves a 23-fold gain over slotted Aloha, at target PER = 1%.

Table 1 Configuration A parameters

| Parameter | Value |
| --- | --- |
| Number of EDT resources (carriers) per occasion | 12 |
| Number of EDT occasions | 4 |
| Replicas | 2 and 3 |
| Single-user PER (w/o interference) | 0.01 |
| SIC | 5 iterations |

Figure 1 Packet Error Rate vs. Channel Load, Configuration A. CRDSA and DSA with 2 replicas.

Figure 5 Packet Error Rate vs. Channel Load, Configuration A. Comparison between 2 and 3 replicas.

More advanced techniques to exploit the energy used to transmit packet copies for improving the energy efficiency of CRDSA-like protocols are discussed in [6].

Proposal 1: RAN2 to study improved random access protocols  for Msg3-EDT transmissions without msg1/ Random Access Response, with focus on low-complexity techniques as DSA and CRDSA.

- Conclusion

Following observations and proposals are made:

Observation 1: To provide the indexes of the EDT resources where the packet copies are transmitted, one possibility is to employ a hash function that generates the indexes as a function of the packet payload.

Observation 2: The gains provided by DSA/CRDSA over slotted Aloha are large already at high PER regimes, and tend to become larger as the target PER is lowered.

Proposal 1: RAN2 to study improved random access protocols for Msg3-EDT transmissions without msg1/ Random Access Response, with focus on low-complexity techniques as DSA and CRDSA.

- References

[1] RP-240776, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, March 18-21, 2024.

[2] R2-2402811, “Discussion on EDT enhancements”, Qualcomm INC., 3GPP TSG-RAN WG2 Meeting #125bis, Changsha, China, April 2024.

[3] R2-2405202, “Discussion on EDT enhancements”, ESA, 3GPP TSG-RAN WG2 Meeting #126, Fukuoka, Japan, May 2024.

[3] ETSI EN 301 545-2, “Digital Video Broadcasting (DVB); Second Generation DVB Interactive Satellite System (DVB-RCS2); Part 2: Lower Layers for Satellite standard”.

[4] E. Casini, R. De Gaudenzi and O. Del Rio Herrero, "Contention Resolution Diversity Slotted ALOHA (CRDSA): An Enhanced Random Access Schemefor Satellite Access Packet Networks," in IEEE Transactions on Wireless Communications, vol. 6, no. 4, pp. 1408-1419, April 2007.

[5] G. Liva, “Graph-Based Analysis and Optimization of Contention Resolution Diversity Slotted ALOHA,” in IEEE Transactions on Communications, vol. 59, no. 2, pp. 477–487, February 2011.

[6] P. Agostini et al., “Evolution of the 5G New Radio Two-Step Random Access towards 6G Unsourced MAC,” May 06, 2024, arXiv: arXiv:2405.03348. Also available at http://www.smalldatanet.com.