3GPP TSG-RAN WG2 Meeting #127	R2-2406594

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:	8.7.4

Source:		Lenovo

Title:			Enhanced uplink scheduling for XR

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss enhancements to the uplink scheduling for XR traffic. A detailed look is taking to the  logical channel prioritization (LCP) procedure.

# Discussion

The Rel-19 Work Item on XR Phase 3 was agreed in RP-240791.

The Rel-19 XR Phase 3 objectives are as follows: Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2].

In RAN2#125bis initial discussion on the UL scheduling related enhancements for Rel-19 took place with the following agreements:

- RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied: - Alternative 1: Enhance LCP restrictions/LCH selection. - Alternative 2: Enhance LCH prioritization. - RAN2 should consider potential impact on traffic from SRBs. - RAN2 will study enhancing existing DSR with additional information, e.g. multiple pairs of remaining time/buffer information, importance. FFS whether this only includes more information on delay-critical data or also information about non-delay critical data.

In RAN2#126 the following agreements were reached:

- Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. - The solution should consider impact on UE complexity (as already indicated in SI objective description) - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data - FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH - We try to avoid RAN1 impacts. - RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. - LCP prioritization within a logical channel will not be considered in RAN2 discussions - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included

### LCP procedure

As concluded in RAN2#126 meeting, delay-aware LCP enhancements to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data are supported in Rel-19 XR.

As already discussed in the last meetings, the issue with the current LCP procedure is that only the logical channel priority (partly also Bj) is used to determine the order (and amount of data) in which packets from different LCHs are multiplexed in a TB/MAC PDU. The remaining delay of a data packet, w.g. RLC PDU is not considered when distributing UL resources to the LCHs.

For XR applications, it is important to ensure that packets/PDU sets are received within the associated PDU set/packet delay budget, e.g. PSDB.  Since application layer doesn’t benefit from packets which are received beyond its PSDB, e.g. packets are dropped, it’s of vital importance that data packets are successfully received within the associated delay boundaries. Given that the LCP procedure currently prioritizes data of different LCHs only based on the associated static LCH priority, it may happen that UE is unable to transmit data of a lower priority LCHs having a small remaining delay within its PDSB/PDB requirements if there is also data of a higher priority LCH pending in UEs buffer for transmission. Even though the higher priority data may have a large remaining delay and hence there is sufficient time for the transmission of the data, UE will prioritize the high priority data and potentially not assign any UL resources to the lower priority data which is close to its delay boundary. In particular for scenarios, where pdu-SetDiscard is configured, this may in turn result in PDUs /PDU sets being discarded. UL transmissions which have been already carried out for PDUs of a PDU set which is finally discarded due to exceeding its PSDB have a negative impact on the system capacity.

Even though the DSR procedure, which was introduced in Rel-18 to provide the serving gNB with delay status information, is already a tool to improve the uplink scheduling efficiency and also the uplink capacity, the full potential will be only exploited if also the LCP procedure considers remaining delay/time of the data. When using the legacy LCP procedure, it may still happen that UE can’t transmit the complete delay-critical data – reported in a DSR - on the UL resources allocated by the gNB in response to the reception of the DSR MAC CE, if there is higher priority data sitting in UEs’ buffer which is not delay-critical (and hence not being reported within the DSR MAC CE).

Therefore the remaining time of a PDU/PDU set should be considered to determine the priority of a LCH respectively PDU of a LCH.

Proposal 1: RAN2 to agree that remaining time of a PDU/PDU set should be considered when determining the priority of (data of) a LCH during LCP procedure.

Enhanced LCP procedure

- In RAN2#126bis meeting it was concluded that the option of overriding/adjusting the priority of a LCH during LCP based on the remaining time of the data of a LCH is used as a baseline for a delay-aware LCP procedure. UE may use an additional LCH priority configured to LCHs in case those LCHs have delay-critical data available for transmissions.

There was another option for LCP enhancements proposed, where new LCP restrictions were introduced.In this alternative, new LCP restrictions are introduced which should ensure that certain UL grants are only utilized by LCHs buffered with delay-critical PDUs/SDUs. It should be noted that the additional LCP restriction which is done during the LCH selection process/step it not based on a static configuration but based on the UE/MAC determining that there is data buffered for a LCH for which the remaining time until expiry of the associated PDCP discard timer is below a configured threshold. Furthermore, a new UL DCI needs to be introduced which explicitly indicates e.g. by a new field, that the UL grant is only allowed for LCHs which have been determined as delay-critical. Since the UL grant is only allowed to be used by certain LCHs for which delay-critical data is pending for transmission, we think that such additional restriction may impact the resource efficiency and capacity negatively. Furthermore, such approach may require some new UL DCI, e.g. RAN1 impact.  Given that RAN2 agreed that impact to RAN1 should be avoided, we propose that this option is not further pursued in Rel-19.

Proposal 2: The approach where additional LCH restrictions are introduced may have some negative impact on the capacity and has some impact to RAN1, i.e. new UL DCI format, and hence should not be further considered in Rel-19.

For the agreed baseline solution of an enhanced delay-aware LCP procedure, there could be different flavors on how the enhanced LCH prioritization is implemented. The general approach would be to adapt the LCH priority based on the remaining delay time of data within the LCH and perform the legacy LCP procedure with the (adapted LCH priorities). UE may for example increase the priority of a LCH in case the remaining delay/time of data of this LCH is becoming lower than a threshold or use a second configured priority. It should be noted that RAN2 would need to further discuss when MAC entity determines the LCH priority for a LCH for the enhanced LCP procedure. MAC may for example only determine the priority of a LCH by considering the remaining time of data buffered for this LCH once before the (legacy) LCP procedure is executed. This determination step would be similar to determining of a delay-critical LCH as mentioned above. In order to provide more fairness, MAC may determine the priority of a LCH (by considering the remaining time of data buffered data) in addition before executing the second step of the LCP procedure, i.e. assigning UL resource in a strict priority order regardless of Bj . This would ensure that a LCH is not further prioritized during the second step of the resource assignment procedure (LCP) if all the delay-critical data of a LCH has been already multiplexed in the MAC PDU during the first step. This additional fairness would however come at the expense of a slightly increased complexity.

In our view, complexity is an important factor to consider when designing an enhanced LCP procedure, however it is also of equal importance to consider aspects such as fairness.

Proposal 3: RAN2 should discuss when MAC determines the priority of a LCH for/during a LCP by considering the delay status of the data of the LCH, e.g. only once before executing the LCP procedure or before each “round” of the LCP procedure.

RAN2 needs to also further discuss how to define a delay-critical LCH, e.g. when UE should apply a higher LCH priority for a LCH during LCP. In our understanding a LCH is considered a delay-critical LCH and should use a second higher LCH priority if there is data available for transmission for the LCH for which the remaining time is less than a configured threshold. We think that the threshold for determining a delay-critical LCH should be separate to the delay threshold used for the triggering of a DSR. The reason is that the DSR related remaining Time threshold needs to account for the delay incurred by SR transmission and reception of corresponding UL grant.

Proposal 4: RAN2 to agree that a delay-critical LCH is defined as a LCH having at least one PDU/PDU set available for transmission for which the remaining time till discardTimer expiry is less than the a configured threshold.  The threshold for determining a delay-critical LCH is a separate threshold than remainingTimeThreshold used for DSR triggering/reporting.

Regardless of which option is chosen as the detailed enhanced LCP procedure, we think that NW should control/configure if and for which LCHs the remaining time should be considered when executing the LCP. For example, the remaining time doesn’t need to be considered during LCP for every kind of service/LCH, e.g. only for specific LCHs carrying XR traffic. As another example, the enhanced LCP procedure may be only used when DSR has been reported to the gNB in order to ensure that delay-critical data – as reported in the DSR – is transmitted in the corresponding UL grant, e.g. received in response to the DSR. It should be also further discussed whether the enhanced LCP procedure is explicitly activated/deactivated by NW, e.g. similar to the PSI-based discarding procedure by a MAC CE.

Proposal 5: NW should control/configure whether the enhanced LCP procedure taking remaining delay/time into account is used or not by the UE.  Further, NW can configure whether remaining time should be considered during LCP procedure for a LCH.  RAN2 to further discuss whether enhanced LCP procedure is explicitly activated/deactivated by NW.

Similar to the considerations for the LCP procedure, also for the intra-UE prioritization mechanism which was introduced in Rel-16 it should be also discussed whether to consider the remaining time/delay of the data. According to the current specification the priority of an overlapping UL grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, i.e., only the logical channel priority is considered to determine which UL grant is prioritized/deprioritized.

However, it doesn’t make sense to prioritize an UL grant which carries data for which the PSDB/PDB is already exceeded. Such UL grant should be rather deprioritized. Similarly, a MAC PDU/UL grant which carries delay-critical data or a DSR MAC CE should be treated as a prioritized grant in the intra-UE prioritization procedure.

Proposal 6: RAN2 should discuss enhancements to the intra-UE prioritization procedure, e.g. considering the remaining delay budget when determining the priority of an UL grant (prioritized/deprioritized UL grant).

The PDU set importance based PDCP discard has been introduced for Rel-18 XR. In case of UL congestion, PSI based discard can be activated, and a lower discard timer value, e.g. discardTimerForLowImportance is applied to the low importance data.  When PSI based discarding is activated, UE only supports DSR reporting for high importance data. Following the same principle, when LCP procedure considers the remaining time/delay of data prioritization, the enhancement may only be applied to high importance data.

Proposal 7: RAN2 to discuss whether the LCP enhancements is only applicable for high importance packets in case of UL congestion, e.g., PSI-based discard is activated.

### DSR enhancements

In RAN2#125bis meeting several companies proposed enhancements to the current DSR procedure. As already mentioned before, delay status reporting was introduced in Rel-18 to facilitate timely resource allocation by the gNB. The DSR MAC CE provides the smallest remaining time and total data buffered below the configured threshold for the LCG. Furthermore, low importance data – when PSI-based discarding is enabled – is not considered in the DSR, even though low importance data might also use allocated UL grant resources. A potential concern of the DSR reporting functionality as specified in Rel-18 is, that the DSR MAC CE doesn’t provide a full picture of UEs’ buffer and hence the gNB may not be able to efficiently assign UL resources in response to a received DSR.

In RAN2#126bis meeting, it was concluded to enhance DSR reporting by including multiple pairs of remaining time and buffer size information for a LCG in a report. When multiple PDU sets in the LCG have different remaining times UE may report remaining time information on a PDU set level if DSR is triggered for an LCH. RAN2 needs to further discuss whether UE may report multiple pairs of remaining and buffer size regardless of whether the remaining time is less than the configured remaining time threshold (DSR triggering), so that gNB is provided with the status of both delay-critical and non-delay-critical data pending for transmission. Correspondingly, NW can allocate UL resources accounting for the detailed delay information provided in the DSR, and UE performs resource allocation based on delay status, e.g. enhanced LCP, in response to receiving the UL grant. Therefore, it is beneficial to provide additional pairs of delay information in an enhanced DSR even though this would increase the signalling overhead.

Proposal 8: DSR information reporting should be done on a PDU set level.  It needs to be further discussed whether UE reports a pair of remaining time and corresponding buffer size when remaining time is larger than configured remainingTimeThreshold.

In Rel-18, DSR reporting of the LCH is triggered when the smallest remaining delay of a PDU that has not been transmitted in any MAC PDU and has not been reported as data volume is below NW configured threshold. The threshold is configured per LCG. In Rel19, we consider the DSR triggering is not impacted by the new DSR format.

Proposal 9: DSR triggering is not impacted by new DSR format.

In Rel-18, PSI information is not included in a DSR. If it is reported in DSR, NW can be aware of PSI information of data associated with the reported delay status and may perform resource allocation considering also the importance of the data. Even when PSI level would be provided together with delay status information, the full potential will be only exploited if also the LCP procedure considers the PSI of the data, which is however not the in the scope of Rel-19. Therefore, we are not sure that there sufficient benefit of providing PSI information within the new DSR report for the extra overhead. RAN2 can further discuss whether PSI should be included in new DSR format.

Proposal 10: RAN2 to further discuss whether there is a sufficiently large benefit to include PSI information in the new DSR format.

# Conclusion

In this contribution, we discuss enhancements to the uplink scheduling. We have the following proposals:

Proposal 1: RAN2 to agree that remaining time of a PDU/PDU set should be considered when determining the priority of (data of) a LCH during LCP procedure.

Proposal 2: The approach where additional LCH restrictions are introduced may have some negative impact on the capacity and has some impact to RAN1, i.e. new UL DCI format, and hence should not be further considered in Rel-19.

Proposal 3: RAN2 should discuss when MAC determines the priority of a LCH for/during a LCP by considering the delay status of the data of the LCH, e.g. only once before executing the LCP procedure or before each “round” of the LCP procedure.

Proposal 4: RAN2 to agree that a delay-critical LCH is defined as a LCH having at least one PDU/PDU set available for transmission for which the remaining time till discardTimer expiry is less than the a configured threshold.  The threshold for determining a delay-critical LCH is a separate threshold than remainingTimeThreshold used for DSR triggering/reporting.

Proposal 5: NW should control/configure whether the enhanced LCP procedure taking remaining delay/time into account is used or not by the UE.  Further, NW can configure whether remaining time should be considered during LCP procedure for a LCH.  RAN2 to further discuss whether enhanced LCP procedure is explicitly activated/deactivated by NW.

Proposal 6: RAN2 should discuss enhancements to the intra-UE prioritization procedure, e.g. considering the remaining delay budget when determining the priority of an UL grant (prioritized/deprioritized UL grant).

Proposal 7: RAN2 to discuss whether the LCP enhancements is only applicable for high importance packets in case of UL congestion, e.g., PSI-based discard is activated.

Proposal 8: DSR information reporting should be done on a PDU set level.  It needs to be further discussed whether UE reports a pair of remaining time and corresponding buffer size when remaining time is larger than configured remainingTimeThreshold.

Proposal 9: DSR triggering is not impacted by new DSR format.

Proposal 10: RAN2 to further discuss whether there is a sufficiently large benefit to include PSI information in the new DSR format.

# References

TS 38.321 v18.0.0