3GPP TSG-RAN WG2 Meeting #127	R2-2406595

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:	8.7.2

Source:		Lenovo

Title:			Enhancements for support of Multi-Modal XR applications

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss potential RAN enhancements to efficiently support Multi-modal XR applications. Impacts to the LCP procedure, discarding and DRX operation are analysed.

# Discussion

In RAN#103 the Rel-19 Work Item on XR (eXtended Reality) for NR Phase 3 has been approved.

As part of the WI, enhancements addressing efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination are investigated.

The Rel-19 XR Phase 3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2].

initial discussions took place in the RAN2#125bis meeting with the following agreements:

- For the purpose of study, RAN2 assumes that UE and gNB have some kind of multi-modal information. FFS what information is needed/useful, e.g. just multi-modal ID, association between the flow, synchronization requirement etc. - RAN2 will study both UL and DL directions based on the assumption of multi-modality association knowledge at RAN/UE - RAN2 will focus on analysing potential usage and benefits (e.g. in terms of capacity and power saving) of multi-modal association knowledge - Areas to study include: synchronization between the flows, FFS impact on QoS insurance and other areas - RAN2 assumes that traffic of different modals having different QoS requirements is mapped to different QoS flows - For different XR traffic flows belonging to the same Multi-modal service and having different QoS requirements, it should be possible to provide differentiated QoS handling over the air. RAN2 should study if that is possible with current mechanism or new ones are needed - Existing QoS flow to DRB mapping framework is used as a baseline, i.e. up to gNB how to map QoS flows to DRBs

In RAN2#126 meeting RAN2 agreed after further discussions the following:

- Support Multi-Modality awareness in RAN in Rel-19 for UL and DL - Ask SA2 whether/what information could be provided to RAN

An LS to SA2/SA4 was agreed in R2-2405782, where RAN2 asks SA2 whether/what multi-modal information for UL and DL can be provided to RAN (e.g., MMSID and/or synchronization thresholds in order to allow e.g., coordinated handling of the flows at RAN).

### Multi-Modal XR applications

For Multi-Modal XR applications transmitted via a mobile communication system like NR the interactions between different input signals can be translated to some inter-dependencies between transmissions of different bearers/LCHs.
Due to the separate handling of the multiple media components, synchronization between different media components is critical in order to avoid having a negative impact on the user experience. As the asynchrony between different modalities increases, users’ sense of presence and realism will decrease.

The table below from TS22.847 depicts typical synchronization thresholds between two or more modalities.

Table 5.1.6-2: Typical synchronization thresholds for immersive multi-modal VR applications

|  | synchronization threshold (note 1) |  |
| --- | --- | --- |
| audio-tactile | audio delay: 50 ms | tactile delay: 25 ms |
| visual-tactile | visual delay: 15 ms | tactile delay: 50 ms |
| NOTE 1: For each media component, “delay” refers to the case where that media component is delayed compared to the other. |  |  |

For instance, for VR applications that require visual-tactile synchronization, the visual media component should not be delayed for more than 15ms when comparing to the tactile media component.

SA2 also discussed the support of multi-modal services. They introduced the concept of a Multi-modal Service ID which is an explicit indication that data flows being associated with such Multi-modal service ID are related to a multi-modal service. PCF may use this information to derive the corresponding PCC rules and apply appropriate QoS policies for the inter-related data flows being part of a multi-modal application. In the following the relevant text from TS23.501 is copied:

### 5.37.2 Policy control enhancements to support multi-modal services A multi-modal service is a communication service that consists of several data flows that relate to each other and that are subject to application coordination. The data flows can transfer different types of data (for example audio, video, positioning, haptic data) and may come from different sources (e.g. a single UE, a single device or multiple devices connected to the single UE, or multiple UEs). For the single UE case, it is expected that those data flows are closely related and require strong application coordination for the proper execution of the multi-modal application and therefore, all those data flows are transmitted in a single PDU session. The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, for each data flow that belongs to the multi-modal service, a Multi-modal Service ID, the service requirements and the QoS monitoring requirements: - The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and to apply appropriate QoS policies for the data flows that are part of a specific multi-modal application. - The AF may provide QoS monitoring requirements for data flows associated to a multi-modal service to the PCF . The PCF generates the authorized QoS Monitoring policy for each data flow. NOTE: In order to start the QoS monitoring for the data flows associated to a multi-modal service within a certain period of time, the PCF needs to receive the QoS monitoring requirements for those data flows from AF within a single request or, in case of multiple requests, within a short period of time.

As discussed and agreed in RAN2#126, Multi-Modality awareness is also supported in RAN for UL and DL, even though there is still the pending reply from SA2 regarding what multi-modal information can be provided to RAN.

This new synchronization requirements between the modalities and the related inter-dependencies among data of different modalities pose new challenges for an efficient support in RAN as discussed in the following.

### RAN enhancements for Multi-Modal XR applications

We think that in order allow an efficient support of multi-modal applications at the RAN level, RAN needs to be aware of the relations among different LCHs/RBs/QoS flows belonging to one multi-modal application. Essentially similar to the multi-modal service ID, also RAN needs to be made aware of a group of inter-related LCHs/RBs/QoS flows belonging to a multi-modal application. Even though RAN2 is awaiting the reply from SA2 from our point of view, we think that the multi-modal awareness/information should include information of the relations among different LCHs/RBs/QoS flow belonging to a multi-modal application (similar to MMSI information) as well as the corresponding synchronization requirements, e.g. relative delay requirements, between the different LCHs/RBs/QoS flows belonging to one multi-modal application.

In our view, the awareness of synchronization requirements among multiple modalities is not only required to ensure the synchronized delivery of packets of multiple inter-correlated flows and hence to allow for an optimized user experience but may represent an additional opportunity for RAN to save radio resource. Specifically, some enhanced discarding operation considering the synchronization requirements may contribute to a capacity improvement, e.g. when a packet of one of the flows is discarded, or when a packet cannot be transmitted before the synchronization deadline, RAN/UE may discard some of the pending packets of the group of inter-related flows as they may no longer be useful from Application perspective.

As mentioned before the feasibility of transferring the multi-modal information such as MMSID and/or synchronization thresholds in order to allow e.g., coordinated handling of the flows at RAN, from CN to RAN still needs to be confirmed by SA2. However, we assume that similar to the PSDB, PSIHI information also some multi-modal information could be provided as PDU Set QoS Parameters by the SMF to the gNB as part of the QoS profile of a QoS flow. Depending on level of granularity required for the inter-dependency information PDU set information signalled within the  GTP-U header may be also a candidate for conveying multi-modal information, e.g. PDU set A of LCH X is related to PDU set B of LCH Y. A motivation to provide the inter-dependency information on a PDU set level is, that resource allocation/scheduling is usually done on a PDU/PDU set level. For an efficient resource allocation, e.g. RAN scheduling DL transmissions, it would be best to provide the dependency information on a PDU set level. This would allow for a very accurate resource allocation/scheduling. This would however come at the expense of an increased signalling overhead, e.g. PDU set information in GTP-U header is increased.

Proposal 1: The level of granularity of the inter-dependency information, e.g. PDU set level or QoS flow level, needs to be further discussed.

In general, such multi-modal information could be also provided by the UE as well. By extending the UAI of uplink traffic information defined in Rel-18 XR, the UE could additionally indicate whether a QoS flow has a synchronization requirement with at least one other QoS flow(s). However, we are not sure, whether it is necessary to actually extend the UE assistance information reporting. Since it will anyways depend on the reply from SA2, we think that this should be discussed later.

Proposal 2: Whether an extension of the UAI, to provide the assistance information relating to multi-modal synchronization among multiple uplink QoS flows to RAN, is needed/beneficial should be discussed after having the received the reply from SA2.

### QoS flow – DRB mapping

In the table from TS22-847 below it is shown that different modalities of a XR Multi-Modal application, e.g. audio, video, haptics, have different QoS requirements.

Table 5.7.3-1 Typical QoS requirements for multi-modal streams [14] [15] [16] [17] [18]

|  | Haptics | Video | Audio |
| --- | --- | --- | --- |
| Jitter (ms) | ≤ 2 | ≤ 30 | ≤ 30 |
| Delay (ms) | ≤ 50 | ≤ 400 | ≤ 150 |
| Packet loss (%) | ≤ 10 | ≤ 1 | ≤ 1 |
| Update rate (Hz) | ≥ 1000 | ≥ 30 | ≥ 50 |
| Packet size (bytes) | 64-128 | ≤ MTU | 160-320 |
| Throughput (kbit/s) | 512-1024 | 2500 - 40000 | 64-128 |

According to RAN2#125bis discussions RAN2 assumes that traffic of different modals having different QoS requirements are mapped to different QoS flows. Even though this might need to be confirmed by SA2, we think that RAN2 can further work with this assumption. Furthermore, it was agreed that it should be possible to provide a differentiated QoS handling on the Uu interface for different XR traffic flows belonging to the same Multi-modal service having different QoS requirements. Basically, we think that this means that QoS flows requiring a differentiated QoS handling on the Uu interface should be mapped to different DRB, which is inline with the current QoS mechanism. So in general we think that modals of a MM application having different QoS requirements will be mapped to different DRBs.

On the other hand, there are proposals to map QoS flows carrying different modals of the same multi-modal service to one radio bearer in order to better address the new synchronization requirements.  It is argued that it would be easier to satisfy the synchronization requirements when all the data of the different modals are transmitted via the same DRB/LCH. Furthermore, any potential enhancements for discarding or other coordinated treatment of multi-modal data could be easier implemented within one common e.g. PDCP entity rather than introducing some inter-DRB/PDCP communication/procedures which would be needed when the related QoS flows would be mapped to different DRBs.

In our understanding there are issues which would need to be addressed when mapping QoS flows associated with the same multi-modal service but having different QoS requirements to the same DRB. For example, currently there is only one common PDCP discardTimer configured for a PDCP entity (exception is the discardTimerlowImportance). However, since the different QoS flows mapped to the DRB may have quite different QoS requirements as shown above, it would be very inefficient to apply a common PDCP discardTimer value for the different data. At least multiple different PDCP discardtimer would need to be configured for the PDCP entity for QoS flows with different QoS requirements. Similarly, the reordering operation would be inefficient since only one reordering timer is supported currently, e.g. same t-reordering is used regardless of the different QoS requirement of the PDUs/SDUs.

It should be also noted that RAN2 agreed in RAN2#126 that no prioritization within a LCH is further considered in Rel-19. That means that it will not be possible to further enhance LCP for the support of multi-modal services when multiple QoS flows carrying the different modals of a MM service are mapped to the same LCH.

- LCP prioritization within a logical channel will not be considered in RAN2 discussions

Apart from the different latency requirements (PDCP discardTimer), different priority and/or reliability requirements of the different QoS flow would also need to be addressed when mapping multiple QoS flows with distinct QoS requirements to the same radio bearer, e.g. data of QoS flows with different QoS requirements are mapped to a different RLC entity to support a differentiated packet handling.

In light of the issues mentioned above we are not sure that mapping the QoS flows of a MM service to one RB will be easier. Therefore, we think that different modals of a MM service having different QoS requirements should be rather mapped to different DRBs.

Proposal 3: Different modals of a MM application having different QoS requirements should be mapped to different DRBs.

### UL scheduling enhancements for Multi-Modal applications

According to the current legacy LCP procedure UE shares UL resources among the configured LCHs based on the LCH priority and the prioritized bit rate configured for a LCH. The idea behind prioritized bit rate is to support for each logical channel, including low priority non-GBR (Guaranteed Bit Rate) bearers, a minimum bit rate in order to avoid a potential starvation. Each bearer should at least get enough resources to achieve the prioritized bit rate (PBR). The LCP procedure is in general specified as a “two-step” procedure, e.g. not considering the step of selecting the LCHs which are considered during the LCP procedure for TB generation based on the configured LCH restrictions. In the first round the LCHs are served (in decreasing priority order starting with the highest priority logical channel) up to their configured PBR (implemented by means of a token bucket model). In the second round of the LCP if any uplink resources remain (after meeting the PBR of the LCHs in the first round), all the logical channels are served in a strict decreasing priority order (regardless of the value of bucket) until either the data for that logical channel or the UL grant is exhausted.

As analyzed in detail in the companion contribution (reference to LCP contribution), the current LCP procedure doesn’t consider the remaining delay/time of a data packet when distribution UL resources among the LCHs.

Given that the LCP procedure prioritizes data of different LCHs only based on the associated static LCH priority, it may happen that UE is unable to transmit data of a lower priority LCHs having a small remaining delay within its PDSB/PDB requirements if there is also data of a higher priority LCH pending in UEs buffer for transmission. This may have a negative impact on the system capacity. For multi-modal applications the impact on the capacity might be even more pronounced, since there is not only an absolute delay requirement for LCHs being part of a multi-modal application, but there is in addition also a relative synchronization requirement between the inter-dependent LCHs.

Therefore, we think that the LCP procedure needs to be enhanced in order to efficiently support multi-modal services in the RAN. The LCP procedure should take into account the absolute remaining delay/time of data as well as the relative remaining delay, e.g. enforcing the Multi-Modal synchronization requirement, in order to determine the priority order in which LCH data are multiplexed on the UL resources. It should be for example ensured that data of a LCH being part of a multi-modal application which is close to its relative delay requirement is transmitted within the synchronization threshold and hence prioritized during LCP. The priority of the data should be during LCP procedure adapted depending on the absolute remaining time and the relative remaining time (synchronization threshold).

Proposal 4: RAN2 to discuss enhancements to the LCP procedure, where the absolute remaining time and in addition the relative remaining time, e.g. enforcing the multi-modal synchronization requirements between PDU sets of LCHs of a multi-modal application, of data is considered when determining the order in which data of LCHs is multiplexed in a TB/UL grant.

Similar to enhancements of the LCP procedure, RAN2 should also discuss whether DSR reporting needs to be improved to provide gNB with Multi-modal related information allowing some efficient UL scheduling.

According to Rel-18 UE triggers a DSR and provides gNB with the remaining time information (smallest remaining time) and the amount of delay-critical data pending in the UE buffer, when the remaining time of a SDU becomes smaller than a configured threshold remainingTimeThreshold. It may be also beneficial if UE informs gNB about data for which the relative remaining delay, e.g. enforcing the multi-modal synchronization requirement, is small, becoming lower than a threshold. This would allow for an enhanced uplink scheduling of multi-modal XR services considering the relative delay/synchronization constraints.

Proposal 5: RAN2 to discuss DSR enhancements for multi-modal applications, e.g. UE providing information to gNB on data for which the relative remaining delay, e.g. enforcing the multi-modal synchronization requirement, becomes lower than a threshold.

### Discarding operation

According to the current specifications, the PDCP discard timer enforces the PSDB/PDB which is an absolute delay requirement of data carried within a LCH/RB. Upon arrival of new data from upper layer, PDCP entity starts a PDCP discard timer for each PDCP SDU. In response to the expiry of a PDCP discard timer, PDCP entity discards the associated PDCP SDU and the corresponding PDCP PDU. In case pdu-SetDiscard, UE discards all PDCP SDUs of the corresponding PDU Set along with the corresponding PDCP Data PDUs. 
RAN2 should further discuss how the relative delay requirement between PDUs/PDU sets of different LCHs belonging to one multi-modal service is enforced at the Layer 2/PDCP. We assume that UE should discard a PDCP SDU/PDU if the relative delay budget is exceeded, e.g. when data of a LCH belonging to a multi-modal application has not been delivered within the synchronization requirements. One option would be to introduce a second PDCP discardTimer which is configured according to the relative delay requirement/synchronization requirement. RAN should avoid the transmission of out-dated data, which is of no use for the user experience, e.g. discarded at the application layer. It should be first confirmed by SA2/SA4 whether UE should discard data of a LCH belonging to a multi-modal application for which the synchronization requirements cannot be fulfilled, e.g. relative delay budget exceeded. In general RAN2 should discuss how discarding of PDUs in different inter-dependent PDCP entities is implemented/specified.

Proposal 6: RAN2 to confirm with SA2/SA4 whether UE shall discard data of a LCH belonging to a multi-modal application for which the synchronization requirement cannot be fulfilled, e.g. relative delay budget exceeded.

Proposal 7: RAN2 to further discuss the discarding behaviour at PDCP layer for multi-modal applications, e.g. discarding of PDUs in different inter-dependent PDCP entities enforcing the relative delay budget/synchronization requirement by means of PDCP discard timer.

### DRX operation for multi-modal services

XR services, such as AR, VR, and Cloud Gaming services, over 5G are important vertical applications for 5G commercial deployments. Many of the end user XR and CG devices are expected to be mobile and of small-scale, thus having limited battery power resources. The DRX mechanism is a legacy power saving mechanism to achieve power saving gain for UE in RRC_CONNECTED state. However, some characteristics of XR traffic are not addressed by the legacy DRX mechanism and could result in limited UE power savings.

Current specifications allow the NW to configure only one DRX configuration for a serving cell and, therefore, this single DRX configuration cannot really match the traffic characteristics of all XR flows. Thus, it cannot closely follow the potential short sleep opportunities and result in either low power saving gains, or in an increased traffic delay. During Rel-18 there have been several proposals proposing to study solutions with multiple active DRX configurations each of them aligned with a different XR flow/stream. The motivation is that a single DRX configuration is not efficient from power saving perspective since traffic characteristics are too different and can’t be “tracked/covered” by a single DRX configuration. As shown above the different streams of a multi-modal application have different traffic characteristics such as periodicities/jitter.

With a single data flow for the UE, if the DRX periodicity matches the traffic arrival of that data flow, then dynamic grant can be used as the main means to support XR traffic. However, for multi-modal applications with the modalities having different periodicities, it can be difficult to determine a periodicity and Onduration which satisfies requirements of all these data flows thereby enabling a good power saving gain. When there are multiple data flows, especially when at least integer relationship does not exist among the periodicities of the different data flows, there might be the risk that packet arrival of some data flows arrival occurs in the DRX-Off duration, e.g. UE not monitoring PDCCH which would in turn lead to some additional delay.

Having multiple simultaneous DRX configurations could help schedule transmissions corresponding to different traffic characteristics in a timely manner; e.g., by providing on-durations in several occasions according to expected traffic arrivals. It has been proposed to have the UE monitor the PDCCH while the drx-onDurationTimer (or drx-InactivityTimer) is running in any of the DRX configurations, i.e. the overall active time is a logical ‘OR’ of the active times given by each DRX configuration.

We also see multiple active DRX configurations or the DCI-based switching between DRX configurations as potential enhancements for reducing PDCCH monitoring. DCI-based switching between DRX configuration refers to the case that NW dynamically activates/re-activates/deactivates DRX configurations, which allows for a dynamic adaptation of already activated DRX configurations or activation of additional DRX configuration.

Proposal 8: RAN2 to study the usage of multiple active DRX configuration and the DCI-based switching/activation of DRX configurations.

# Conclusion

In this contribution, we discuss potential RAN enhancements to efficiently support Multi-modal XR applications. We have the following proposals:

Proposal 1: The level of granularity of the inter-dependency information, e.g. PDU set level or QoS flow level, needs to be further discussed.

Proposal 2: Whether an extension of the UAI, to provide the assistance information relating to multi-modal synchronization among multiple uplink QoS flows to RAN, is needed/beneficial should be discussed after having the received the reply from SA2.

Proposal 3: Modalities of a MM application transmitted on distinct QoS flows and having different QoS requirements should be mapped to different DRBs.

Proposal 4: RAN2 to discuss enhancements to the LCP procedure, where the absolute remaining time and in addition the relative remaining time, e.g. enforcing the multi-modal synchronization requirements between PDU sets of LCHs of a multi-modal application, of data is considered when determining the order in which data of LCHs is multiplexed in a TB/UL grant.

Proposal 5: RAN2 to discuss DSR enhancements for multi-modal applications, e.g. UE providing information to gNB on data for which the relative remaining delay, e.g. enforcing the multi-modal synchronization requirement, becomes lower than a threshold.

Proposal 6: RAN2 to confirm with SA2/SA4 whether UE shall discard data of a LCH belonging to a multi-modal application for which the synchronization requirement cannot be fulfilled, e.g. relative delay budget exceeded.

Proposal 7: RAN2 to further discuss the discarding behaviour at PDCP layer for multi-modal applications, e.g. discarding of PDUs in different inter-dependent PDCP entities enforcing the relative delay budget/synchronization requirement by means of PDCP discard timer.

Proposal 8: RAN2 to study the usage of multiple active DRX configuration and the DCI-based switching/activation of DRX configurations.

# References

TS 38.321 v18.0.0