3GPP TSG-RAN WG2 Meeting #127	R2-2406598

# Maastricht, Netherlands, 19th– 23rd August 2024

Agenda Item:	7.5.3

Source:	Ericsson

Title:	Remaining Open Issues for PDCP SN Gap Reporting

Document for:	Discussion, Decision

# 1	Introduction

We address some of the remaining open issues in the PDCP SN gap reporting procedure, with suggested changes to the 38.323 specification.

# 2	Discussion

We address two issues relating to the current implementation of the specification in 38.323 for PDCP SN gap reporting as described below.

- Handling More-than-One RLC Entity

- Handling PDCP Rx State Variables

## 2.1 	Handling More-than-One RLC Entity

The current implementation of the transmit operation in the 38.323 spec is as follows:

### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - the PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by RLC to lower layers. If a PDCP SN gap report is triggered, the transmitting PDCP entity shall: - compile a PDCP SN gap report as indicated below by: - setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s); - if more than one PDCP SDUs are discarded: - allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first discarded PDCP SDU up to and including the last discarded PDCP SDU, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first; - setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded; - setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded; - submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface. ### NOTE: It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

The highlighted part shows that a PDCP SN gap report is triggered if the discarded PDCP SDU(s) have not been submitted by RLC to the lower layers. However, it does not consider the case when there is more than one RLC entity i.e., DC/CA. So, we propose to cover the case of DC and CA as captured in the TP in the Annex.

Update the triggering conditions text to include DC and CA by adding “RLC entities” in the PDCP SN gap reporting procedure. Adopt the TP as shown in the Annex.

## 2.2 	Handling PDCP Rx State Variables

The following was discussed in the last meeting on the issue as raised in [1]:

Proposal 1: RX_NEXT is incremented (upon reception of a SN gap report and upon being updated by a received SDU) only if, without the incrementing, the SDU with associated COUNT equal to RX_NEXT would be considered as discarded (i.e. indicated as discarded in some received SN gap report). - Samsung shares the concerns. LG doesn’t think there is a problem as the reporting is done later than stored SDUs. Nokia thinks that it assumes no pre-processing and the scenario can occur so we can even prevent the scenario to occur or we have a fix. - Ericsson has some sympathy for the proposal, the prioritization in the DL is possible but there may be difference in solution. Intel also acknowledges the existence of this problem. - Vivo thinks that this is a big impact to UE and it can be handle by network implementation. Nokia thinks that we have to compare it to the TP we have, and we are already changing a behavior. - Huawei thinks that this can be solved by NW implementation by setting T-ordering time properly. Ericsson doesn’t think we can solve it with configuration. => Noted

Figure 1: Current PDCP SN Gap Reporting Procedure

As seen from the Chair’s notes, many companies acknowledge the issue that upon reception of the PDCP SN gap report containing a non-contiguous discarded SDU(s), RX_NEXT, RX_REORD are updated to the highest value of the SN yet to be received. Then, due to transmission latencies, it is possible that t-Reordering expires and RX_DELIV (i.e., lower edge of the PDCP Rx window) is updated to RX_REORD creating an unnecessary ‘void’ (refer to Figure 1). Larger the difference between the discarded PDCP SNs, larger the void.

As shown in Figure 1, a PDCP SN gap report is received indicating SN3 and SN11 have been discarded and RX_NEXT and RX_REORD are moved to SN12. Then, if t-Reordering expires for SN2, then RX_DELIV also moves to SN12, thereby creating the ‘void’. Due to this ‘void’, any SDU received whose SN is less than RX_DELIV is rendered redundant i.e., corresponding SDUs with RCVD_COUNT < RX_DELIV are discarded, although they should not be discarded. Without the PDCP SN gap report, the RX_NEXT, RX_REORD are pending on the next highest SN yet to be received and hence, such a ‘void’ is not created.

- The current implementation of the PDCP SN gap report could create unnecessary voids especially in the case when the PDCP SN gap report consist of non-contiguously discarded PDCP SDU(s).

As pointed out in [1], this is worsened when there are high and low importance packets in the buffer whereby, when a set of low importance packets are discarded and t-Reordering expires, all interleaved high importance packets less than RX_REORD are also rendered discarded. The solution as proposed in [1] is to change the way RX_NEXT is updated considering the discarded SDUs, thereby not creating the ‘void’ as explained above. However, the approach as suggested has larger specification impact in that it changes the PDCP Rx state handling mechanism. Such changes in late releases (of NR) are not desirable and hence, should not be pursued.

- Suggestion to change the state variable handling has a large impact on specification especially in such late stages of NR.

Our preference would be to have a solution with least specification impact, but also address the ‘void’. One way to address this problem would be to give sufficient time for the PDCP Tx entity to perform subsequent transmissions without the PDCP Rx state variables updating prematurely i.e., RX_DELIV is updated beyond the PDCP SDU(s) yet to be received.

- Prefer to have the least specification impact by providing sufficient time for the Tx entity to perform transmissions without the PDCP RX state variables updating prematurely.

Figure 2: Proposed Splitting of the PDCP SN Gap Report

This can be achieved by simply not combining the PDCP SN gap reports when non-contiguous PDCP SDUs are discarded. This prevents RX_DELIV from being updated beyond the PDCP SDU(s) yet to be received and would allow the Tx entity a time of t-Reordering to perform subsequent transmissions.

As illustrated in Figure 2, for the same scenario where SN3 and SN11 are discarded. The first PDCP SN gap report only consists of the discarded SN3. Upon receiving, this report, RX_NEXT and RX_REORD are pushed to SN4 and RX_DELIV is pending on SN2. If the t-Reordering expires, then, RX_DELIV is updated to SN4 and this would give the TX entity more time (equal to t-Reordering) to perform subsequent transmissions, thereby mitigating the issue.

This approach has the advantage that the current PDCP Rx state variable handling is maintained. It is also very unlikely that transmission delays are beyond 2*t-Reordering. Subsequently, a second PDCP SN gap report can then consists of the rest of the discarded SNs i.e., SN11 in the example above.

- Not combining the PDCP SN gap reports for non-contiguously discarded PDCP SDU(s) mitigates the void issue with the advantage that the current PDCP Rx state variable handling mechanism is maintained.

The current version of the specification allows for unconditional combining of the PDCP SN gap reports as this is linked to the frequency of reporting. It was also agreed that it is up to UE implementation how to limit the frequency of the reporting, as captured in the NOTE shown below:

NOTE:	It is up to UE implementation how to limit the frequency of PDCP SN gap reporting

On the one hand, the reports must be combined to limit the frequency of reporting and reduce overhead. On the other hand, reporting non-contiguous discarded SNs could result in the ‘void’ (as explained above).

- Reports could be combined to limit the frequency of reporting, reduce overhead. But unconditional combining could lead to the void at the PDCP Rx entity.

Hence, a straightforward solution is to conditionally allow combining of the PDCP SN gap reports only for contiguous discarded PDCP SDU(s) and disallow combining for non-contiguously discarded PDCP SDU(s). The details can be left to UE implementation.

Update the NOTE about limiting the frequency of reporting to include “for e.g., combining of the PDCP SN gap report(s) for contiguous PDCP SDU(s) discarded”. Adopt the TP as shown in the Annex.

Capture a NOTE to prevent the RX_DELIV to be updated beyond the PDCP SDU(s) yet to be received when t-Reordering expires for e.g., by not combining the PDCP SN Gap reports for non-contiguous PDCP SDU(s) discarded. Adopt the TP as in the Annex.

# 3 	Conclusion

In the previous sections we made the following observations:

Observation 1	The current implementation of the PDCP SN gap report could create unnecessary voids especially in the case when the PDCP SN gap report consist of non-contiguously discarded PDCP SDU(s).

Observation 2	Suggestion to change the state variable handling has a large impact on specification especially in such late stages of NR.

Observation 3	Prefer to have the least specification impact by providing sufficient time for the Tx entity to perform transmissions without the PDCP RX state variables updating prematurely.

Observation 4	Not combining the PDCP SN gap reports for non-contiguously discarded PDCP SDU(s) mitigates the void issue with the advantage that the current PDCP Rx state variable handling mechanism is maintained.

Observation 5	Reports could be combined to limit the frequency of reporting, reduce overhead but unconditional combining could lead to the void at the PDCP Rx entity.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Update the triggering conditions text to include DC and CA by adding “RLC entities” in the PDCP SN gap reporting procedure. Adopt the TP as shown in the Annex.

Proposal 2	Update the NOTE about limiting the frequency of reporting to include “for e.g., combining of the PDCP SN gap report(s) for contiguous PDCP SDU(s) discarded”. Adopt the TP as shown in the Annex.

Proposal 3	Capture a NOTE to prevent the RX_DELIV to be updated beyond the PDCP SDU(s) yet to be received when t-Reordering expires for e.g., by not combining the PDCP SN Gap reports for non-contiguous PDCP SDU(s) discarded. Adopt the TP as in the Annex.

# 4 	References

- R2-2404287, PDCP SN gap reporting and updating RX_NEXT, RAN2#126, Fukuoka, Japan.

# 5 	TP – 38.323

START OF CHANGE

### 5.16.1	Transmit operation

For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when:

-	the PDCP SDU(s) are discarded as specified in clause 5.3; and

-	there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and

-	the discarded PDCP SDU(s) have not been submitted by RLC entities to lower layers.

If a PDCP SN gap report is triggered, the transmitting PDCP entity shall:

-	compile a PDCP SN gap report as indicated below by:

-	setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s);

-	if more than one PDCP SDUs are discarded:

-	allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first discarded PDCP SDU up to and including the last discarded PDCP SDU, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first;

-	setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded;

-	setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded;

-	submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface.

NOTE:	It is up to UE implementation how to limit the frequency of PDCP SN gap reporting for e.g., by combining the PDCP SN gap report(s) when contiguous PDCP SDU(s) are discarded.

NOTE X: 	It is up to UE implementation to avoid RX_DELIV to be updated beyond the PDCP SDU(s) yet to be received, when t-Reordering expires for e.g., by not combining the PDCP SN gap report(s) when non-contiguous PDCP SDU(s) are discarded.

END OF CHANGE