3GPP TSG-RAN WG2 #127	R2-2406600

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	7.9.5

Source:	Ericsson

Title:	Remaining Open Issues in 38.323

Document for:	Discussion, Decision

# 1 	Introduction

We address the remaining open issues as described in the next section based on our review of the latest version of the 38.323 specification.

# 2 	Discussion

### 2.1 Data Transfer – Transmit Operation

The data transfer – transmit operation in the current version of the 38.323 specification is implemented as follows:

- else, if the transmitting PDCP entity is associated with one or more RLC entities and, either one SRAP entity or the N3C: - if PDCP duplication is activated for the RB: - if the PDCP PDU is a PDCP Data PDU: - duplicate the PDCP Data PDU and submit the PDCP Data PDU to each of the MP primary path and MP secondary path which is activated for PDCP duplication, including any associated Uu RLC entities activated for PDCP duplication; - else: - if the MP primary path is the direct path: - submit the PDCP Control PDU to the primary RLC entity; - else: - submit the PDCP Control PDU to the SRAP entity or N3C; - else (i.e., PDCP duplication is deactivated for the RB): - if the total amount of PDCP data volume, RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the primary RLC entity, and data volume pending for initial transmission in the N3C (if available) or mapped SL RLC entity associated with the SRAP entity, is equal to or larger than ul-DataSplitThreshold: - submit the PDCP PDU to either the primary RLC entity or SRAP entity/N3C; - else: - if the MP primary path is the direct path: - submit the PDCP PDU to the primary RLC entity; - else: - submit the PDCP PDU to the SRAP entity or N3C;

The highlighted text in yellow seems to suggest that there is a scenario where the SRAP entity and the N3C entity can co-exist i.e., co-existence between Scenario-1 and Scenario-2. However, at no point during Rel-18 discussions was such a co-existence discussed, hence for clarity, we suggest update the text my removing the ‘/’ punctuation mark. A similar situation in the else part but is captured correctly as it mentions that “submit the PDCP PDU to the SRAP entity or N3C”.

Remove the ‘/’ punctuation mark between SRAP entity and N3C. Adopt the TP as in the Annex.

### 2.2 Duplicate PDU discard

- The current implementation of the duplicate PDU discard is as follows:

For the PDCP entity configured with pdcp-Duplication or for the PDCP entity associated with two RLC entities for an SLRB, the transmitting PDCP entity shall: - if the successful delivery of a PDCP Data PDU is confirmed by one of the associated AM RLC entities and the AM RLC entity is not associated with an SRAP entity: - indicate to the other AM RLC entities to discard the duplicated PDCP Data PDU; - if the deactivation of PDCP duplication is indicated for the DRB: - if the transmitting PDCP entity is associated with one or more RLC entities and, either one SRAP entity or the N3C; and - if the MP primary path is on the indirect path: - indicate to the RLC entities on the MP secondary path to discard all duplicated PDCP Data PDUs; - else: - indicate to the RLC entities other than the primary RLC entity to discard all duplicated PDCP Data PDUs; - if the deactivation of PDCP duplication is indicated for at least one associated RLC entities: - indicate to the RLC entities deactivated for PDCP duplication to discard all duplicated PDCP Data PDUs.

- In the highlighted yellow text above, “indicate to the RLC entities” is captured to cover the duplicate PDCP data PDU discard case. However, it is possible that there are configurations where not all the RLC entities are deactivated for PDCP duplication for e.g., when the UE is configured with both CA on Uu and MP over direct and indirect path. Hence, the text should be updated as in legacy to include ‘deactivated for PDCP duplication’.

Include the text ‘RLC entities deactivated for PDCP duplication’ in the duplicate PDU discard section when PDCP duplication is deactivated.

# 3 	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Remove the ‘/’ punctuation mark between SRAP entity and N3C. Adopt the TP as in the Annex.

Proposal 2	Include the text ‘RLC entities deactivated for PDCP duplication’ in the duplicate PDU discard section when PDCP duplication is deactivated.

# 4 	References

# 5 	TP – 38.323

START OF CHANGES

<unchanged parts omitted>

-	else, if the transmitting PDCP entity is associated with one or more RLC entities and, either one SRAP entity or the N3C:

-	if PDCP duplication is activated for the RB:

-	if the PDCP PDU is a PDCP Data PDU:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to each of the MP primary path and MP secondary path which is activated for PDCP duplication, including any associated Uu RLC entities activated for PDCP duplication;

-	else:

-	if the MP primary path is the direct path:

-	submit the PDCP Control PDU to the primary RLC entity;

-	else:

-	submit the PDCP Control PDU to the SRAP entity or N3C;

-	else (i.e., PDCP duplication is deactivated for the RB):

-	if the total amount of PDCP data volume, RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the primary RLC entity, and data volume pending for initial transmission in the N3C (if available) or mapped SL RLC entity associated with the SRAP entity, is equal to or larger than ul-DataSplitThreshold:

-	submit the PDCP PDU to either the primary RLC entity or SRAP entity or N3C;

-	else:

-	if the MP primary path is the direct path:

-	submit the PDCP PDU to the primary RLC entity;

-	else:

-	submit the PDCP PDU to the SRAP entity or N3C;

<unchanged parts omitted>

NEXT CHANGE

<unchanged parts omitted>

### 5.11.2	Duplicate PDU discard

For the PDCP entity configured with pdcp-Duplication or for the PDCP entity associated with two RLC entities for an SLRB, the transmitting PDCP entity shall:

-	if the successful delivery of a PDCP Data PDU is confirmed by one of the associated AM RLC entities and the AM RLC entity is not associated with an SRAP entity:

-	indicate to the other AM RLC entities to discard the duplicated PDCP Data PDU;

-	if the deactivation of PDCP duplication is indicated for the DRB:

-	if the transmitting PDCP entity is associated with one or more RLC entities and, either one SRAP entity or the N3C; and

-	if the MP primary path is on the indirect path:

-	indicate to the RLC entities deactivated for PDCP duplication on the MP secondary path to discard all duplicated PDCP Data PDUs;

-	else:

-	indicate to the RLC entities deactivated for PDCP duplication other than the primary RLC entity to discard all duplicated PDCP Data PDUs;

-	if the deactivation of PDCP duplication is indicated for at least one associated RLC entities:

-	indicate to the RLC entities deactivated for PDCP duplication to discard all duplicated PDCP Data PDUs.

<unchanged parts omitted>

END OF CHANGES