3GPP TSG-RAN WG2 Meeting #127	R2-2406601

# Maastricht, Netherlands, 20th – 24th May 2024

Agenda Item:	8.7.5

Source:	Ericsson

Title:	Further Discussions on RLC AM Enhancements

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#126, the following agreements were made on the RLC AM enhancements:

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives:

- Rx initiated approach

- Tx initiated approach

RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer.

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options:

  - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered)

  - Retransmission based on enhanced status report

  - Retransmission based on enhanced polling

- FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism

Impact on capacity should be considered

RAN2 focuses on the enhancements for UL traffic

In this contribution, we discuss the two tracks of enhancements and provide our views.

# 2	Discussion

## 2.1 	Avoiding Unnecessary Retransmissions

As discussed in the last meeting there are two approaches namely the Tx-initiated and Rx-initiated to solve the issue of avoiding unnecessary RLC AM retransmissions. We analyse both options below.

### Rx-Initiated Approach

The following was the agreement from the last meeting on the Rx-initiated approach:

For Rx initiated approach:

  - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter.

  - Tx side just processes the status report as in legacy

  - FFS how Rx side determines that an SDU should be abandoned

We first address the FFS on how the Rx side determines that an SDU is abandoned. In principle, this is related to whether the corresponding PDU is still relevant to the PDCP layer. The relevance is based on if the t-Reordering timer for this PDCP PDU has not expired. However, if t-Reordering expires, the lower edge of the PDCP Rx window is advanced and even if it was successfully delivered by the RLC layer, the corresponding SDU is discarded.

- Rx side can determine SDU abandonment based on whether the corresponding PDCP PDU is still relevant i.e., the t-Reordering timer is still running for this PDCP PDU.

However, as PDCP and RLC layers run independently, the RLC layer is not aware of t-Reordering timer status. A simple mechanism to make the RLC layer aware of the SDUs relevance is to introduce a new timer. When running, the RLC layer is aware that this SDU is still relevant to the PDCP layer but upon expiry, would recognize that the corresponding PDCP PDU is irrelevant and hence, the SDU can be abandoned.

- New timer can be introduced to make the RLC layer aware of the SDUs relevance at the PDCP layer.

With the new timer, the RLC layer is now able to determine that SDU abandonment status, the legacy RLC status reporting mechanism can be reused to send a positive acknowledgement for PDUs when the new timer has expired although they were not successfully received at the RLC receiver. This would enable the Rx entity to advance the receive window and allow for the Tx entity to perform more relevant retransmissions.

- Reuse the legacy RLC status reporting mechanism to send a positive acknowledgement for PDUs when the new timer expires.

The new timer need not be perfectly synchronized with t-Reordering, but it should allow for a certain number of retransmissions before abandonment thereby limiting the number of ARQ retransmissions to deliver the missing RLC data. As a result, the length of the new timer can have values in the range of t-Reordering.

- The Length of the new timer can be of the order of t-Reordering and need not be synchronized. The point is to allow for a finite number of retransmissions before abandonment.

The new timer can be started when a higher RLC SN than RX_Next has been delivered to the upper layers (i.e. when a gap gets created), then RLC knows that the PDCP will only wait a finite time (i.e., t-Reordering) for the missing PDCP SN before advancing the PDCP Rx window.

- New timer can be started when a higher RLC SN than RX_Next has been delivered to the upper layers. In this case, the RLC knows that the PDCP will only wait a finite time (t-Reordering) before advancing the PDCP Rx window.

The Rx-initiated mechanism using the new timer is an easy and straightforward method to identify the status of abandonment at the Rx entity with no cross-layer interaction between RLC and PDCP. In addition, most of the legacy procedures can also be reused.

- Rx-initiated mechanism is an easy and straightforward method to identify the SDU abandonment status with no cross-layer interaction between RLC and PDCP which includes reusing most of the legacy procedures.

It should also be noted that RLF mechanism is the same i.e., triggered by the Tx RLC entity when the maxReTxThreshold is reached.

- RLF detection mechanism is the same as legacy in the Rx-initiated mechanism i.e., declared by the Tx RLC entity when maxReTxThreshold is reached.

### Tx-Initiated Approach

For Tx initiated approach:

  - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs

  - Tx side stops retransmit obsolete SDUs

  - Rx side updates state variables according to the information from Tx side

The Tx entity is aware of the data buffered and the remaining PDB for any of the PDUs. Therefore, the transmitter can assess how many retransmissions can be performed for a certain RLC PDU. This also depends on the PSDB and the buffered time for each of the PDU Sets.

- The RLC transmitter is aware of the buffered data and their remaining PDB thereby can assess the number of retransmissions possible for a certain RLC PDU.

As a result, in the transmitter-based mechanism, the RLC transmitter could indicate the RLC SNs for which the RLC receiver need not wait and consider them to be discarded. Based on the indication, the RLC Rx entity can move the lower edge of the receiving window as these RLC SDU(s) and its segments would be considered as discarded.

- RLC transmitter can indicate that certain RLC SDU(s) is/are out of time bounds, thereby enabling the receiver to advance the lower edge of the receiving window.

In NR, the PDCP and RLC entities run independently without any cross-layer interaction i.e., PDCP and RLC SNs are assigned independently with no knowledge between them of the assigned SNs. In the Tx-initiated approach, it is the Tx PDCP entity which determines whether a particular SN is outdated and then informs the Rx RLC entity. Technically, this would require the Tx RLC entity to maintain a mapping between the PDCP and RLC SNs or peek into the PDCP header, which in either case results in much more cross-layer interaction and a departure from the current procedures in NR. In addition, such cross-layer interaction in a CU-DU split scenario would also affect RAN3 specifications and complicate the procedure.

- Tx-Initiated approach results in much more cross layer interaction than the current NR procedure and is complicated especially in the DL with CU-DU split scenarios.

Such a Tx-initiated approach was specified in UMTS for discarding packets deemed as outdated. However, the downside of this approach was it required the Tx entity to wait for a confirmation from Rx entity before it can advance its Tx window. This sometimes led to the Tx/Rx RLC entities becoming out of sync and then needing to initiate the reset procedure. This issue would also be applicable to NR resulting in RRC reestablishment.

- Tx entity needs to wait for confirmation from Rx entity before it could advance the Tx window leading to situations where the RLC Tx/Rx entities going out of sync thereby resulting in RRC reestablishment.

Tx-initiated approach also requires modifications to the RLF mechanism because the SDUs can be discarded before the maxReTxThreshold is reached and as a result, RLF is never declared even in situations where the UL is poor meaning that the UE may be stuck in a situation where a failed connection is never abandoned or abandoned too late.

- With the Tx-initiated approach, the SDUs can be discarded before the maxReTxThreshold is reached and hence, RLF is never declared.

### Comparison

The following table evaluates the two approaches based on different criteria of the two mechanisms as explained above:

|  | Rx-Initiated Approach | Tx-Initiated Approach |
| --- | --- | --- |
| Tx Procedures | Legacy procedures can be fully reused | New procedure is to be introduced to notify the RLC Rx entity and stop retransmission of obselete SDUs |
| Rx Procedures | A new timer is needed to identify that PDUs are irrelevant to the PDCP layer | RLC Rx state variables handling procedure needs to be updated. |
| Signaling Procedures | Existing RLC status PDU can be reused | New signaling has to be introduced in addition to the RLC status PDU |
| RLF Mechanism | Existing procedures can be reused | New procedure is to be introduced |
| Cross-Layer Interaction | Little (from a configuration standpoint) to None | More cross-layer interaction is needed for this solution to work |
| Specification Impact | Simple! See TP in Annex. Small impact as most legacy procedures are reused with the advantage of needing to only specify this in the DL | Large impact as many new procedures are to be introduced. Has impact on both UL and DL. |

As detailed in the table, the Rx-initiated approach has the advantage of reusing most of the legacy procedures with only the addition of introducing a new timer to identify SDU abandonment. Hence, we propose the following:

Pursue only the Rx-initiated approach to avoid unnecessary RLC AM retransmissions.

Introduce a new timer at the RLC Rx entity.

The new timer can be started when a higher RLC SN than RX_Next has been delivered to the upper layers.

The new timer can be stopped/reset when RX_Next_Highest is equal to RX_Next.

Upon expiry of the new timer, indicate in the RLC status report a positive acknowledgement for the corresponding PDU.

## 2.2 	Timely Retransmissions

In the other topic on the objective on RLC AM enhancements, to achieve timely retransmissions on RLC layer for XR traffic, RAN2 decided to consider the following options:

- Autonomous retransmissions

- Retransmission based on enhanced status report.

- Retransmission based on enhanced polling.

With a related FFS on whether such enhancements are needed, or this can be solved with proper configuration and current mechanism. To this end, we investigate the current mechanism for triggering such RLC retransmissions with the aid of a timing analysis using existing configurations for the ARQ procedure.

Figure-1: Timing Diagram

Figure-1 illustrates the timing diagram for the existing mechanism of RLC AM PDU (AMD PDU) transmission, ARQ including polling. In the figure, Z ms represents the discard timer for the AMD PDU i.e., the outer time limit for relevant AMD PDU retransmissions. Y ms (including gNB processing time) represents the time to complete the HARQ procedure for an AMD PDU and X ms represents the time to set the polling bit.

Assuming a 30 kHz subcarrier spacing i.e., TTI is 0.5 ms, with pollPDU = 4 (lowest configurable value), the polling bit is set every 2 ms i.e., X = 2 ms. Assuming the gNB processing time is 1 ms, we have two cases:

- No HARQ: the remaining time until the AMD PDU is discarded is (Z – 3) ms.

- With HARQ: Y = (around) 6 ms, leaving a remaining time of (Z – 6) ms until the AMD PDU is discarded.

For longer discard timers for e.g., 50 ms, the remaining times are 44 ms (with HARQ) and 47 ms (no HARQ). Hence, there is sufficient time for the RLC Rx entity to send the RLC status report and perform RLC retransmissions under network control.

- For longer discard timers, the remaining time until discard is sufficient to obtain the RLC status report from the RLC Rx entity to perform RLC retransmissions under network control.

In addition to the pollPDU, the pollByte can also be configured to enable more frequent reporting of the RLC status report. Reusing the example above, the polling bit can be set every 0.5 ms hence, the remaining times are (Z – 1.5) ms with no HARQ and the same (Z – 6) ms with HARQ. Similarly, at least for longer discard timers, we see that the remaining time is sufficient.

- Existing values of pollPDU and pollByte satisfy the timeline requirements to perform RLC retransmissions.

For shorter discard timers for e.g., 10 ms, the remaining times are 4 ms (with HARQ) and 7 ms (no HARQ). Although, one could argue that although the RLC status PDUs (RLC control PDUs) are prioritized, there is not enough time to trigger RLC retransmissions i.e., RLC AM is not suited for such short discard timers. However, for such short discard timers, the gNB can continue to perform blind HARQ retransmissions. HARQ retransmissions are better when compared to RLC retransmissions because of the 3 dB gain from the soft combining across different transmissions.

- For shorter discard timers, the Tx entity (under gNB control) can continue to perform blind HARQ retransmissions (rather than triggering RLC retransmissions) thereby getting the 3 dB gain from soft combining.

For autonomous retransmissions, with a BLER target usually set to 10%, 90% of these transmissions are redundant and thus leads to wastage of resources. In addition, from a system perspective, UEs in bad radio channel conditions would essentially consume 2x UL resources per UE, per RLC SDU retransmission, where x depends on segmentation. This would be detrimental in an already resource constrained environment exacerbating UL congestion in the cell. Further, there is no guarantee that such autonomous retransmissions will be successful.

- Autonomous retransmissions will be redundant 90% of the time and would consume 2x UL resources per UE, per RLC SDU retransmission. For example, if segmented into 4 AMD PDUs, a UE would consume eight times the UL resources which would exacerbate UL congestion.

Although the autonomous retransmissions could be based on triggers and assuming it is the remaining time to discard, it would be very difficult to set a correct time threshold. It is a trade-off between UL capacity impairment and remaining time. In addition, it would make sense to allow for HARQ retransmissions to complete before triggering an autonomous retransmission as explained above.

- Very difficult to set a correct trigger as it is a trade-off between UL capacity impairment and remaining time.

The retransmissions based on enhanced status reporting was also discussed in the last meeting, however, it is unclear what enhancements are necessary to the status reporting mechanism as such. The current format to trigger the status report based on polling works as expected and as explained above, the timelines are also favourable enough to support multiple retransmissions.

- Enhancements to the RLC status report are unclear given that the current polling mechanism works and meets the timelines.

One could optimize the current polling mechanism based on the argument that this mechanism might result in the RLC status reports being sent either too frequently (too early) or sparsely (too late). Such frequent status reports are not always necessary because not all the AMD PDUs are delay critical at a given time and hence, it leads to overheads in transmission. On the other hand, it cannot be too sparse (too late) because this would result in the AMD PDU to be delay critical or even discarded if received too late i.e., current polling mechanism might not always be synchronized with the delay requirements of the AMD PDU as the Rx entity is unaware of the remaining time to discard.

- The current polling mechanism cannot consider the delay criticality of the AMD PDU in the buffer. It is a trade-off between transmission overhead and timely reception of the RLC status PDU.

As a result, if any enhancements are to be pursued to enable timely retransmission for XR traffic, it should be enhancements to the polling mechanism. Such enhancements to the polling mechanism could consider the delay criticality of the AMD PDU in the buffer. The advantages are that the polling is set only when required and it helps the Rx entity to send the RLC status report in a timely manner.

Pursue enhancements to the polling mechanism where the delay criticality of the AMD PDU is considered.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

Observation 1	Rx side can determine SDU abandonment based on whether the corresponding PDCP PDU is still relevant i.e., the t-Reordering timer is still running for this PDCP PDU.

Observation 2	New timer can be introduced to make the RLC layer aware of the SDUs relevance at the PDCP layer.

Observation 3	Reuse the legacy RLC status reporting mechanism to send a positive acknowledgement for PDUs when the new timer expires.

Observation 4	The Length of the new timer can be of the order of t-Reordering and need not be synchronized. The point is to allow for a finite number of retransmissions before abandonment.

Observation 5	New timer can be started when a higher RLC SN than RX_Next has been delivered to the upper layers. In this case, the RLC knows that the PDCP will only wait a finite time (t-Reordering) before advancing the PDCP Rx window.

Observation 6	Rx-initiated mechanism is an easy and straightforward method to identify the SDU abandonment status with no cross-layer interaction between RLC and PDCP which includes reusing most of the legacy procedures.

Observation 7	RLF detection mechanism is the same as legacy in the Rx-initiated mechanism i.e., declared by the Tx RLC entity when maxReTxThreshold is reached.

Observation 8	The RLC transmitter is aware of the buffered data and their remaining PDB thereby can assess the number of retransmissions possible for a certain RLC PDU.

Observation 9	RLC transmitter can indicate that certain RLC SDU(s) is/are out of time bounds, thereby enabling the receiver to advance the lower edge of the receiving window.

Observation 10	Tx-Initiated approach results in much more cross layer interaction than the current NR procedure and is complicated especially in the DL with CU-DU split scenarios.

Observation 11	Tx entity needs to wait for confirmation from Rx entity before it could advance the Tx window leading to situations where the RLC Tx/Rx entities going out of sync thereby resulting in RRC reestablishment.

Observation 12	With the Tx-initiated approach, the SDUs can be discarded before the maxReTxThreshold is reached and hence, RLF is never declared.

Observation 13	For longer discard timers, the remaining time until discard is sufficient to obtain the RLC status report from the RLC Rx entity to perform RLC retransmissions under network control.

Observation 14	Existing values of pollPDU and pollByte satisfy the timeline requirements to perform RLC retransmissions.

Observation 15	For shorter discard timers, the Tx entity (under gNB control) can continue to perform blind HARQ retransmissions (rather than triggering RLC retransmissions) thereby getting the 3 dB gain from soft combining.

Observation 16	Autonomous retransmissions will be redundant 90% of the time and would consume 2x UL resources per UE, per RLC SDU retransmission. For example, if segmented into 4 AMD PDUs, a UE would consume eight times the UL resources which would exacerbate UL congestion.

Observation 17	Very difficult to set a correct trigger as it is a trade-off between UL capacity impairment and remaining time.

Observation 18	Enhancements to the RLC status report are unclear given that the current polling mechanism works and meets the timelines.

Observation 19	The current polling mechanism cannot consider the delay criticality of the AMD PDU in the buffer. It is a trade-off between transmission overhead and timely reception of the RLC status PDU.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Pursue only the Rx-initiated approach to avoid unnecessary RLC AM retransmissions.

Proposal 2	Introduce a new timer at the RLC Rx entity.

Proposal 3	The new timer can be started when a higher RLC SN than RX_Next has been delivered to the upper layers.

Proposal 4	The new timer can be stopped/reset when RX_Next_Highest is equal to RX_Next.

Proposal 5	Upon expiry of the new timer, indicate in the RLC status report a positive acknowledgement for the corresponding PDU.

Proposal 6	Pursue enhancements to the polling mechanism where the delay criticality of the AMD PDU is considered.

# 4 	References

# TP – 38.323

START OF CHANGES

#### 5.2.3.2	Receive operations

##### 5.2.3.2.1	General

The receiving side of an AM RLC entity shall maintain a receiving window according to the state variable RX_Next as follows:

-	a SN falls within the receiving window if RX_Next <= SN < RX_Next + AM_Window_Size;

-	a SN falls outside of the receiving window otherwise.

When receiving an AMD PDU from lower layer, the receiving side of an AM RLC entity shall:

-	either discard the received AMD PDU or place it in the reception buffer (see clause 5.2.3.2.2);

-	if the received AMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see clause 5.2.3.2.3).

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update state variables and start t-Reassembly as needed (see clause 5.2.3.2.4).

##### 5.2.3.2.2	Actions when an AMD PDU is received from lower layer

When an AMD PDU is received from lower layer, where the AMD PDU contains byte segment numbers y to z of an RLC SDU with SN = x, the receiving side of an AM RLC entity shall:

-	if x falls outside of the receiving window; or

-	if byte segment numbers y to z of the RLC SDU with SN = x have been received before:

-	discard the received AMD PDU.

-	else:

-	place the received AMD PDU in the reception buffer;

-	if some byte segments of the RLC SDU contained in the AMD PDU have been received before:

-	discard the duplicate byte segments.

##### 5.2.3.2.3	Actions when an AMD PDU is placed in the reception buffer

When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

-	if x >= RX_Next_Highest:

-	update RX_Next_Highest to x+ 1.

-	if all bytes of the RLC SDU with SN = x are received:

-	reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Highest_Status:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received.

-	if x = RX_Next:

-	update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received.

-	if t-Reassembly is running:

-	if RX_Next_Status_Trigger = RX_Next; or

-	if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or

-	if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size:

-	stop and reset t-Reassembly.

-	if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above):

-	if RX_Next_Highest> RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

-	if t-WinAdvance is running:

-   if RX_Next_Highest = RX_Next; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	stop and reset t-WinAdvance;

-	if t-WinAdvance is not running (includes the case t-WinAdvance is stopped due to actions above):

-   if RX_Next_Highest > RX_Next + 1 and at least one RLC SDU has been delivered to upper layer with SN > RX_NEXT; or

-	start t-WinAdvance;

##### 5.2.3.2.4	Actions when t-Reassembly expires

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Status_Trigger for which not all bytes have been received;

-	if RX_Next_Highest> RX_Highest_Status +1: or

-	if RX_Next_Highest = RX_Highest_Status + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Highest_Status before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

NEXT CHANGE

##### 5.2.3.2.4	Actions when t-WinAdvance expires

When t-WinAdvance expires, the receiving side of an AM RLC entity shall:

-	update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received:

-	start t-WinAdvance;

NEXT CHANGE

### 5.3.4	Status reporting

An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them).

Triggers to initiate STATUS reporting include:

-	Polling from its peer AM RLC entity:

-	When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall:

-	if the AMD PDU is to be discarded as specified in clause 5.2.3.2.2; or

-	if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size:

-	trigger a STATUS report.

-	else:

-	delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size.

NOTE 1:	This ensures that the RLC Status report is transmitted after HARQ reordering.

-	Detection of reception failure of an AMD PDU

-	The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires.

NOTE 2:	The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated.

When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall:

-	if t-StatusProhibit is not running:

-	at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer.

-	else:

-	at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer.

When a STATUS PDU has been submitted to lower layer, the receiving side of an AM RLC entity shall:

-	start t-StatusProhibit.

When constructing a STATUS PDU, the AM RLC entity shall:

-	for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

-	for an RLC SDU for which no byte segments have been received yet:

-	include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU.

-	for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN, SOstart and SOend.

-	for a continuous sequence of RLC SDUs that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN and NACK range;

-	include in the STATUS PDU, if required, a pair of SOstart and SOend.

-	set the ACK_SN to the SN of the next not received RLC SDU or the skipped SN due to t-WinAdvance expiry which is not indicated as missing in the resulting STATUS PDU.

END OF CHANGES