3GPP TSG-RAN2 Meeting #127	R2-2406602

Maastricht, Netherlands, 19th – 23rd August 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on the SFN issues of Cell DTX/DRX

Agenda Item:	7.3.2

Document for:	Discussion and decision

# Introduction

During the last RAN2#126 meeting, the unaligned SFN issue on cell DTX/DRX was discussed and no consensus was achieved [1]:

Clarifications R2-2404945 Discussion on the unaligned SFN for Cell DTXDRX in MAC CATT discussion Rel-18 Netw_Energy_NR-Core Proposal: In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the Cell DTX/DRX duration. - LG doesn’t think we need that change as each serving cell would look for it’s own time, as DTX is executed per serving cell. Apple doesn’t think this is even in scope. - Nokia wonders if the UE knows the SFN of the SCells. LG thinks that the UE knows the SFN of the serving cell. Nokia explains that the SFN is in the MIB. LG and Lenovo explains that the UE has to read the MIB for synchornization purposes. Samsung thinks that there is no requirement to read the SFN in the MIB - Xiaomi aks if we can do CELL DTX/DRX with SSBless => Noted

In this contribution, we will discuss the unaligned SFN issue.

# Discussion

In the RAN2#123bis meeting, we have agreed that a maximum of two cell DTX/DRX patterns can be configured per MAC entity [2]

Agreements@RAN2#123bis - Cell DTX/DRX configuration is provided per Serving Cell with the following restrictions: - A maximum of two cell DTX/DRX patterns can be configured per MAC entity - The two configured patterns are aligned, - The start and slot offset are common for the two patterns. - one periodicity is an integer multiple of the other.

The intention of this restriction is to align the cell DTX/DRX patterns between the two configurations. The start and slot offset and the periodicity were considered, but the reference SFN was not. Note that the SFN and the subframe number is used in the following formula:

[(SFN × 10) + subframe number] modulo (cellDTX-DRX-Cycle) = (cellDTX-DRX-StartOffset)

Based on the SFN and the subframe number the StartOffset can be calculated. If different cells share the same SFN, there is no problem and all cells’ cell DTX/DRX on duration time is aligned. The above is based on the assumption that the SFN of different cells is the same. However, for the unaligned SFN carrier aggregation case, how to align cell DTX/DRX in two serving cells has not been discussed. If the StartOffset is calculated based on each serving cell, there will be misalignment on the two cells. The problem can be illustrated as follows:

Figure 1 An illustrate figure of the unaligned SFN case with one cell DTX pattern

Figure 2 An illustrate figure of the unaligned SFN case with two cell DTX patterns

Figure 1 and Figure 2 show an unaligned SFN of the PCell and SCell with a single cell DTX pattern and two cell DTX patterns, where the SCell’s SFN is a little earlier than the PCell’s SFN. By using the SFN and subframe calculation formula, the cell DTX on duration in each frame is shown. As we can see, the cell DTX active time between two cells is not aligned. The behaviour of UE PDCCH monitoring of two cells becomes complex since the UE needs to stay in active time longer than in the aligned case.

In TS 38.321, the unaligned SFN across carriers in C-DRX was considered. To make the C-DRX on duration time aligned, the SFN of the SpCell is considered as the reference SFN.

NOTE 2: In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration.

By combining the cell DTX/DRX and UE C-DRX, the problem can be illustrated as follows:

Figure 3 An illustrate figure of combining unaligned cell DTX and UE C-DRX

Figure 3 shows a combination of cell DTX and UE C-DRX. The C-DRX is aligned based on the SFN of the PCell, while cell DTX is not aligned, which is calculated based on the SFN on each concerned serving cell. There is a problem that the overlap of cell DTX and C-DRX can be satisfied in the PCell but not SCell. In RAN2#123 meeting, we have agreed that “The gNB should ensure that there is at least partial overlapping between UE C-DRX on-duration and cell DTX/DRX on-duration.”. If the cell DTX/DRX is not aligned in different carriers, the overlapping of cell DTX/DRX and UE C-DRX may not be satisfied.

In order to decrease the implementation complexity and UE monitoring complexity, the cell DTX/DRX SFN should be aligned with the same principle as in C-DRX.

Proposal 1: In case the SFN is unaligned across carriers, the UE uses the same principle as in C-DRX for on-duration calculations.

A TP for proposal 1 is provided in the Annex.

# Conclusion

In this contribution, we discussed unaligned SFN issues on Cell DTX/DRX, and made the following proposal:

Proposal 1: In case the SFN is unaligned across carriers, the UE uses the same principle as in C-DRX for on-duration calculations.

# References

- Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

- Report of 3GPP TSG RAN WG2 meeting #123bis, Xiamen, China

# Annex

## 5.34	Cell-Level Energy Saving

### 5.34.1	General

Each Serving Cell may be configured by RRC with a periodic cell DTX pattern (i.e., Active and Non-Active Periods). The cell DTX operation controls UE's monitoring activity of PDCCH and configured downlink assignments in RRC_CONNECTED. For all activated Serving Cells with cell DTX configured and activated, the MAC entity may monitor PDCCH and configured downlink assignments using the cell DTX operation specified in clause 5.34.2.

Each Serving Cell may be configured by RRC with a periodic cell DRX pattern (i.e., Active and Non-Active Periods). The cell DRX operation controls Scheduling Request and configured uplink grant transmission activity in RRC_CONNECTED. For all activated Serving Cells with cell DRX configured and activated, the MAC entity may transmit configured uplink grant transmissions and Scheduling Request using the cell DRX operation specified in clause 5.34.3.

RRC controls cell DTX and cell DRX operation by configuring the following parameters in cellDTX-DRX-Config per Serving Cell:

-	cellDTX-DRX-ConfigType: defines whether only cell DTX is configured, only cell DRX is configured, or both are configured;

-	cellDTX-DRX-onDurationTimer: the active duration at the beginning of a cell DTX/DRX cycle;

-	cellDTX-DRX-StartOffset: defines the subframe where the cell DTX/DRX cycle starts;

-	cellDTX-DRX-SlotOffset: the delay before starting the cellDTX-DRX-onDurationTimer;

-	cellDTX-DRX-Cycle: the cell DTX/DRX cycle period.

-	ActivationStatus: the initial activation status of cell DTX and cell DRX operation.

### 5.34.2	Cell Discontinuous Transmission

Cell DTX is configured if cellDTX-DRX-ConfigType is set to dtx or dtxdrx. Cell DTX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DTX indication from lower layers indicating activation or deactivation of cell DTX operation, as specified in TS 38.213 [6];

-	configuring cellDTX-DRX-Config by upper layers: if cell DTX is configured and cellDTX-DRX-ActivationStatus is set to activated, cell DTX operation is activated upon cell DTX configuration; if cell DTX is configured and cellDTX-DRX-ActivationStatus is set to deactivated, cell DTX operation is deactivated upon cell DTX configuration; if cellDTX-DRX-Config is released, cell DTX operation is deactivated and all the corresponding configurations are released.

When cell DTX is configured and activated for a Serving Cell, the cell DTX Active Period includes the time while:

-	cellDTX-DRX-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DTX, the MAC entity shall:

1>	if cell DTX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (cellDTX-DRX-Cycle) = (cellDTX-DRX-StartOffset):

3>	start cellDTX-DRX-onDurationTimer for this serving cell after cellDTX-DRX-SlotOffset from the beginning of the subframe.

NOTE:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the cell DTX duration.

1>	if cell DTX operation is deactivated for this Serving Cell; or

1>	if the Serving Cell is in the cell DTX Active Period:

2>	monitor PDCCH on this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if any drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL (as described in clause 5.7) is running on any Serving Cell in the DRX group of this Serving Cell; or

1>	if ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

1>	if a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5); or

1>	if a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a):

2>	monitor PDCCH on the Serving Cells in the DRX group of this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if ra-ResponseWindow (as described in clause 5.1.4) is running and this Serving Cell is the SpCell:

2>	monitor PDCCH on this Serving Cell (as described in clause 5.1.4).

For each Serving Cell configured with cell DTX, the MAC entity need not:

1>	if cell DTX operation is activated and the Serving Cell is not in the cell DTX Active Period:

2>	monitor PDCCH for the MAC entity's RNTIs listed in clauses 5.7 and 5.7b, irrespective of the requirements of clauses 5.7 and 5.7b, unless stated otherwise in this clause;

2>	instruct the physical layer to receive transport block on the DL-SCH of this Serving Cell according to a configured downlink assignment for SPS;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with the PDSCH duration of a configured downlink assignment;

2>	consider the NDI bit for the HARQ process corresponding to the PDSCH duration of a configured downlink assignment to have been toggled for the configured downlink assignment.

### 5.34.3	Cell Discontinuous Reception

Cell DRX is configured if cellDTX-DRX-ConfigType is set to drx or dtxdrx. Cell DRX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DRX indication from lower layers indicating activation or deactivation of cell DRX operation, as specified in TS 38.213 [6];

-	configuring cellDTX-DRX-Config by upper layers: if cell DRX is configured and ActivationStatus is set to activated, cell DRX operation is activated upon cell DRX configuration; if cell DRX is configured and -ActivationStatus is set to deactivated, cell DRX operation is deactivated upon cell DRX configuration; if cellDTX-DRX-Config is released, cell DRX operation is deactivated and all the corresponding configurations are released.

When cell DRX is configured and activated for a Serving Cell, the cell DRX Active Period includes the time while:

-	cellDTX-DRX-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DRX, the MAC entity shall:

1>	if cell DRX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (cellDTX-DRX-Cycle) = (cellDTX-DRX-StartOffset):

3>	start cellDTX-DRX-onDurationTimer for this serving cell after cellDTX-DRX-SlotOffset from the beginning of the subframe.

NOTE 1:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the cell DRX duration.

1>	if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period:

2>	not instruct the physical layer to signal a SR on a PUCCH resource for SR;

2>	not increment the SR_COUNTER for a SR;

2>	not start the sr-ProhibitTimer for a SR;

2>	not deliver any configured uplink grant and the associated HARQ information to the HARQ entity;

2>	not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission;

2>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH;

2>	if an emergency service is initiated by upper layers and this Serving Cell is the SpCell:

3>	initiate a Random Access procedure (as specified in clause 5.1.1).

NOTE 2:	How the MAC layer in the UE is aware of an ongoing emergency service is up to UE implementation.